Computer Lab Midterm Exam

27.11.2020

1) D Flip flop shown in figure. Complete the given verilog code below. Fill the code with given ……… blanks.

module dff (clk, reset,  d, q, qb);

input      clk;

input     reset;

input      d;

output   q;

output     qb;

reg        q;

assign qb = ~q;

always @(posedge clk or posedge reset)

begin

if (reset) begin

// Asynchronous reset when reset goes high

q <= 1'b0;

end else begin

q <= d;

end

end

endmodule

2)  Write the code for “case” code part for  4x1 mux.

module mux4x1 (sel, a, b, c, d, o);

input [1:0] sel;

input a, b, c, d;

output reg o;

always @(sel or a or b or c or d)

begin

case(sel)

2'b00:o <=a;

2'b01:o <=b;

2'b10:o <=c;

2'b11:o <=d;

endcase

end

endmodule