1. What training should the workers in the construction work have received?

a. primary school

b. College

c. literate

d. Vocational training

2. Which of the following is true for ammonia?

a. Causes of paralysis in the respiratory system.

b. Exposure limit value is lower than carbon monoxide limit value.

c. It is explosive at values exceeding 15%.

D. There is a sharp smell.

3.Which of the following are suitable for the legislative provisions in the studies conducted in open areas far from the settlements where 3,100 persons have worked?

a. Making a cabin and urinal for every 30 male workers and at least one toilet for every 25 female workers

b. Construction of toilet cabins, one for each 50 male and 50 female workers

c. Construction of toilet cabins, one for every 20 men and 20 women workers

D. Construction of toilet cabins, one for each 40 male and 40 female workers

4. How long is the obligation for the occupational disease caused by the disease?

a. 2 months

b. 2 years

c. 2 weeks

D. 2 days

5. According to the Occupational Health and Safety Regulations, the safety factor of the hooks of the lifting gears shall be at least equal to the number of loads in dangerous cargoes such as molten metal or abrasive materials?

a. 1.5

b. 5

c. 3

D. 7

6. Which of the following will not be included among the training subjects specified in the "Procedures and Principles of the Occupational Health and Safety Training of Employees"?

a. Working with display equipment

b. Safety in maintenance work

c. Cleanliness and order

D. Warning signs

7. Which of the following is not the workplace factors that cause occupational diseases and occupational accidents?

a. Chemical

b. Mechanical

c. Personal

D. Physical

8. Which of the following should be done first in order to prevent workers' lives and health from becoming dangerous in workplaces that are suffocating, poisonous or irritating gas and smoke?

a. Mask must be operated

b. Ventilated

c. Gloves must be worn

D. Training should be done

9. Which of the following materials is electrically better conductive?

a. Metals

b. Inactive gases

c. Ametals

D. Pure water

10. Which of the following is the phone number of the gas notice?

a. 187

b. 112

c. 190

D. 110

11. What type of material should be used for pipes or pipes used in the inspector installation?

a. Steel

b. Fireproof

c. Stainless

D. Painted

12. Which of the following is one of the workplaces for which occupational health and safety establishment is to be established?

a. Scientific research centers and lab work

b. Railway transport work

c. Production of milk and dairy products

D. All kinds of business and income animals

13. Which of the following is not the risk of indoor environment?

a. Electricity

b. Flammable gases and vapors

c. Oxygen excess

D. Too dark

14. In which law is the employer's legal liability arising from work accidents and occupational diseases?

a. Public Hygiene Law

b. Social Insurance Law

c. Law of Obligations

D. Act on the Establishment of the Ministry of Labor

15. Which of the following is not the purpose of interviewing witnesses when a work accident investigation is being conducted?

a. Verifying what other witnesses are saying

b. Obtain opinions on how to protect from injury

c. Explain and confirm what is

D. Calculating the amount of compensation for those who caused the accident

16. When should the "SGK Recruitment Declaration" be issued indicating that the worker has started work in construction work?

a. After a day

b. The same day

c. End of the month

D. One day before

17. Who is the physician who says, "We have to ask our patients what they are doing" and the father of the worker's health?

a. Agricola

b. Ramazzini

c. Plini

D. Nicand is

18. Which of the following is not one of the things to be aware of before using cutting and drilling hand tools?

a. Make sure that the tips rotate freely without touching them

b. Apply high pressure to the field

c. To check the tips of the device before

D. Do not use damaged or vibrating tips

19. What is important to the preparation of the camp sites in the work done in remote open areas from the settlements?

a. The campground should not be an area with prior permission for activity.

b. The camp site should not be in a position to cut off natural drainage channels.

c. The campground should be planned so that the use of the vehicle is as great as possible.

D. The camp site should be kept as large as possible.

20. According to the Labor Code, the definition of "night" in working life starts at what time, what time does it last, and how many hours it lasts?

a. The earliest time is from 9:00 am to 5:00 pm at the latest and at most 9 hours per hour.

b. It is the period that starts at 20.00 the earliest and lasts until 06.00 the latest, and in any case it lasts for at most ten hours.

c. It is the period that starts at 18.00 latest and lasts until 07.00 the earliest and lasts at most twelve hours in any case.

D. It is the period starting from 20:00 at the latest until 6:00 am at the earliest and lasting at most eleven hours in any case.

21. Which of the following is not one of the work accident investigation steps?

a. Preventive actions

b. Conclusion and opinion

c. Preparation

D. Media measurement and reporting

22. When can explosives be used?

a. Where there is an old and new manufacturing gaps and cracks that are likely to have grizzles and can not be controlled,

b. In quarries with less than 1% methane

c. In the opening of closed fire dams

D. Opening of clogged coal, burs and silos

23. Which of the following can not be done while working with hand tools?

a. Wear loose-fitting, long-sleeved clothes

b. To wear neck, ring or wedding ring

c. Using a tie

D. All

24. Which one of the following is true for the gas volumes that make up the air?

a. 2.91% hydrogen

b. 4,12% carbon dioxide

c. 72,04% nitrogen

D. 20.93% oxygen

25. According to the results of the risk assessment performed at the employer's workplace; In the case where the risks in the workplace can not be remedied or reduced sufficiently by means of work methods, work organization and collective protection measures, which of the following should be kept and appropriately used?

a. Instructions for operating machines

b. Safety and health signs

c. Personal protective equipment

D. Machine protectors

26. How many months should passed to check Dry Powder Fire Extinguishers (KKT)?

a. 3

b. 12

c. 6

D. 9

27. How long should the entire electrical installation including the lighting circuit periodically be checked and maintained by authorized personnel?

a. Once in two years

b. Every three years

c. Every four years

D. Once a year

28. In which legal entity will the rights of the workers working in that company be regulated?

a. Insurance Law

b. commercial law

c. Debt Law

D. Business law

29. What color should not be used to reduce reflection and glare on plastic curtains and panels used to protect from ark rays?

a. Red

b. Green

c. Orange

D. Yellow

30. Which of the following should be on the machine when the work machines are running?

a. Operator and pointer

b. Pointer alone

c. Operator and assistant

D. Operator only

31.Which of the following is not one of the things to be aware of when using power tools?

a. Only move the tool closer to the workpiece when in operation

b. Pay attention to the turning direction of the tool

c. Keeping the hands close to the rotating ends of the tool

D. Protect tool tips from impact, impact and grease

32. Who is responsible for the training of the employees of the sub-employers in the establishments where the actual employer-sub-employer relationship is established?

a. Principal employer

b. Principal employer and sub-employer

c. Sub-employer

D. Job security specialist

33. Which of the following is not one of the fire gases?

a. Methyl bromide

b. Phosgene

c. Hydrogen cyanide

D. Nitrogen oxides

34. According to the Labor Law, the occupational accidents and occupational diseases shall be notified to the relevant Regional Directorate within the latest number of days?

a. Within 15 days at latest

b. Until the day ends

c. Within two working days with the latest writing

D. At the end of the first week

35. To ensure that the compressors operate safely,

Which of the following is wrong for the experiments and controls?

a. Periodically every 6 months

b. After installation and operation of the compressor

c. After a change in the compressor and major repairs

D. Periodically every 12 months

36. Which of the following is the marking on personal protective equipment?

a. TS

b. CE

c. TM

D. TSE

37. How many meters will not pass the length of wooden moving stairs?

a. 4

b. 2nd

c. 8

D. 6

38. In which group is the occupation according to "Communiqué on Hazard Classes Regarding Occupational Health and Safety"?

a. Less dangerous

b. Dangerous

c. Safe

D. Very dangerous

39. According to the "Regulation on Ethical Code of Conduct for Public Officials and Procedures and Principles for Application", which of the following is not ethical code of conduct?

a. Public service awareness

b. Aim and Commitment

c. Institutional profitability

D. Courtesy and respect

40. From what age groups does the age group get into the definition of old people?

a. Older than 60 years

b. Older than 30 years

c. Older than 50 years

D. Older than 40 years

41. Preventing or reducing exposure to noise from the following

Is not the measure to be taken by the employer?

a. A noise-reducing work to limit the exposure time and noise level

Do not organize the organization

b. Taking appropriate measures according to technical developments, risks arising from exposure to noise

Destroying

c. Choose other methods of work that are less susceptible to noise exposure

D. Considering the work done, the lowest possible level of noise emission

Choose suitable work equipment

42. How many decibels is the amount of money you have to pay?

a. 70 dB

b. 100 dB

c. 140 dB

D. 50 dB

43. Health examinations of workers engaged in night work should be done at least every few years

Must?

a. At least twice a year

b. At least once in two years

c. At least once a year

D. At least once in three years

44. Which causes are not examined in the following main groups?

a. Workplace

b. Human

c. Method

D. Material

45. Which of the following is not considered when assessing risk?

a. What are the potential impacts and the likelihood of consequences?

b. What is the amount of product obtained at the end of the production process?

c. What are the hazards?

D. What are the potential impacts and consequences and are they acceptable?

46. ​​What is the concentration of percentage  of methane gas that can be regarded as non-hazardous?

a. 1

b. 10

c. 4

D. 7

47. The risk management process is one of the most important pillars of these studies in order to remove the risk of risk from the workplace environment. Which of the following is not the work of risk management processes?

a. Do not detect control measures

b. Identifying danger

c. Estimate Risk

D. Risk assessment

48. Which one of the following is the most effective and the most effective way to protect workers at health and safety risks?

a. Controlling the source of rain

b. Stop work

c. Employee replacement

D. Use of personal protective equipment

49. Which of the following is the purpose of the risk management process?

a. Emphasize systematic identification, analysis and control of hazards are effective measures. It should contain.

b. Establishment of the risk management process should be based on the objectives and objectives of search for the most effective, fastest and most reliable ways to reach them.

c. All

D. It is a technique that determines the hazards in the environment, investigates their effects on critical variables and functions, and develops mechanisms or strategies for protection.

50. Which of the following safety and health marking standards are not available?

a. Signs of transition

b. Prohibited signs

c. Warning signs

D. Order marks

51. Which of the following materials is not used in the systems used?

a. Virgin

b. Chromium

c. Steel

D. Plastic

52. Which of the following is not one of the topics to be considered in the planning of open work?

a. Communication

b. Distance to public establishments

c. Meteorology

D. High voltage power transmission lines

53. Which law is involved in business law?

a. Historical Law

b. Social Law

c. Public Law

D. Private law

54. According to SSI statistics, in which sector is the mortality accidents most frequent in our country?

a. Chemistry

b. Textile

c. Build

D. Metal

55. Which of the following is the temperature people are feeling?

a. Measured temperature of human body

b. Air is measured by a dry thermometer and the air flow rate in the environment and the humidity of the air

Depending on the temperature

c. Air temperature measured by glop thermometer

D. Air temperature measured by dry thermometer

56. How many meters must the speed of cages pass in human beings to be carried out by extraction systems with an automatic speed regulator?

a. 11th

b. 15

c. 10

D. 12

57. Which of the following is not one of the branches of public law?

a. Administrative law

b. constitutional law

c. Criminal law

D. Business law

58.What is the maximum time limit for night work?

a. 8 hours

b. 7.5 hours

c. 8.5 hours

D. 6 hours

59. Where there is danger of slipping and tipping, where should the height be higher than the number of meters from the base, should workers be given security belts?

a. 4

b. 5

c. 6

D. 3

60. According to Article 81 of the Labor Law; The number of workers at the workplace, the nature of the workplace, the number of employees at the workplace, the number of employees at the workplace, the number of employees at the workplace, the number of employees at the workplace, the number of employees at the workplace, And are not liable for any of the following according to the hazard class and degree of work:

a. Assign one or more workplace physicians and other health personnel as needed

b. Certified job security manager appoint

c. Establishing a workplace health and safety unit

D. One or more engineers who are experts in occupational safety

Commission technical staff

61. What should the employer do about occupational diseases and work accident reports in the workplace?

a. To notify the competent authorities

b. Keep in the workplace

c. To give a job security expert

D. None

62. Which of the following is not included among the general principles in taking measures for the protection of health and safety?

a. Worker's cautious and careful behavior

b. Assessing risks that can not be avoided

c. Prevention of risks

D. Fighting in the source with risks

63. Which of the following is not among the duties and responsibilities of employees of the health worker?

a. Knowledge and expertise

b. Take risks in the workplace

c. Prevention and emphasis on rapid movement

D. Follow-up of remedial actions

64. Which of the following is not the objective to conduct risk assessment work in enterprises in terms of OHS?

a. To ensure the safety of employees' work

b. To keep work accidents and occupational diseases at minimum level

c. Providing social security for employees

D. Taking precautions about risky situations in the workplace

65. What percentage of work accidents are made up of dangerous movements?

a. 50%

b. 88%

c. 44%

D. 10%

66. If the concentration of methane, which is a repellent gas and an explosive gas, is below% what does not explode and just burns?

a. 9

b. 7

c. 11th

D. 5

67.Which of the following is not the protection for the environment?

a. Closed work method

b. Local ventilation

c. Personal protectors

D. Change to non-hazardous

68. What kind of rules are included in business law?

a. Supplementary rules

b. Descriptive rules

c. Absolute imperative rules

D. Relative rules

69. Which of the following is called the electric circuit?

a. The system consisting of computer, fax and telephone is called electric circuit.

b. The system consisting of dam, turbine and water pipes is called electric cycle.

c. Magnet, metal dust and the card is called the electric cycle in the community.

D. The conductors or conductors that provide the transmission of the electric current are the chains and elements (electrical Power supply, lamp, conductor wire, key etc. Connect items in series or parallel circuits).

70. In our Republic, during the Republican period, 180th article of the workplace physician

Is the law that governs the operation of the

a. Social Insurance Law

b. The Ministry of Health Establishment Act

c. Public Hygiene Law

D. Law of Obligations Law of Hygiene Hygiene Law Establishing the Ministry of Health

71. According to the Occupational Health and Safety Regulations, how many months will the lifting machines and tools be fully controlled by a qualified technical person?

a. Once in a six months

b. Monthly

c. Quarterly

D. Once a year

72. What should be the targets of the risk assessment of the plant?

a. Determine how these errors will be assessed

b. All

c. When any error occurs, it determines the effects on goods, life and environment

D. In terms of operation, public / employee and environment, it determinres what level of safety and health to be provided

73. Which of the following topics is included in the education of occupational health and safety board members?

a. Safety precautions on hoisting machines

b. Safety precautions in welding work

c. Occupational diseases

D. Safety precautions in casting works

74. Which of the following is not one of the main features of enclosures and protection devices?

a. Must be located far enough away from danger zone

b. No additional hazards should arise

c. Must be easily deactivated or easily bypassed

D. Must be at a minimum barrier to monitor the production process

75. Should the bearings be able to withstand how many kg of the ball falls on 1.5 meters?

a. 3

b. 4

c. 2

D. 1

76. to reduce the possibility of cancers and mutagens, which of the following rules is the employer obliged to follow?

a. The employer has to follow  the quality management system of the company.

b. The employer shall keep the carcinogens and mutagenic substances in custom-built stores outside the workplace boundaries In special containers.

c. The employer shall, where technically possible, consider the health and safety of workers by using no  hazardous or less dangerous substances.

D. Employer, should research on the replacement of carcinogens or mutagen substances

77.Which way does the gust explosion become the most violent?

a. 10,5%

b. 9.5%

c. 15%

D. 10%

78. In which of the following international organizations both (employers representatives) and (workers representatives) are equally represented and have the same voting rights?

a. WHO

b. The ILO

c. UN

D. OHSA

79.Which one of the following is wrong according to the 79th Safety Directive of Machines?

a. For machines intended for use by non-professional operators, general education info most be provided within the operating instructions of the machine.

b. All machines must have instructions in Turkish.

c. The contents of the instructions are not limited to the intended use of the machines, They should also include possible misuse that can be reasonably predicted.

D. The instructions should be prepared in the community language that the expert staff can understand

80. Which of the following definitions of basic health and safety rules of the Machinery Safety Regulation is wrong?

a. Danger: Possible injury or damage to health

b. Protective equipment: The machine which protects by means of a physical obstacle part.

c. Person exposed to danger: Person who is in a danger zone

D. Risk: injury to health that may occur in a dangerous situation.

81. Which of the following is one of the jobs that are considered to be in business?

a. Oil and gas exploration

b. Works done in accommodation facilities such as hotels, pensions, camps, dormitories and guesthouses

c. All kinds of raw, semi-finished and finished vegetable, animal or industrial products and goods.

D. Warehousing, and similar works

82. Which of the following is not one of the causes of dangerous movements?

a. Inappropriate mechanical conditions

b. Inadequacy of knowledge and mastery

c. Physical disability

D. Machine without protection

83. Various gases may form in the masses spontaneously or as a result of some operations. Which of the following is not one of these gases?

a. Methane

b. Nitrogen

c. Carbon dioxide

D. Hydrogen sulphide

84. Which of the following is not one of the thermal comfort factors?

a. Temperature

b. Radiant heat

c. Humidity of air

D. Oxygen density of air

85. What kind of gloves should be used to protect the hands against impact and compression?

a. Gloves reinforced by placing steel compounds in gloves

b. Gloves with steel ferrules on their ends

c. Wire reinforced gloves

D. Gloves made of good fabric

86.Which approach has been adopted , following the Labor Law No.4857?

a. Development of a general prevention policy at workplaces

b. At every stage of the work, risk assessment approach is used to identify hazards

c. All

D. Continual improvement of occupational health and safety

87.What gas comes into contact with water when mixed with sodium?

a. Acetylene

b. Methane

c. Hydrogen

D. Nitrogen

88. what are Determination of methods for risk assessment according to the OHSAS 18001 Standard

a. 4.4.3 Targets and programs

b. 4.3.1 Hazard identification, risk appraisal and determination of controls

c. 4.3.2 Legal and other conditions

D. 4.5.1 Performance measurement and monitoring

89. How many ear audiograms should be taken when working in noisy jobs?

a. once in 3 months

b. None

c. Every 6 months

D. Every 2 months

90. Which of the following is true about working  buildings?

a. Must be designed according to the architecture

b. Designed according to workers wishes

c. Must be designed according to employer's view

D. Designed to work-specific

91.Which of the following is the fire department phone no.?

a. 112

b. 187

c. 110

D. 155

92. Which one of the following is the first employment law that was introduced in our Republic during the Republican period?

a. 4857

b. 1475

c. 506

D. 3008

93. How long is the period of legal work for the week?

a. 50 hours

b. 45 hours

c. 35 hours

D. 60 hours

94. Lifting and binding for the heaviest load according to Occupational Health and Safety Regulations

Will the safety coefficient of chains and hooks be at least?

a. 7

b. 5

c. 3

D. one

95. Which of the following is absolutely necessary on the machines?

a. Suitcase

b. Operator certificate

c. User guide

D. Fire extinguisher

96. Which of the following is not one of the factors that the ignition incident is connected to?

a. Mixing of the material with oxygen

b. Madden's genus

c. The shape of the material

D. Properties of Madden

97. Which of the following is one of the direct causes of occupational accidents?

a. Dangerous situation and dangerous movements

b. Chemical factors

c. Ergonomic factors

D. Physical factors

98. Which of the following 10% of work accidents is due to:

a. Thoughtfulness

b. Carelessness

c. Dangerous action

D. Dangerous situation

99. Which of the following belongs to the transition superiority in work machines?

a. Installed

b. from the side road

c. Empty

D. From the main road

100. Which of the following checks will be applied to reduce the risks according to the hierarchy

Should be considered first?

a. Engineering controls

b. Signs / warnings

c. Remove from the ground

D. Personal protective equipment

TEST 2

101. Which of the following is not included in the concept of work security?

a. Protection of production

b. Protection of employees

c. Protection of the economy

D. Protection of the operator

102. Which of the following is known as the founder of occupational health?

a. Nicander

b. Ramazzini

c. Paraselsus

D. Agricola

103 What is the average number of hours per day during in the term of industrial revolution?

a. 14-16

b. 10-12

c. 16-18

D. 8-9

104. Who did cause the acceptances of the law("Health and Morality of the Apprentices") at the  Parliament, in the initiative of 1802?

a. Sir Robert Peel

b. Antony Ashly Cooper

c. Robert Owen

D. Thomas Percival

105. Which of the following is the following is The first law on coal mines in the National Assembly?

a. Amale Menafii of Existing Coal Powders in Zonguldak and Ereğli Basin Fanned (Law No. 114 Relating to Public Prosecution)

b. Social Insurance Law numbered 506

c. Ereğli Havzai Fahmiyesi Law No. 151 concerning the Mining Law

D. Maadin Nizamnamesi

106. The legal responsibility of the employer arising from work accidents and occupational diseases Edited?

a. Public Hygiene Law

b. Law of Obligations

c. Act on the Establishment of the Ministry of Labor

D. Social Insurance Law

107. "International Labor Contract on Labor Inspection in Industry and Trade" number of ILO is the contract?

a. 91

b. 81

c. 102

D. 116

108. Which of the following is applicable in accordance with Article 74 of the Labor Code No. 1475,

Is not one of the statutes in force?

a. Occupational Health and Safety Measures to be Taken in Construction Work

b. In Workplaces and Workplaces Where Flammable, Explosive, Hazardous and Harmful Materials Work

Regulation on Security Measures to be Taken Workers to be Taken in Construction Works

c. Severe and Dangerous Works Statute

D. Occupational Health and Safety Regulation

109. Which of the following is not available  between new legislation and new innovations based on  EU directives?

a. Risk assessment

b. Employee involvement

c. Prevention and protection activities

D. Occupational health and safety board

110. Which of the following do not exist in the contemporary business health and safety approach?

a. Use less harmful substances instead of harmful substances

b. To protect against risks, first use personal protective equipment

c. Engineering controls

D. Administrative controls

111. Among the basic principles to be applied to protect workers from chemical agents

Which of the following is not available?

a. Destroy the elements in the source

b. Take precautions in the receiver (worker)

c. Do not subcontract

D. Prevent the agents in the spreading environment

112. Which of the following is the basis of occupational health and safety is not one of the factors?

a. Biological factors

b. Physical factors

c. Economic factors

D. Chemical factors

113.What is the meaning of the abbreviation of WHO?

a. World Health Organization

b. International Labor Organization

c. International Workers Confederation

D. World Agriculture and Food Organization

114. Occupational health and safety, 100 items in the Ottoman period which is the first legal document concerned?

a. Coal mines Nizamnamesi

b. Dilaver Pasha Nizamnamesi

c. Maadin Nizamnamesi

D. Eregli Fahmiyesi Nizamnamesi

115. Which of the following is the law that governs the operation of the our case, the 180th article of the Republican period?

a. Social Insurance Law

b. Public Hygiene Law

c. Law of Obligations Law of Hygiene Hygiene Law Establishing the Ministry of Health

D. The Ministry of Health Establishment Act

116. Which one of the following is the first employment law in our Republic during the Republican era?

a. 1475

b. 3008

c. 506

D. 4857

117. Which of the following is the current employment law in our country?

a. 1475

b. 2655

c. 4857

D. 4573

118. Which of the following is not one of the employers' obligations?

a. To give women a privilege

b. Creating a culture of security

c. Establishing a business health and safety management system

D. Providing social dialogue

119. Which of the following is responsible for the implementation of Labor Law No. in 4857?

a. The Ministry of Labor and Social Security

b. Ministry of Industry and Trade

c. Council of Ministers

D. Premiership

120.What date was the Ministry of Labor established?

a. 1945

b. 1930

c. 1950

D. 1965

121. Which of the following is not one of the national organizations on occupational health and safety?

a. TİSK

b. Labor Inspection Board

c. Turkish Red Crescent Society

D. Turkish Union of Territories

122. In which of the following international organizations are the government representatives of the countries, The representative and the employee representative are equally represented and have the right to vote?

a. OHSA

b. ILO

c. UN

D. WHO

123. Which of the following international organizations is not directly related to occupational health and safety?

a. OHSA

b. WHO

c. ILO

D. UN

124. In the workplace, during the execution of the work, What is the systematic work done with the aim of protecting from harmful situations?

a. Safety culture

b. Occupational health and Safety

c. Security precautions

D. Risk assessment

125. Which of the following is not one of the main objectives of occupational health and safety?

a. Protection of production

b. Protection of rights

c. Protection of employees

D. Protection of the operator

126. What is the cause of human accidents and accidents?

a. Dangerous situation

b. Dangerous action

c. Thoughtfulness

D. Carelessness

127. What percentage of workplace accidents consist of dangerous movements?

a. 50%

b. 88%

c. 44%

D. 10%

128. Which of the following is not one of the dangerous movements?

a. Maintenance-free machine

b. Dangerous use of tools and machines

c. Unsafe loading, handling and stacking

D. Take an insecure status

129. Which link of injury chain are injured or damaged?

a. Second

b. Fourth

c. Fifth

D. Third

130. Which of the following is not one of the dangerous situations?

a. Defective tools, machinery and equipment

b. Removing the machine protector

c. Ungrounded electric machines

D. Unopened gaps

131. According to the statistics, what percent of the accidents will never be prevented?

a. 2%

b. 8%

c. 1%

D. 5%

132. Which of the following is not one of the causes of dangerous movements?

a. Inadequacy of knowledge and mastery

b. Inappropriate mechanical conditions

c. Physical disability

D. Machine without protection

133. Which of the following is one of the methods of protection from accidents?

a. Implement control measures

b. Taking measures to prevent or reduce risks

c. Setting control measures

D. All

134. How many times are the payments for occupational health and work safety measures for harmful accidents as a result of work?

a. 4

b. 5

c. 10

D. 2

135. Which of the following is true for you about a business that fulfills its legal obligations on occupational health and safety?

a. Production will increase and cost will decrease.

b. Production will increase and cost will rise.

c. Production will decrease and the cost will increase.

D. None

136. Who has duties and responsibilities regarding work health and work safety?

a. Employer

b. Worker

c. State

D. All

137. Which of the following is not one of the employer's obligations?

a. To inform workers about legal rights and responsibilities

b. To educate workers about occupational risks

c. audit

D. Participate in occupational health and safety training

138. "Workers are also obliged to comply with the terms and conditions of worker health and safety."

Which provision of the Labor Law is available?

a. Article 47

b. Article 77

c. Article 66

D. Article 88

139. Which of the following institution is publisher of the statistics of occupational accidents and occupational diseases once a year in Turkey?

a. Occupational Diseases Hospital

b. Labor Institution

c. Ministry of Health

D. Social Security Institution

140. Which of the following concerns the provision of work safety?

a. All

b. Training of employees about the subject

c. Experience and knowledge sharing in occupational health and safety

D. Instructions and procedures, emergency preparedness, management programs and process safety

141. 10% of work accidents are due to:

a. Dangerous situation

b. Thoughtfulness

c. Dangerous action

d. Carelessness

142. Which of the following is a tariff suited to the definition of danger?

a. An estimated situation

b. Adverse conditions

c. Potential damage situation

d. Unlikely and unlikely occurrence

143. Which of the following should be the top priority?

a. Isolate the danger

b. Taking dangerous source

c. Giving personal protective equipment

d. Reduce the duration of exposure

144. Which of the following is not from social order rules?

a. Rules of law

b. Nature rules

c. Ethics

d. Religious rules

145. Which of the following is not one of the characteristics of the rules of law?

a. Continuous

b. Abstract

c. General

D. Being private

146. Which of the following is not one of the branches of public law?

a. constitutional law

b. Criminal law

c. Administrative law

D. Business law

147. Which of the following is not one of the basic rights and duties of a person?

a. Private privacy

b. Freedom of the person and security

c. Forcible operation prohibition

D. Select election

148. Which of the following is not one of the regulatory acts of the executive organ?

a. Decree-law

b. Law

c. Regulations

d. Regulations

149. Which of the following is not one of the principles adopted by criminal law?

a. Infinite recuperation

b. The rule of law state

c. The principle of perfect crime and punishment

D. Legal principle

150. Which of the following is not one of the branches of private law?

a. commercial law

b. Debt Law

c. Tax Law

D. civil law

151. Which of the following is not one of the mixed law branches?

a. Business law

b. State Private Law

c. Banking Law

D. Idea Law

152 In which legal entity will the rights of the workers working in that company be regulated?

a. Business law

b. commercial law

c. Debt Law

D. Insurance Law

153. In order to make a cancellation case against the Constitution, one of the following authorities

It is not?

a. Council of Ministers

b. Ruling Party Parliamentary Group

c. President

D. Main Opposition Party Parliamentary Group

154. Which establishment in the government in Turkey, is authorized to put valid laws?

a. Premiership

b. ministries

c. Council of Ministers

D. Turkish Grand National Assembly

155. Which body is authorized to remove the article?

a. Council of Ministers

b. Premiership

c. President

D. Turkish Grand National Assembly

156. Which of the following judicial decisions is binding at the law level?

a. Heavy Penal Court Decision

b. Decision of the Council of State

c. Decision of the Supreme Court of Appeals

D. Decision of the Supreme Court of Appeal

157. What law is involved in business law?

a. Public Law

b. Historical Law

c. Private law

D. Social Law

158. What kind of rules are included in business law?

a. Absolute imperative rules

b. Descriptive rules

c. Supplementary rules

D. Relative rules

159. Which of the following will not be among the basic concepts of labor law?

a. Employer

b. Commercial company

c. Worker

D. Sub-Employer

160. Which of the following is not one of the elements of the worker's conception?

a. An economic job

b. Work based on a business contract

c. Fee pay study

D. Independent study

161. Which of the following is not an employer?

a. Ordinary company employing workers

b. Mehmet Bey

c. Worker contractor

D. The ministry employee

162. Which of the following is not the employer?

a. Personnel manager

b. Master head

c. Director of human resources

D. Mechanical engineer worker

163. Which of the following is not considered a workplace?

a. Workplace vehicle

b. Workplace dining room

c. The special tool that the worker brings to the workplace

D. The workplace's creek

164. Which of the following is not from workers subject to the Labor Law?

a. Limousine crane operator

b. Factory cleaning worker

c. Cartoonist working

D. Classroom teacher

165. Which of the following is covered by Labor Law?

a. The gardener working in the garden of the factory

b. Super league player

c. Housekeeper

D. The hostess working on the plane

166. Which of the following business contracts is not in writing?

a. Borrowing contract

b. Team contract

c. Job contract on call

D. Collective labor contract

167. As a rule in the Labor Code, is the minimum age of employment?

a. 18

b. 13

c. 15

D. 14

168. How long is the legal working period of the week?

a. 60 hours

b. 45 hours

c. 50 hours

D. 35 hours

169. "No one can be forced to work. Judgment which place in our life

It is located?

a. Labor Law No. 1475

b. Code of Obligations

c. Labor Law No. 4857

D. 1982 Constitution

170. According to which law is "Business Certificate" given?

a. Public Hygiene Law

b. Business rules

c. Code of Obligations

D. Social Insurance Law

171. According to the Labor Law, occupational accidents and occupational diseases should be reported to the relevant Region

Is your manager informed?

a. Within two working days with the latest writing

b. Until the day ends

c. Within 15 days at the latest

D. At the end of the first week

172. In case of any of the following conditions, the job is stopped or the workplace is closed

Apply?

a. If a danger to a workers' life is identified

b. When there is no risk assessment at work

c. If there is no immediate action plan in the workplace

D. If adequate precautions have not been taken against the possibility of explosive atmosphere in the workplace

173. According to Article 81 of the Labor Code; Employers, employing at least fifty workers on a regular basis Determining the occupational health and safety measures to be taken at workplaces and Monitoring of implementation, prevention of occupational accidents and occupational diseases, first aid of workers And for the implementation of emergency treatment and preventive health and safety services, The number of workers, the nature of the workplace and the hazard class and degree of work

Which ones are not liable?

a. One or more engineers who are experts in occupational safety

Commission technical staff

b. Certified job security manager appoint

c. Establishing a workplace health and safety unit

D. Assign one or more workplace physicians and other health personnel as needed

174. The employer shall not disclose the personal health files of the workers for the number of years

Do you have to keep it?

a. 20

b. 40

c. 5

D. 10

175. Which of the following is true about occupational health and safety services to be provided to workers It is not?

a. Training time considers as working time.

b. Participants in trainings are paid overtime.

c. The trainings are arranged outside the rest periods.

D. Trainings should not put financial burden on workers.

176. Occupational health and safety units must have one of the following It is not?

a. Inspection, first aid and emergency room

b. Occupational safety specialist room

c. Room of business physician

D. Standby

177. The minimum number of workers in a workplace that is included in the dangerous class and counted as an industrial

Full-time job security specialist employment is compulsory?

a. 250

b. 500

c. 750

D. one thousand

178. If a worker in a very dangerous workplace has the least number of workers, Is it necessary to employ workplace physicians?

a. 750

b. 500

c. 250

D. 1000

179. Occupational health and safety units and tasks of common health and safety units Which is not among the following?

a. Health surveillance of workers

b. Fire alarm and exercise

c. Education, counseling and information

D. Supervision of working environment

180. What classroom occupations can the C class occupational safety specialists work with?

a. Dangerous and less dangerous

b. Dangerous

c. Less dangerous

D. Very dangerous

181.Who are not obliged to do business security experts?

a. Establishing a health and safety board

b. Prepare an annual work plan

c. Prepare an annual assessment report

D. Planning occupational health and safety training

182. The total number of training sessions for the job safety specialization can not?

a. 180

b. 220

c. 300

D. 160

183. Who is responsible for establishing the health and safety committee?

a. Job security specialist

b. Employer

c. Employer representative

D. Occupational physician

184. Which of the following should establish the occupational health and safety establishment in one of the workplaces?

a. Scientific research centers and laboratory work

b. Railway transport work

c. All kinds of business and income animals

D. Production of milk and dairy products

185. Which of the following is not one of the establishment of occupational health and safety?

a. Mechanical processing of wood

b. Manufacture and storage of ammunition and explosives

c. All kinds of shoe manufacturing jobs

D. Preservation of forests, planning (management), training, operation,

Limit work, fire fighting exercises, infrastructure works for these

186. Determination of the number of workers for establishing a health and safety board

Which one is based on

a. The number of workers belonging to the main employer and a sub-employer

b. Employer number of workers

c. Number of employees in the contracting organization

D. All workers in the workplace

187. Employees of multiple enterprises, including factories, establishments, enterprises or groups of enterprises

Which are related to establishment of occupational health and safety Is this true?

a. Occupational health and safety board is established based on the total number of workers.

b. Occupational health and safety is established with the participation of one representative from each workplace.

c. In each workplace where there are too many workers, a separate occupational health and safety board

Established.

D. Establishment of a health and safety board is not compulsory.

188. The employer shall ensure that the occupational health and safety to be established in each of his /

To organize the working procedures of their committees, to provide business and The reports on occupational health and safety at work, at least every few months, the related technical Staff and specialists?

a. 3

b. 12

c. 6

D. 1

189. Which of the following is a member of the Occupational Health and Safety Board?

a. Quality management representative

b. Employer representative

c. OHSAS management representative

D. Shift supervisor

190. Which of the following is not one of the board members of occupational health and safety?

a. Worker representative

b. Occupational physician

c. Master representative

D. Site chief

191. Which of the following is the office of health and safety establishment?

a. Occupational physician

b. Job security specialist

c. Union representative

D. Employer representative

192. Which of the following is the employer or the employer's representative for the establishment of occupational health and safety

Assigned by

a. Occupational physician

b. Health and safety worker representative

c. Worker representative

D. Master representative

193. Which of the following will be openly elected for the establishment of occupational health and safety?

a. Job security specialist

b. Person responsible for managing human resources, personnel, social affairs or administrative and financial affairs

c. Master representative

D. Civil defense expert

194. Which of the following is the case of occupational health and safety

Not involved?

a. Frequent occupational accidents and causes of dangerous cases

b. Basic principles of industrial hygiene

c. First aid

D. Effective communication techniques

195. Which of the following is the case of occupational health and safety between board members

Take place?

a. Safety precautions in casting works

b. Occupational diseases

c. Safety precautions in welding work

D. Safety precautions on hoisting machines

196. In accordance with the nature of the workplace health and safety establishment,

Is it task to prepare?

a. Workplace discipline regulation draft

b. Guidelines for workplace safety instructions

c. Draft internal regulation

D. Staff job descriptions

197. Which of the following is not a job health and safety board task?

a. To establish OHSAS 18001 business health and safety management system

b. The adequacy of fire, natural disasters, sabotage and the like in the workplace;

Tracking teams' work

c. Planning occupational health and safety education and training

D. Maintenance and repair work to be carried out on the premises

Plan and control the implementation of these measures

198. Workplace health and safety in the workplace to disrupt the health of the body

A worker who is faced with an urgent and vital danger that puts his integrity in danger,

And to determine the situation by applying for the safety board and to take necessary precautions

May be required to make a decision. In this case, when will the site be collected and decided?

a. Within 3 days

b. in 2 days

c. The same day

D. Within 4 days

199. Health and safety board meet at least every few months?

a. 3

b. 1

c. 2

D. 6

200. The meeting agenda, location, day and time of the health and safety board should be at least

How many hours ago are the board members notified?

a. 72

b. 48

c. 24

D. 36

201. The total duration of the regular meetings of the health and safety

Can not pass the clock?

a. 6

b. 18

c. 24

D. 12

202. According to which principle are the decisions of the health and safety establishment taken?

a. Voting

b. With voting multiple

c. According to the employer's view

D. According to the workers' view

203. Which of the following is true about the decisions of the health and safety establishment?

a. Board decisions are advisory.

b. Board decisions are concerned only with workers.

c. It is the obligation of the job security specialist to implement board decisions.

D. The employer is obliged to apply the decisions of the board in accordance with the legislation.

204. Which of the following is used for management systems? of the cycle

Is not one of the steps?

a. Planning activities

b. Implementation activities

c. Learning activities

D. Control activities

205. Which of the following statements is not true for management systems?

a. The basis of management systems is the principle of continuous improvement.

b. Implementation of management systems is based on legal obligations.

c. Management systems provide competitive advantage.

D. Management systems govern social, environmental and financial risks.

206. Which of the following statements is true or what?

I. 1st party audit is the customer's supervision of the establishment.

II. 3rd party audit is the audit of the organization by an independent organization.

III. Audits to certify organizations' management systems 3.

Party controls.

a. All

b. II and III

c. Only I

D. Alone II

207. Which of the following is the standard for occupational health and safety?

a. SA 8000 Standard

b. OHSAS 18001 Standard

c. ISO 9001 Standard

D. ISO 14001 Standard

208. Which of the following is not the principles of the quality management system?

a. Customer orientation

b. Process approach

c. profitability

D. Employee involvement

209. In which clause of ISO 9001 is excluded in the scope of certification

Are permitted activities?

a. Article 5

b. Article 8

c. Article 7

D. Article 6

210. What is the latest revision date of ISO 14001 Standard?

a. 1999

b. 1996

c. 2007

D. 2004

285.When treated for long-term exposure to high noise, the result
What happens to the following?

a. When a person is taken without noise, he / she will return to his / her old condition.
b. Hearing loss is chronic and does not improve.
c. But it will heal in a long time.
D. It'il heal right away.

286. Which of the following should be taken from the person who is affected by the noise
Are not measures?
a. Use of effective personal protection against noise
b. Taking a person who is exposed to noise into a well-insulated compartment
c. Employing loud medley training
D. Abbreviation of working time in noisy environment

287. How many ear audiograms should be taken when working in noisy jobs?
a. None
b. once in 3 months
c. Every 6 months
D. Every 2 months

288. How many decibels is the amount of money you have to pay?
a. 70 dB
b. 100 dB
c. 140 dB
D. 50 dB

289. The employer who does not take adequate measures against the noise,
What will he have to pay for his disease?
a. A salary compensation
b. Legal compensation
c. He stops work without paying anything
D. None

Hearing loss brought to the square by high-level workers, SSK Health How are the procedures defined according to the Charter?
a. Neural disease
b. Mental illness
c. Occupational disease
D. None

291. Regarding the noise audit at the workplaces, Which published in the Official Gazette dated 23.12.2003 Do you need to proceed according to the provisions?
a. Noise Regulation
b. Regulation on the Procedures and Principles of Occupational Health and Safety Trainings of Employees
c. Preparation, Completion and Cleaning Works Regulation
D. Implementing Regulation on Health and Safety Precautions in Working with Asbestos
292. Which of the following are the ear protectors that most reduce the noise intensity?
a. Earplug

b. Paraffinous cotton
c. Cuffed headphone for hygienic conditions
D. Cotton

293. Which of the following is the highest exposure exposure to noise exposure Is not the measures to be taken by the employer when these values ​​are exceeded?
a. Worker overtime paid to noise
b. Isolation of noise source
c. Cutting at the source of strain
D. Personal protective use

294.While avoiding or reducing noise-induced exposure Is not the measure to be taken by the employer?
a. Taking appropriate measures according to technical developments, risks arising from exposure to noise destroying
b. Considering the work done, the lowest possible level of noise emission Choose suitable work equipment
c. A noise-reducing work to limit the exposure time and noise level do not organize the organization
D. Choose other methods of work that are less susceptible to noise exposure

295. Hearing loss caused by noise can be classified as occupational disease
It has been identified?
a. Occupational Health and Safety Regulation
b. SSK Health Transactions Regulation
c. Regulation on Severe and Dangerous Works
D. Noise Regulation

296. How can the hearing and healthy human beings hear the sounds of which frequency range?
a. 20 kHz to 200 kHz
b. 2 Hz to 20 Hz
c. 20 kHz to 20000 Hz
D. 20 Hz to 20 KHz

297. Is the sound intensity defined as the threshold of hearing equal to one of the following?
a. 20 Ba
b. 20 dB
c. 20 Pa
D. 20 μPa

298. How long is the obligation for hearing loss due to noise?
a. 6 weeks
b. 6 years
c. 6 days
D. 6 months

299. In what frequency range can people perceive the vibrations?
a. 0.01 Hz to 0.1 Hz
b. 1 Hz to 1 kHz
c. 1 kHz to 10 kHz
D. 0.1 Hz to 1 Hz

300. How long is the period of obligation for occupational disease caused by chronic illness?
a. 2 weeks
b. 2 months
c. 2 years
D. 2 days
TEST 4

301. Daily exposure limit value for eight-hour working time in hand-arm vibrations
what?
a. 5 cm / s2
b. 0.5 m / s2
c. 0.5 cm / s2
D. 5 m / s2

302 Effective daily exposure value for eight hours of working time in body vibrations
what?
a. 5 cm / s2
b. 0.5 m / s2
c. 0.5 cm / s2
D. 5 m / s2

303. Which of the following is not one of the thermal comfort factors?
a. Radiant heat
b. Humidity of air
c. Oxygen density of air
D. Temperature

304. Which of the following factors affect people's heat exchange with the environment?
It is not?
a. Convective heat
b. Metabolic heat
c. The nature of work done
D. Dust density of air

305. Occupational disease resulting from the thermal comfort factors Which one is located?
a. None
b. Occupational Health and Safety Regulation
c. Regulation on Severe and Dangerous Works
D. SSK Health Transactions Regulation

306. Which of the following is the temperature people feel?

a. Measured temperature of human body

b. Air temperature measured by glop thermometer

c. Air temperature is measured by a dry thermometer and depends on the ambient air flow rate and the humidity of the air

D. Air temperature measured by dry thermometer

307. Which of the following rays do not ionize when taken for human body?

a. Proton beams

b. Microwave rays

c. Gamma rays

D. Beta rays

308.If any of the following types of radiation are taken into human body,

Feature?

a. X-ray

b. Ultraviolet rays

c. Visible light

D. Radio waves

309. At which point do we have radiation control at our premises?

a. General Directorate of Occupational Health and Safety

b. I OHS

c. Turkish Atomic Energy Authority

D. Labor Inspection Board of the Ministry of Labor

310. Which of the following statements is true?

a. When X-rays are given to the human body due to their low energies,

They can not pass to the other side.

b. Cosmic rays are in space or in atmospheric environments thanks to their high energies

They can travel for miles.

c. Alpha rays are emitted in space or in atmospheric environments thanks to their high energies

They can travel for miles.

D. Beta rays easily travel when they enter the human body through the skin, and the internal organs

They reach.

311. Which of the following units refers to the pressure on health and safety of workers?

a. Newton / cm

b. Bar / cm2

c. Newton / m2

D. Pascal / m2

312. Which of the following statements refers to an under-pressure work on occupational health and safety?

a. Workers carry heavy cargo

b. Workers carry light loads

c. Workers work underwater in deep water

D. Workers work in high places

313 How long is the duration of the obligation in the non-acute asylum for the cause of the change?

a. 10 years

b. 10 weeks

c. 10 months

D. 10 days

314. Which workers should be elected primarily in the work that the lower pressure requires?

a. Young and inexperienced workers

b. Young and experienced workers

c. Workers under 15

D. Old and experienced workers

315. Which of the following is the maximum amount of chemical substances allowed to be contained in closed workplace air and which will not disturb the health of those who work there eight hours a day?

a. STEL

b. ARD

c. TWA

D. MAC

316. Which of the following is not included in the definition of hazardous chemicals?

a. oxidising

b. Toxic

c. Flammable

D. Allergic

317. The legislation on the Safety Data Sheets (MSDS)

It is being prepared?

a. Ministry of Industry and Trade

b. The Ministry of Labor and Social Security

c. Ministry of Health

D. Ministry of Environment and Forestry

318. What does the blue color mean according to the NFPA Indication System?

a. All

b. Health

c. Flammability

D. reactivity

319. Which gas comes into contact with sodium in water?

a. Acetylene

b. Nitrogen

c. Methane

D. Hydrogen

320. What type of electrical appliances and luminaires should be used in the environment in which explosive  gases and vapors are present?

a. Globe

b. Waterproof

c. Ex-proof

D. the Antikro

321. "Regulation on the Protection of Employees from the Dangers of Explosive Atmospheres"

, What does "Zone 0" mean?

a. Places where spark sources may cause explosion

b. Locations where the explosive gas air mixture may be continuous

c. Places where explosive gas air mixture is not available

D. Places where explosive gas air mixture can occur

322. How many main topics does the harmfulness of the chemicals accumulate?

a. 7

b. 9

c. 5

D. 11t

323.What risk assessment should be done at least every few years in the workplaces that work with the chemicals?

a. 5

b. 4

c. 3

D. 2

324.What is the following phone number of gas emergency?

a. 110

b. 190

c. 112

D. 187

325. In workplaces where asbestos is being worked, the eight-hour time-weighted average value (TWA)

Which should be the following?

a. 0.1 fiber / cm3

b. 0.4 fiber / cm3

c. 0.2 fiber / cm3

D. 0.3 fiber / cm3

326. "Explosive, oxidizing, easily extinguishing, toxic, very toxic, harmful, abrasive, irritating,

Allergic, carcinogenic, mutagenic, toxic for reproduction and dangerous for the environment "

How do you name the chemicals with a few?

a. Inorganic chemical

b. None

c. Hazardous chemicals

D. Organic chemical

327. Work carried out by asbestos or asbestos material workers,

The records indicating the amount of exposure should be kept for at least how many years?

a. 20

b. 10

c. 40

D. 30

328. Regarding the safe use of chemicals by the International Labor Organization (ILO)

Which of the following is the contract and recommendation decision accepted in 1990?

a. Contract no 170 - Recommendation 177

b. Contract 177 - Recommendation 170

c. Contract # 170 - Recommendation No. 172

D. Contract # 172 - Recommendation 170

329. What does red color mean by the Danger Rating System?

a. Health

b. None

c. reactivity

D. Flammability

330. Workers working in the eight-hour study period found air in the respiratory area

The upper limit of the time-weighted average of the chemical concentration

Which is?

a. MAC

b. TWA

c. None

D. STEL

331. As a result of the direct exposure, the lead is mixed into the blood and the enzyme found in erythrocytes

Which of the following is causing the inhibition of the activities?

a. None

b. Lead poisoning

c. Solvent exposure

D. Allergic condition

332. Which of the following is synonymous with irritants and sensitizers?

a. diluents

b. allergens

c. insecticides

D. mutagens

333. A method for treating a repellent material with a burning substance,

What is the cause of what is happening?

a. Explosion

b. Fire

c. Flare

D. Combustion

334The substances are easily flammable when they are in contact with water when they are not flammable.

The flammable gases can be released. When the calcium carbide comes into contact with water,

Which one comes out?

a. Hydrogen

b. Nitrogen

c. Oxygen

D. Acetylene

335. How many pieces of information are required to be included in the Safety Data Sheets?

a. 22 items

b. 14 items

c. 16 items

D. 10 items

336.If the tolerance limits are lowered to the lower limit and the risk is lifted,

The non-hazardous level corresponds to the hygienic condition until the risk to the future is minimized

What should be given to the workers as?

a. None

b. Milk and yogurt

c. Personal protectors

D. Warning signs

337.Which of the following is the phone for the fire?

a. 112

b. 155

c. 110

D. 187

338.What should be done the ventilation of chemical substance deposits?

a. Must be ventilated both from top and bottom

b. Only top ventilated

c. Only ventilated at all

D. should not be ventilated

339.Which of the following is international transport related to dangerous chemicals

Is one of the terms used?

a. IMDG (Regulation on the Transport of Dangerous Goods on the Sea)

b. All

c. ADR (International Transport by Dangerous Goods Road)

D. RID (International Transport by Dangerous Goods Railway)

340. Which of the following are the biological risk factors?

a. Non-living organisms only

b. All kinds of living organisms

c. Microorganisms that can cause infection

D. Microorganisms that do not cause infection, allergies or poisoning

341. The most dangerous group of risks from the determined biological factors by the level of infection risk

Is it from the following?

a. Group 2

b. Group 3

c. Group 4

D. Group 1

342. Which of the following is the employer

Liabilities?

a. Educate workers

b. To inform workers about risks and precautions in the workplace

c. All

D. To make a risk assessment in the workplace

343.What are the most likely risk factors for biological risk factors?

a. If not infected, digestive

b. Not working with proper work clothes in well-avoided environments

c. Through the skin when not in contact

D. Inhaled by inhalation

344. Which of the following sectors are subject to biological risk factors

It is not?

a. Metal coating works

b. Hair and body care sectors

c. Food production, packaging and storage works

D. Medical and dental work

345. Which of the following tasks is considered a psychological risk-taker?

a. Metal coating work resting every three hours

b. Uniform and monotonous work for at least four consecutive hours

c. Furnishing work done every half hour

D. Firefighting work without interruption

346 "Stress" determined at risk assessment in workplace

Which of the following factors is involved?

a. Biological risk factors

b. Psychological risk factors

c. Physical risk factors

D. Chemical risk factors

347. Which of the following stress reduction or absence from work conditions

Is not one of the methods of

a. In the 8-hour study period, giving two breaks outside the meal

b. Arranging work hours and monotonous work

c. Listen to light music during work

D. Increase resting intervals and do breathing exercises

348. Which of the following diseases is mental illness resulting from work?

a. Corneal disorders

b. Hearing loss

c. neuroses

D. Pneumoconiosis

349. One of the following neuroses that are limited in time and lead to repetitive tasks

It is not?

a. Anemia

b. Neural fatigue syndrome

c. Sleep disorders

D. Temperament disorders

350. Person, tool-equipment, environmental conditions that interfere with the interaction and reveal this interaction

The science of working to reduce and prevent emergent physical and psychosocial problems

of the following

a. Worker health

b. Job security

c. psychology

D. Ergonomics

351. What is the main goal of ergonomics?

a. Adaptation of the workplace to the worker

b. Giving the workplace a competitive edge

c. Adaptation of the worker to the workplace

D. Increasing the quality of workplace

352. Which of the following components of the ergonomic is absent?

a. Sociological

b. Psychological

c. Anatomical

D. Physiological

Anthropometry, a special science that deals with the dimensions of the human body,

Which does not cover the dimensions?

a. Depth

b. Length

c. Width

D. Weight

354. Which of the following does not fall into the field of ergonomics?

a. Working conditions

b. Product quality

c. Environmental conditions

D. Human characteristics

355. With the successful implementation of ergonomic practices, which of the following

It takes place?

a. The business runs up.

b. Quality, productivity, profit increase.

c. Absence increases.

D. Material waste increases.

356. Which of the following criteria does not apply to evaluating worker ergonomics?

a. foldability

b. Feasibility

c. expectability

D. Discontent

357. In terms of anthropometric,No way?

a. Reduce the strain on the employee.

b. Reduce worker fatigue.

c. Muscle skeletal system prevents the emergence of disturbances.

D. Muscle skeletal system causes discomfort.

358.While taking precautions for the protection of health and safety,

Are not among the general principles?

a. Fighting in the source with risks

b. Preventing risks

c. Worker's cautious and careful behavior

D. Assessing risks that can not be avoided

359. Which of the following safety and health marking standards are not available?

a. Prohibited signs

b. Warning signs

c. Signs of transition

D. Order marks

360. Which of the following is the definition of electricity?

a. Electrons in an atom of matter, the movement around their atomic nucleus

It called.

b. A physical phenomenon caused by static or dynamically charged particles.

c. A permanent magnet is called electricity to attract metals.

D. The spread of sound waves in metals is called electricity.

361. Which of the following is called the electric circuit?

a. The system consisting of dam, turbine and water pipes is called electric cycle.

b. Magnet, metal dust and the card is called the electric cycle in the community.

c. The system consisting of computer, fax and telephone is called electric circuit.

D. The conductors or conductors that provide the transmission of the electric current are the chains and elements (electrical

Power supply, lamp, conductor wire, key etc. Connect items in series or parallel

Is the arrangement).

362. Which of the following are electrically conductive materials?

a. Metals

b. Pure water

c. Ametals

D. Inactive gases

363. Which formula is expressed by the Ohm law? Choose an answer.

a. I = R / V

b. I = V / R

c. R = VxI

D. V = RxL

364. Which of the following units does not relate to electricity?

a. Current

b. Power

c. Speed

D. Business

365. Which of the following should be occurred function in the case of a short circuit in the electric circuit with a supply of power, a button or a lamp?

a. The fuse is tripped and the circuit is opened.

b. The power button automatically powers on.

c. The lamp continues to illuminate when it is on.

D. The lamp fails and fires.

366. Which of the following statements is the electrically correct expression?

a. In an electric circuit, current flowing in one direction is called alternating current.

b. Flow is a direct current in an electric circuit that changes direction with time.

c. In an electric circuit, current flows in one direction

It called.

D. Voltages that change direction with time are called voltage.

367. What is the highest limit value for the small voltage below?

a. Voltages of up to 75 volts are called small voltages.

b. Voltages with a maximum voltage of 100 volts are called small voltages.

c. Voltages of up to 25 volts are called small voltages.

D. Voltages with a maximum voltage of 50 volts are called small voltages.

368. Which of the following materials has the highest conductivity?

a. Metals

b. Inactive gases

c. A metals

D. Plastics

369. Which of the following is one of the risks that may arise from electricity?

a. Exposure to a noisy electric motor

b. Exposure to azeotropic glaze during electrolysis

c. Exposure to source fumes emitted from the backside of the electricity

D. Fall from high due to electric shock

370. In an event where an electrical artifact may occur in a paddling environment,

It can take place?

a. During the operation of an electrical appliance that does not produce a flame

b. During operation of an electric motor with an Exproof feature

c. During the operation of the necessary tools to prevent the build-up of static electricity

D. The polarity of the lamp with an open battery, cable and a lamp assembly during burning

371. Which of the following is the cause of work accidents that occur during electrical work

Is not between?

a. Failure to ground the metal bodies of electrically operated devices

b. Do not show haste, carelessness and care when working with electricity

c. Working with electrical insulated rust, insulated pliers and similar tools while working with electricity

D. Excessive reliance on electrical workers

372.All of the following general safety precautions to be taken when working with electricity

Include?

a. Grounded metal with high voltage bare conductors

To work with bars

b. Using a 16 A fuse on the electric circuit

c. Using a suitable diode on the electric circuit

D. Using 30mA leakage current relay on electric circuit

373. Which of the following is one of the methods of occupational health and safety protection?

a. All

b. Preventive measures for the environment

c. Personal protective equipment

D. Safeguard measures against welding

374. The most effective and most effective way to protect workers at health and safety risks

What is the following method?

a. Controlling the source of rain

b. Use of personal protective equipment

c. Stop work

D. Employee replacement

375. National and workplace level health and safety conditions in the workplace

For the establishment of protection policies and approved by the TGNA 16

Contract entered into force by publication in Official Gazette dated March 2004 and numbered 25404

Is it from the following?

a. Contract number 170

b. Contract No 177

c. Contract No. 150

D. Contract No. 155

376. After the Labor Law No. 4857, the Ministry of Labor and Social Security Legal regulations on occupational health and safety during the nomination process; Contemporary and international Norms and regulations in accordance with the EU Directives.

Health and safety in the prevention and development of the following new Which approach has been adopted?

a. Development of a general prevention policy at workplaces

b. At every stage of the work, risk assessment approach is used to identify hazards and

Determination of measures to be taken

c. Continual improvement of occupational health and safety

D. All

377. Which of the following is not the health risks in the workplace environment?

a. Chemical factors

b. Personal factors

c. Biological factors

D. Physical factors

378. Which of the following is not the chemical agents that affect employees?

a. Radiation

b. fumes

c. vapors

D. Gases

379. Which of the following is the cause of lung dust diseases?

a. Cotton powder

b. Asbestos

c. Silicon

D. All

380. Which disease is caused by the influence of cotton dust?

a. pneumoconiosis

b. Bisinosiz

c. Silikokosiz

D. asbestosis

381. Which of the following is not a dangerous move?

a. Do not use personal protectors

b. Machine without protection

c. Working at dangerous speed

D. Not to use the machine suitable for the job

382 Which of the following should not be done to remove dangerous situations and behaviors?

a. Change of workplace

b. Development of necessary measures

c. Identification of dangerous situations

D. Analysis of dangerous situation

383. The work to be done on health and safety,

Have been summarized in view of the basic principles of the directive?

a. Framework Directive 80/480

b. Framework Directive 89/391

c. Framework Directive 90/290

D. Framework Directive 98/490

384.Available training for each employee's own work

Does not contain the criterion?

. a. It should be given when the job is changed.

b. It should be given when starting work.

c. It should be given when the fee is changed.

D. It should be given when the machine used changes.

385. In relation to the occupational diseases and occupational accidents reports that arise in the employer

Need to do?

a. Keep in the workplace

b. To give a job security expert

c. To notify the competent authorities

D. None

386.Which of the following is not from the protection of the environment?

a. Local ventilation

b. Closed work method

c. Personal protectors

D. Change to non-hazardous

387. Which of the following is not from person protection practices?

a. Local ventilation

b. Periodic examinations

c. Education

D. Entrance examination

388. Which of the following is not the factors that influence people's heat exchange with the environment?

a. Air flow rate

b. Vibration

c. Temperature

D. Moisture

389. Which of the following is one of the security measures to be taken at the source of the work

It is not?

a. Process change

b. Local ventilation

c. Use less harmful substances

D. General ventilation

390. According to the characteristics of toxic substances workers are exposed to in the workplace

Which of the following does not need to be done?

a. Scans should be repeated periodically.

b. It should be avoided at the source of risk in the workplace.

c. Worker replacement should be applied.

D. Media measurements should be made.

391. Which of the following is the most prevalent occupational accidents in welding operations

Is one?

a. Fire and explosion

b. Eye injury

c. Unprotected skin burn

D. Flaming clothing

392. Which of the following is not one of the threats that arise during welding operations?

a. Rays

b. Noise

c. Eczema

D. fumes

393. Work accidents that occurred during the welding and cutting operations are mostly the type of welding

In the process?

a. Oxygen-gas supply

b. Gas metal source

c. Arc source

D. Carbon source

394. How many micron-sized solid particles are called dust?

a. 250

b. 50

c. 150

D. one hundred

395. Which one of the following is not one of the gases that came into being during the welding process?

a. Carbon monoxide

b. Ozone

c. Phosgene

D. Oxygen

396. Profession emerging as a result of prolonged inhalation of dust, gas and fumes containing metal

Which of the following is the disease?

a. Allergy

b. Flu

c. pneumoconiosis

D. Asthma

397. Which of the following is true for carbon monoxide?

a. It is irritant.

b. It's a simple choking gas.

c. A chemical is a stifling gas.

D. It is a physical stifling gas.

398. Which of the following is true for carbon dioxide?

a. A chemical is a stifling gas.

b. It is poisonous.

c. A chemical is a stifling gas.

D. It's a simple choking gas.

399. Which of the following is one of the beams emitted from the source behind?

a. Magnetic rays

b. Cosmic rays

c. X-rays

D. Ultraviolet rays

400. Which of the following is the most effective source beam (wave length, high energy, hard character and damage to employees eyes) ?

a. Bright rays

b. Ultraviolet rays

c. Infrared rays

D. None

401. How many decibel noise peaks in metal arc welding and metal gas source

it consists?

a. 110

b. one hundred

c. 150

D. 120

402. For unit workshop area in general ventilation systems of welding workshops

How many m3 air changes are predicted in hours?

a. 45

b. 60

c. 70

D. 50

403. How many mg / m3 of acceptable limit metal oxide fumes in working ambient air?

a. 2nd

b. one

c. 0.5

D. 1.5

404. In plastic curtains and panels used to protect from light beams,

What color should not be used to reduce glare?

a. Yellow

b. Green

c. Orange

D. Red

405. Which of the following materials provide the best protection against radiation?

a. Poplin

b. Cotton

c. Wool

D. Skin

406. How many volts should the idle operating voltage be in alternating current at the source machines?

Choose an answer.

a. 60

b. 70

c. 40

D. 50

407. How many volts should the idle operating voltage in the source machines be in the direct current?

a. 70

b. 110

c. one hundred

D. 90

408.Which valve of oxygen tubes should not come into contact with the following substances?

a. Powder

b. Paper

c. Water

D. Oil

409. Which of the following materials are not used in the systems used?

Choose an answer.

a. Virgin

b. Steel

c. Plastic

D. Chromium

410. Which of the following can be stored with oxygen cylinders?

a. Acetylene tubes

b. Argon tubes

c. Propane tubes

D. Hydrogen tubes

411.What's with the gas tanks? Should not be exposed to temperature on C?

a. 85

b. 55

c. 15

D. 35

412.Hoses of the gas tubes should be at least how many meters long?

a. 7

b. 9

c. 3

D. 5

413. Which of the following is the device that performs the pressure adjustment task?

a. Regulator

b. Prosestat

c. Differential

D. Manometer

415. Which type of mask is used in welding operations in places where there is no local ventilation

It should be used?

a. P1

b. P2

c. P3

D. D1

416. Which of the following is wrong for welding operations in closed and narrow areas?

a. One person should wait outside.

b. Ventilation should be done.

c. Oxygen should be given.

D. Welding gases must be absorbed.

417. According to the Makina Safety Regulation; Lifting accessories and their accessories as a general rule, the number of working strands of wire-rope / rope-rope combinations Which of the following is equal to?

a. 2

b. 5

c. 3

D. 4

418. According to the Makina Safety Regulation; Lifting accessories and their accessories As a general rule, the number of working floors of chains originating from welded bridges Which of the following is equal to?

a. 2

b. 4

c. 1

D. 3

419. According to the Makina Safety Regulation; Lifting accessories and their accessories

The operating coefficient of the reading rope or slings is, as a general rule,

Which is equal to?

a. 1

b. 7

c. 3

D. 5

420. According to the Machinery Safety Directive of  For machines to remove persons Operating coefficients to be applied for lifting accessories and their accessories Among the working coefficients to be applied, one of the following is handled as a general rule Is taken?

a. This is to be considered as 2 times for machines for lifting persons.

b. The working coefficients are treated the same, they do not change.

c. This should be considered as 4 times for machines to remove persons.

D. This should be considered as 3 times for machines to remove persons.

421. Metal rope / rope holder from separate lifting equipment according to Machine Safety Directive As a general rule, Which of the following is operation factor?

a. 2

b. 4

c. 5

D. 3

422. What type of vehicle is the G class driver's license in Turkey?

a. Cars, minibuses or vans

b. Attractive

c. Motor vehicles, such as work machines

D. Wheeled tractor

Which one of the following cues is wrong?

a. A person with a Certificate of Occupational Use You can use.

b. A person with a Certificate of Occupational Use You can use.

c. A person with a Work Machine Certificate can use the machine anywhere.

D. The Certificate of Occupational Handling applies to vehicles in closed areas.

424 To be able to use a work machine on an open road, the most accurate Which is?

a. G class driver's license and the type of vehicle used in the document.

b. It must be an operator certificate.

c. It must be a G-class driver's license

D. There is no need for any documents, it is enough to use the machine.

425. Carriage Transport Activities According to Vocational Qualification Training Regulation Those who carry hazardous substances with the obligation to obtain a Vocational Qualification Certificate

Which documents should I buy?

a. SRC 1

b. SRC 3

c. SRC 7

D. SRC 5

426. Carriage Transportation Activities According to Vocational Qualification Training Regulation

Given to transport sector employees who have to take vocational qualification training

Which of the following is the meaning of "UDY" Professional Qualification Certificate?

a. Superstructure Supervision Managers

b. Country Counselors

c. Country Counseling Managers

D. Senior Manager

427. Which of the following is not the purpose of psycho-technical evaluation for the drivers.

a. Visual continuity in shape perception

b. Traffic rules

c. Speed ​​and distance detection

D. Response rate

428 Abbreviated European Treaty on the Transport of Dangerous Goods by Road

Which is the following?

a. DGR

b. ADR

c. ETR

D. TMT

429. How much is the periodic test of the Rip-Proof machines (forklift, crane, etc.)?

Need to be done in time?

a. Bimonthly

b. Every four months

c. Once in a month

D. Quarterly

430. Which of the following jobs can employ female workers?

a. Transportation of gas, oil, water and similar materials through all kinds of pipes and this transfer

Maintenance, repair, repairs and similar works carried out on their lines

b. Electricity production, transport and distribution works, electrical maintenance and repair work

c. Maintenance, supply, repair and parking for land, railway, sea and lake vehicles

The work done

D. All kinds of communication means such as mail, telephone, telegraph, radio, radio, television

Construction, maintenance and repair work

431. The entire electrical installation, including the lighting circuit, is periodically

How long should it be controlled and maintained by authorized personnel?

a. Every four years

b. Every three years

c. Once a year

D. Once in two years

431. Which one of the following cues is wrong?

a. A system should be established for allowing maintenance and repair work.

b. In the workplace, a system should be installed for maintenance and repair procedures, instructions and

Guidelines should be prepared.

c. In order to allow maintenance and repair work, system contractors

And the precautions and conditions that other persons should observe.

D. A system should be established for allowing maintenance and repair work.

432. Which of the following statements is wrong?

a. In maintenance and repair works to be done in wells or other underground facilities, experienced and skilled

Workers will be employed.

b. Before heat or welding work is carried out on the covered or double-walled or enclosed cups,

These are to be thoroughly ventilated and, as long as welding is done,

It will be given.

c. Pressure vessels and vessels will not be repaired under pressure.

D. Maintenance and repair work in wells or other underground facilities requires an experienced

Or more than one observer will be deployed.

433. Which of the following cautions should be taken in repair and maintenance systems

Is a process that is missing from the point of view?

a. The maintenance and repair team must take the necessary precautions before starting work

b. Planned repair work, planning for unexpected failures

c. Obtaining verbal permission from required places

D. Allowing authorities to take and take the necessary precautions

434. In what works are the hand tools used?

a. All

b. ream

c. Cut

D. compress

435.Which of the following does not cause any work accident while working with hand tools

Is one of the rules that must be observed for?

a. Using the right hand tool according to the work done

b. Use several different tools to do the same thing

c. Not to use a hand-held device to fit a material

D. Excessive pressure on handpiece

436. Which of the following is one of the causes of accidents in the hand tools?

a. Insufficient maintenance

b. Using tools that are not suitable for the work being done

c. All

D. Not to use tools properly

437. Which of the following is a woodworking tool to safely use hand tools?

Is one of the features that your stalks should have?

a. Buddy and thin handle

b. Quality and smoothly trimmed

c. Fringed and sharp surface

D. Coming in a good shape and size

438. Where the spark is dangerous and in which materials

It can be done?

a. Hard tire

b. All

c. Tree

D. Virgin

439. From which materials can be made tools such as cutter, cold cutter, cutter?

a. Steel

b. Hard tire

c. None

D. Tree

440.Which of the following hand tools do you use every time

Time should be used?

a. Just a business trip

b. Light

c. Appropriate personal protectors

D. Power

441. While working on drilling and cutting works which produce harmful, flammable and explosive powders What protection measures should be taken?

a. Use proper dust and chip suction equipment and wear protective dust mask

b. Working with intervals

c. Pressing, angling and shaking during cutting and punching

D. All

442. Which one of the following should be considered when using a portable sander

Is one of the issues?

a. Protective work gloves and shoes should be used

b. All

c. Face, eye and ear protectors should be used

D. Always check that the protector is in place

443. What must be done if the power tool and the working mains connection cable are damaged?

a. Continue to use the tool to finish the job

b. Try to fix it where it is

c. Marking the damaged connection cable

D. Unplug your plug immediately without touching

444. Which of the following should be one of the noted issues before using cutting and drilling hand tools?

a. Checking the edges

b. All

c. The tips are perfectly fitted in the gait and turn freely without contact at all being

D. Trial run for at least 30 seconds without overloading and damage No sawing or vibrating tips used

445. Which of the following should be noted when using power tools

Is not one of the issues?

a. Keeping the hands close to the rotating ends of the tool

b. Only move the tool closer to the workpiece when in operation

c. Pay attention to the turning direction of the tool

D. Protect tool tips from impact, impact and grease

446. One of the things to be aware of when working with hand tools

It is not?

a. If the handpiece is not standing securely with its own weight,

Fasten

b. Always keep the handpiece firmly with two hands while working and ensure that the working position is safe

Take care of

c. Always switch off and wait for the appliance to fully stop before releasing the appliance

D. Always keep the cord in front of the instrument

447. Which of the following should be observed before using cutting and drilling hand tools

Is not one of the necessary things?

a. Do not use damaged or vibrating tips

b. Make sure that the tips rotate freely without touching them

c. Apply high pressure to the field

D. To check the tips of the device before

448. Which of the following should be followed when working with cutting and drilling hand tools

Is not one of the issues?

a. All

b. Change direction while the tool is running

c. Pushing, angling and shaking

D. Braking the free-running cutting discs from the side

449. Which of the following is applicable to burrs, chips, Sparks, coolants and other particles cause serious injury or even death

Is not one of the measures to be taken to prevent it from happening?

a. Disconnecting the instrument before maintenance and repair begins

b. Applying safety instructions

c. Do not attach additional handle in any kind of work done with tool

D. Using visors

450. Which of the following is not true when working with the hand tools?

a. All

b. To wear neck, ring or wedding ring

c. Wear loose-fitting, long-sleeved clothes

D. Using a tie

451.Modern enterprises must be prepared before starting to work and workplace

Significant changes or re-audits when the proposals are made

Which is?

a. Health and safety documentation

b. Risk assessment

c. Emergency plan

D. Annual training plan

452. The supervisor at least every few days, to inspect the places where workers are employed,

The results of the proposal and the measures to be prosecuted

Have to write and sign together?

a. 20

b. 30

c. 15

D. 10

453. Except for the period of preparation, places where underground work has been carried out,

Connected to the ground?

a. One

b. Five

c. Two

D. Three

454. In human carriage to be carried out by extraction systems with automatic speed regulator,

How fast should the cages be, in how many meters?

a. 10

b. 11th

c. 12

D. 15

455. In the wells used for transportation of man and material, in the sewers, on the main breathing roads,

How many meters must the airspeed pass at a minimum in sloping and flat roads?

a. 8

b. 9

c. 10

D. 12

456. Which of the following belongs to the passing superiority in workmanship?

a. Tali from the road

b. From the main road

c. Empty

D. Installed

457. Which of the following is one of the things that must be found on the machines?

a. Operator certificate

b. Fire extinguisher

c. Suitcase

D. User guide

458.Operators, who are in error or need to be regularly maintained and replenished,

Where should the finished work machines be left?

a. In the parking lot

b. At a distance that can be affected by explosion

c. On the road

D. On slopes and on the edges

459. Whichever of the following can be entered into the excipient storage area?

a. Notebook

b. Radio

c. Mobile phone

D. Lighter

460. The firing of the firearms shall be effected not less than several meters from the nearest pier

Is made from a selected place outside the area?

a. 150

b. 200

c. one hundred

D. 120

461. In which type of capsule can the sewers be fired only in the quarries?

a. Radio

b. battery-operated

c. Wired

D. Electric

462. What kind of capsules are not used in coal and sulfur furnaces?

a. Lead hive

b. Aluminum sleeves

c. Plastic hood

D. Copper bush

463. In what situations can explosives be used?

a. Opening of clogged coal, burs and silos

b. Old and new manufacturing gaps that are likely to have a grizzle and can not control the grizzly

And where there are cracks

c. In the opening of closed fire dams

D. In the quarries with less than 1% methane

464. Does the fryer measure the grizzly within a few meters of a radius before filling the sewer holes in the grizzly quarries?

a. 10

b. 25

c. 15

D. 20

465. How many% of methane is detected in the measurements that can not be filled?

a. 2nd

b. 5

c. one

D. 3

466.What is the lower limit of the blast explosion?

a. % 4

b. % one

c. % 3

D. % 2nd

467.What is the upper limit of the discharge blast below?

a. 15

b. 10

c. 5

D. 20

468.Which way does the gust explosion become the most violent?

a. 9.5%

b. 10,5%

c. 15%

D. 10%

469. How do workers who work in the quarries where coal and coal-fired coal vessels are located have to mask during the work?

a. Carbon dioxide

b. Carbon monoxide

c. Butane

D. Methane

470. How many% more methane in the quarry are you forbidding to rescue workers and clean up the gas?

a. 3

b. 1

c. 2

D. 4

471. What is the minimum oxygen concentration allowed in a coal mine?

a. 21

b. 16

c. 19

D. 18

472. What is the maximum allowable concentration of carbon dioxide in the quarries?

a. 0.9

b. 0.5

c. 0.7

D. 1.5

473. Temperature and humidity are important problems in the underground mines as environmental conditions. Location How many centigrade degrees does the temperature increase every 100 meters when it is lowered?

a. 5

b. 20

c. 10

D. 15

474. In order to prevent dusting during the cutting and blasting process, makes?

a. It is wetted.

b. Dried.

c. Cleared.

D. Swept.

475. When the density of gas is above a certain level, what kind of system

It should be established?

a. Warning system

b. Ventilation system

c. Drying system

D. Soaking system

476. Employees using drilling rigs in the six mines of which they are located are subject to the following hazard

They will not be exposed?

a. Vibration

b. Corrosion

c. Powder

D. Noise

477. Underground mines may come into play due to dust exposure Which of the following procedures should be performed early on to identify affected individuals?

a. Spreading workers in mines and other dusty environments by intervals

b. Evaluation of the radiological aspects in the examinations

c. All

D. In addition, the density of the dust in the environment,

Monitoring by measurements

478. In accordance with the outdoors working in superstructure mining activities,

They will not be exposed?

a. Hot, cold, rain, wind

b. sunlight

c. Hazards related to the transport, storage and handling of explosive materials

D. Carbon monoxide

479. If the leading health problem in mining is the discomfort due to dust exposure

Dust control is of great importance in the environment. Which of these controls

Is one?

a. Use masks when needed

b. Establishing effective ventilation systems

c. Wet operating method

D. All

480. As a result of spontaneous combustion in some processes or in some processes,

It may occur. Which of the following is not one of these gases?

a. Carbon dioxide

b. Nitrogen

c. Methane

D. Hydrogen sulphide

481. Which industry is the sector with the highest number of occupational accidents resulting in death in our case?

a. Metal

b. Textile

c. Build

D. Mine

482. Which of the following are the main reasons for occupational accidents in the construction work?

a. Employer characteristics

b. Characteristics of occupational safety specialists

c. None

D. Workplace features

483. Which of the following is the origin of the business accidents on the construction business line

Is not one?

a. Excavation ditches

b. Material drop

c. Machinery and equipment accident

D. Personal protective equipment

484.Width of minimum width of floors used for works such as sweepings, whitewashes, repairs How many cm should it be?

a. 60

b. 100

c. 120

D. 80

485. Which of the following is the least widths of handling scaffolds as centimeters for regulations in walling and covering works

a. 100

b. 110

c. 120

D. 130

486. The width of the scaffolds, the ramp and the corridor should be at least how many centimeters?

a. 90

b. 80

c. 100

D. 110

487. How many meters will pipe attachments be made in steel pipe stands?

a. 6

b. 7

c. 5

D. 8

488. How many meters should the length of the piers be?

a. 5

b. 6

c. 8

D. 7

489. Which of the following is wrong with regard to the dismantling of the beach?

a. Dismantled material will not be thrown directly.

b. The detached material will be connected and connected.

c. The demolition of the pier will begin at the bottom.

D. No elements will be removed from the reinforcements and crosses of the scaffold before the demolition begins.

490. How many meters of wooden stairs will not be lengthened?

a. 6

b. 2

c. 4

D. 8

491. Which of the following is wrong about the winches?

a. It should not be traveled on cargo.

b. No competent or competent operator shall be allowed to use the vinci.

c. The load should not be known on the load while in motion.

D. Only the occupational safety specialist should signal to the crane operator.

492. Which of the following is not true about the crane?

a. A load on the crane must be safely left in the suspended position.

b. The nighttime cranes should be well illuminated.

c. In case of malfunction, the operator should not leave the vinci.

D. The crane operator should communicate very well with the pointer.

Which of the following is wrong with respect to the gulp crane?

a. The crane switchgear must be manual.

b. Steel ropes should not be thrown out of the drum.

c. The purse cabin electric motor should be grounded.

D. Pegs should be used so that the hook does not get rid of the hook.

494.What should be given to the worker's personal protective equipment that employs the purse-crane?

a. Glasses

b. Gasmask

c. Face order

D. Safety belt

495. Who will prepare the health and safety plan for construction work?

a. Occupational physician

b. Employer

c. Employer

D. Job security specialist

496. Which of the following should be on the machine while the work machines are running?

a. Operator and pointer

b. Operator only

c. Operator and assistant

D. Pointer alone

497.The soil excavated in building works should be thrown at least several meters away from the edge of the excavation

a. 1

b. 2

c. 3

d. 4

498. How many workers can be employed in construction work?

A. 16

B. 18

C. 19

D. 17

499.Building works, according to "Communiqué on Hazard Classes Regarding Occupational Health and Safety"

In which group?

A. Dangerous

B. Less dangerous

C. Very dangerous

D. Safe

500. Which vaccine should be done to employees working in construction

A. Tetanus

B. Swine

C. Whooping cough

D. Tuberculosis

TEST 6

501. How many centimeters should the handrails be in the construction work?

A. 80

B. 110

C. 90

D. one hundred

502. At least a few kilograms of load-bearing should the guardrails of building works be?

A. one hundred

B. 80

C. 90

D. 70

503. How many kilograms should be placed on square meters in square meters?

A. one thousand

B. 300

C. 400

D. 500

504. Where there is danger of slipping and tipping, the altitude shall be increased from the

Should safety belts be given to employees in places where there are too many?

A. 4

B. 5

C. 3

D. 6

505. Wool, made of sheet and cement mortar, or worn-out, worn and durable

Which of the following should be used in the reduced detonation?

A. None

B. Security hook

C. Work gloves

D. Roof ladder

506. Which of the following is dangerous due to shipbuilding and shipbuilding accidents

Is not one?

A. Negative thermal comfort conditions

B. Insufficient workspace

C. Inadequate profit ratios

D. Working in closed narrow spaces

507. When is the "SGK Employment Declaration" indicating that the worker has started to work in construction works

Should be given?

A. End of the month

B. After a day

C. One day before

D. The same day

508. Which of the following is not one of the safety conditions for working at high altitude?

A. Scarecrow

B. Wearing steel toe shoes

C. Seat belt use

D. Establishing a safety net

509. What training should the workers working in construction work have received?

A. Vocational training

B. Primary education

C. College

D. Literacy

510. Seat belts should have at least several kilograms of carrying load?

A. 1100

B. 1150

C. 1200

D. one thousand

511. "About the Procedures and Principles of Employee Occupational Health and Safety Trainings

According to the "Regulation", what is the obligation of the employers to inform the employees?

A. Service vehicle hours

B. Annual leave

C. Fees

D. Legal rights and responsibilities

512. In the establishments where the original employer-sub-employer relationship is established, the employees of the sub-employer

Who is responsible for their training?

A. Principal employer

B. Job security specialist

C. Sub-employer

D. Principal employer and sub-employer

513. Which of the following is true for employees in relation to health and safety training

Is it true?

A. They have to attend training.

B. They have to get a training certificate.

C. They have to ask questions in education.

D. They do not have to attend training.

514.Which of the following is wrong with regard to the education of employees?

A. The cost of training should be paid by the employer.

B. The time spent in the training is counted during the study period.

C. The worker must pay the tuition fee.

D. The worker has to attend training.

515. Which of the following workers should be given special attention to education?

A. Engineers

B. Masters

C. Disabilities

D. Formen

516. Which of the following is not the objective of health and safety education?

A. To provide safe working environment

B. Establishing business health and safety awareness

C. Reduce job accidents

D. To increase productivity in the workplace

517. In the preparation of the health and safety training programs,

Is it necessary to get a view?

A. Workers

B. Original and sub-employer

C. Sub-employer

D. Your primary employer

518. Which of the following is not included in the  annual training schedule?

A. education time

B. Education fee

C. Training objective

D. Training history

519. "On the Procedures and Principles of the Occupational Health and Safety Training of Employees

According to the "Regulation", which of the following types of education does not take place?

A. Additional training

B. New training

C. Formal education

D. Further education

520. Which of the following is "Employee's Occupational Health and Safety Training Procedures and

Regulation on the Principles of Education "?

A. Work safety in welding work

B. Work safety in construction work

C. Prevention of accidents, injuries and disease

D. Work safety in mining

521. Which of the following is the "Procedures and Procedures of Occupational Health and Safety Trainings of Employees"

Regulation on the Principles of Education "?

A. Warning signs

B. Safety in maintenance work

C. Cleanliness and order

D. Working with display equipment

522. "On the Procedures and Principles of the Occupational Health and Safety Training of Employees"

According to the Regulation "the persons to be benefited from the training are

Which is not?

A. Labor inspector

B. Employer

C. Engineer or technical staff with work safety

D. Occupational physician

523.Which of the following is not available to be edited after training?

A. History of education

B. Education

C. Place of instruction

D. The duration of the training

524. Which of the following is not one of the factors that the burning event depends on?

A. Mixing of the material with oxygen

B. Properties of Madden

C. The shape of the material

D. Madden's genus

525. Which of the following is the natural heat source?

A. Volcano

B. Friction

C. Spark

D. Electricity

526. Which of the following is not one of the elements that make up the fire triangle?

a. Burned matter

b. Flammable material

c. Heat

D. Oxygen

527. Which of the following is not one of the fire gases?

a. Phosgene

b. Hydrogen cyanide

c. Nitrogen oxides

D. Methyl bromide

528. Which one of the following is not one of the gases that make up the smoke of fire?

a. Carbon dioxide

b. Sulfur oxides

c. Carbonyl chloride

D. Carbon monoxide

529. Which of the following is a class A poll?

a. Flammable liquid fires

b. Flammable gas fires

c. Flammable metal fires

D. Flammable solid fires

530. Which of the following is a Class B spark? .

a. Flammable solid fires

b. Flammable liquid fires

c. Flammable gas fires

D. Flammable metal fires

531. Which of the following is a Class C spark?

a. Flammable gas fires

b. Flammable solid fires

c. Flammable liquid fires

D. Flammable metal fires

532. Which of the following is a class D pollution?

a. Flammable metal fires

b. Flammable solid fires

c. Flammable gas fires

D. Flammable liquid fires

533 Which class fire group is involved in the fire of materials such as foil and thinner?

a. D

b. to

c. C

D. B

534. In which class of fire group is the fire of materials such as leather and leather?

a. C

b. B

c. to

D. TO

535. What class of fire group is involved in the fire of materials such as LPG and natural gas?

a. C

b. B

c. D

D. to

536. Which class of fire group is involved in the fire of materials such as magnesium and aluminum?

a. to

b. C

c. B

D. D

537. Fire of materials such as electrical equipment and electronic equipment which class fire

Group?

a. to

b. C

c. TO

D. D

538. Which of the following fire fighting aids are not used in electrical fires?

a. Foam

b. Carbon dioxide

c. Dry chemical powder

D. Water

539. What is used as an extinguisher in LPG fires?

a. Salt

b. KKT and CO2

c. Sand

D. Water

540. What is used as an extinguisher in metal fires such as magnesium and aluminum?

a. Water

b. CO2

c. Sand

D. WSC

541. Which of the following is not one of the heat spread types?

a. By way of glow

b. Direct contact

c. Through indirect contact

D. By air

542. Which of the following is a jet fire?

a. Burning of fuel flowing from one place

b. Fuel burned in one place

c. Burning fuel dripping from one place

D. Fuel burning from one place

543Which of the following is it?

a. Breaking of the tank due to cooling

b. Explosion of tank due to failure of safety valve

c. Explosion due to deterioration of tank manometer

D. The liquid in the tank starts to boil and boil and the rapid evaporation result

Causing the rising pressure to explode

544. How many centimeters should the fire extinguish the floor from the floor?

a. 90

b. 70

c. 115

D. one hundred

How many months should the 545.CO2 fire extinguishers be checked?

a. 3

b. 6

c. 9

D. 12

546. How many months should Kuretian Powder (KKT) fire extinguishers be checked?

a. 3

b. 9

c. 12

D. 6

546. How many years do fire extinguishers have to undergo a hydrostatic pressure test?

a. 2nd

b. 5

c. 10

D. one

547. How many meters must the distance between the fire alarm buttons exceed?

a. 50

b. 10

c. 20

D. 25

548. How many meters above which the fire alarm buttons should be located?

a. 1.2

b. 1.4

c. 1.1

D. 1.5

549.If only fire cabinets system is available as an extinguishing system, water capacity shall be minimum

How many liters will be available to cover for 60 minutes?

a. 200

b. 300

c. one hundred

D. 500

550. Measured lighting in the basement, upholstery and walking surfaces on escape routes

Should the level be at least how many lux?

a. 10

b. 50

c. 30

D. 20

551. Emergency lighting for at least several hours if normal lighting is interrupted

It should be provided?

a. 2nd

b. one

c. 6

D. 4

552. The events considered as offensive, and carelessness, imprudence, negligence, intent and variety

Which of the following is the situation caused by the events brought to the square by reason?

a. Work accident

b. Emergency

c. Accident

D. Natural disasters

Who is responsible for fire safety in buildings, buildings, facilities and businesses?

a. The greatest supervisor, owner or managers

b. Security and audit staff

c. Civil defense expert

D. Job security specialist

554. Which of the following teams should not be established according to the "Fire Regulations"

a. kidnapping

b. Protection

c. Recovery

D. First aid

555. In internal regulations on fire prevention and extinguishing; "Fire Regulations"

Which of the following items is not available?

a. Training and maintenance

b. Procedures for the use of extinguishing media

c. The age and experience of the staff

D. Number of emergency teams

556. Which of the following is not included among the stages of the application management system?

a. rehabilitation

b. Intervention

c. Do not know beforehand

D. Preparedness

557. To prevent any element of the institution from living a possible danger or

With the aim of eliminating unhealthy hazards with little or no damage

Is the written plan prepared below?

a. Emergency plan

b. Accident prevention plan

c. Risk assessment plan

D. Ramak stays plan

558. Which of the following is wrong for the four steps involved in emergency planning?

a. Analysis of hazards and risks

b. Implementation of the plan

c. Development of the plan

D. Training

559. If the facility or project is in construction phase, all sub-contractors involved in the construction and

Awareness trainings of the staff, if the building is in operation phase,

Which should be done?

a. Risk assessment

b. Establishment of OHSAS management system

c. Preparing internal regulations

D. Emergency training and exercises

560. Which of the following teams are required to receive emergency education?

a. Emergency response teams

b. Passive staff

c. Crisis management staff

D. Workplace safety team

561. Which of the following training topics should the emergency situation teams take?

There is no?

a. Earthquake protection and behavior training for employees of buildings and facilities

b. Hazardous chemicals, storage, handling, protection and transport training

c. Work accidents and occupational diseases education

D. Office workers fire and earthquake training

562. A new approach is introduced in which of the following is the chemical

The risks are claimed to be "the right of the people to know"?

a. Seveso II Directive

b. Risk Assessment Directive

c. Basel Directive

D. Environmental Impact Assessment Directive

563. Which of the following is not among the primary aims of the first-aider?

a. Lifting vital danger

b. Ensure business security

c. Facilitate healing

D. Ensure environmental safety

564. Which of the following are the basic practices of primary care?

a. Report

b. Rescue

c. Protect

D. Remove

565. Which of the following features does not exist first?

a. Must be calm

b. It must be hasty

c. Be knowledgeable

D. Must be confident

566. "abc" of first aid does not have any of the following?

a. Digestion

b. Circulation

c. Respiratory

D. Airline

567. According to the results of the risk assessment conducted by the employer in the workplace; Working methods, work

Organization and collective protection measures can not eliminate the risks in the workplace or

In cases where it can not be sufficiently reduced, Which of the following have to use?

a. Machine protectors

b. Personal protective equipment

c. Signs of safety and health

D. Instructions for operating machines

568. Which of the following is not a sign of safety and health?

A. Plate

B. Machine protective tape

C. Oral communication

D. Hand mark

569. Which of the following describes the prohibitive sign?

A. A sign that prohibits any conduct that would cause or threaten to cause harm

B. A sign that indicates a compulsory behavior

C. A warning sign about the situation that could cause or harm a hazard.

D. Emergency exit routes, signs giving information about first aid or rescue

570. Which of the following describes the warning sign?

A. Emergency exit routes, signs giving information about first aid or rescue

B. A warning sign about the situation that could cause or harm a hazard.

C. A sign that prohibits any conduct that would cause or threaten to cause harm

D. A sign that indicates a compulsory behavior

571. Which of the following describes the mandatory sign?

A. Emergency exit routes, signs giving information about first aid or rescue

B. A sign that prohibits any conduct that would cause or threaten to cause harm

C. A warning sign about the situation that could cause or harm a hazard.

D. A sign that indicates a compulsory behavior

572. Which of the following describes the sign board?

A. Made from transparent or translucent material, illuminated from inside or behind

Illuminated

B. A sign or a special condition used on an illuminated surface or

Shape that defines a specific behavior

C. Plate used together with a sign and providing additional information

D. It is created from geometric shapes, pictures, symbols, pictograms and colors,

Special information transmitting plate made visible with sufficient lighting

573. Which of the following is one of the basic qualities of prohibitive markings?

A. In the form of a circle, a black pictogram on a white background, a red frame and a diagonal line

B. Triangle-shaped, black pictogram on yellow background, black frame

C. A white pictogram on a green background in the shape of a rectangle or a square

D. White pictogram on a blue background in the form of a circle

574. Which of the following is the essential characteristics of emergency aid and first aid indications?

A. In the form of a circle, a black pictogram on a white background, a red frame and a diagonal line

B. A white pictogram on a green background in the shape of a rectangle or a square

C. White pictogram on a blue background in the form of a circle

D. Triangle-shaped, black pictogram on yellow background, black frame

575. Which of the following is one of the essential qualities of the mandatory sign?

A. White pictogram on a blue background in the form of a circle

B. A white pictogram on a green background in the shape of a rectangle or a square

C. Triangle-shaped, black pictogram on yellow background, black frame

D. In the form of a circle, a black pictogram on a white background, a red frame and a diagonal line

576. Which of the following is one of the basic qualities of warning signs?

A. In the form of a circle, a black pictogram on a white background, a red frame and a diagonal line

B. Triangle-shaped, black pictogram on yellow background, black frame

C. A white pictogram on a green background in the shape of a rectangle or a square

D. White pictogram on a blue background in the form of a circle

577. Which of the following is the comfort conditions of the living environment and breathing air?

A. Air humidity, mixture , temperature

B. Air humidity, air mixture

C. Air temperature, air humidity

D. Air mixture, air temperature

578. Which of the following is the air flow speed that can occur in the neighborhoods and disturb people?

A. 0.05-0.10 m / s

B. 0.20-0.30 m / s

C. 0.05-0.15 m / s

D. 0.10-0.20 m / s

579. Which of the following should be included in all personal protections?

A. It should be easy to find in the market.

B. The work dress must be portable in your pocket.

C. The employer should not place too much financial burden.

D. When necessary adjustments are made, they must be used exactly.

580. Which of the following features is not one of the personal protectors?

A. It should be in accordance with the health condition and ergonomic requirements of the employer.

B. It must carry the brand and title of the workplace.

C. It must comply with the current conditions at work.

D. Without additional risk, the relevant risk should be addressed.

581. The use of personal protective equipment is not dependent on which one of the following?

A. Risk grade

B. Exposure duration

C. Usage time

D. Shift scheme

582. Which of the following uses related to the personal protective equipment is wrong?

A. It must be given free by the employer.

B. Hygienic conditions should be maintained.

C. It must be cut from the wage of the worker.

D. The worker must be informed about the risks to which the worker is to be protected

583. Which of the following is true for training on the use of personal protective equipment?

A. Theoritical training should be given

B. Applied training should be given.

C. Voice training should be given.

D. Visual training should be given.

584. Which of the following related to the use of personal protective equipment by workers should be conducted?

A. They will use it occasionally.

B. They will use it in accordance with the instructions.

C. They will use it in every job.

D. Young workers will not use it.

585. Any defect or lack of personal protection seen by workers to whom

They should report?

A. Work safety expert

B. Workplace physician

C. Employer

D. Craftsman expert

586. Which of the following is effective for the use of personal protective equipment

Is not one of the topics to be mentioned in the trainings to be given to the workers?

A. What effect does the Personal Protector protect against

B. How to care for the personal protector and how to clean it

C. What is the standard number of the Personal Protector

D. How to use the Personal Protector

587. Which of the following is not one of the personal protective equipment types?

A. Respiratory system protectors

B. Head protectors

C. Eye and face protectors

D. Circulatory system protectors

588. Which one of the following is wrong with regards to plastic helmets?

A. They provide up to 600 volts of safety with the insulation feature.

B. It is used in iron-steel, mining, building, ship and tunnel construction.

C. Protects molten metal from heat.

D. It protects against impacts of 10-15 kg weight depending on fall distance.

589.When the plastic helmets are used well, how many years can they be used?

A. 4

B. 2

C. 3

D. 5

590. Which of the following is wrong with regard to aluminum helmets?

A. They can be used against crushing risk.

B. They can be used in places where there is a risk of electrical accidents.

C. They can be used in a hot working environment.

D. They can be used in oil wells, refineries and facilities operating with chemicals.

591. Helmets should be resistant to how many kg of the ball falls on 1.5 meters?

A. 3

B. 1

C. 2

D. 4

592. The high-level plastic helmet used in electrical work should show its protective properties without breaking up to how many volts?

A. 30,000

B. 60,000

C. 10,000

D. 20,000

593. How many dB of noise level in the working environment should the ear protectors use?

A. 90.

B. 85

C. 95

D. 75

594. Between How many micron dust masks can be used?

A. 0,2? 3

B. 0,2? 5

C. 0,2? 8

D. 0,2? 4

595. Which one of the following is wrong about the respiratory system protectors?

A. It should be cleaned after each use and after the filters have been removed.

B. It should not be used for more than three months.

C. It should be selected in accordance with the place of use and purpose of use.

D. The protection feature should be changed without losing filters.

596. Which of the following should be on personal protective equipment

Is it marked?

A. TSE

B. CE

C. TM

D. TS

597.Which type of shoe will be used as personal protective equipment in workplaces where explosives are manufactured?

A. Sparking shoes

B. Insulated shoes

C. Steel foot shoes

D. Finger protective shoes

598. Which of the following is the most important characteristic of personal protective equipment given to employees by the employer?

A. Use should be long-lasting

B. Must be in hygienic conditions

C. Must be made of plastic material

D. Must be cheap

599. What type of protective goggles should be used against electric arc rays?

A. Castor eyewear

B. Welder glasses

C. Transparent glass face

D. Normal glasses

600. What kind of gloves should be used to protect the hands against impact and compression?

A. Gloves reinforced by placing steel compounds in gloves

B. Gloves made of good fabric

C. Gloves with steel ferrules on their ends

D. Wire reinforced gloves

TEST 7

601. What kind of shoe should be used to protect toes in rolling and heavy work?

A. Anti-static footwear

B. Steel toe shoes

C. Insulation footwear

D. Spark-free shoes

602. What is called an equioment consisting of a device, a tool, or a material that is integrated by the manufacturer to protect the person against one or more risks at the same time?

A. Machine protectors

B. Personal protective

C. None

D. Technical equipment

603. Risks, could not be prevented by technical measures, to ensure the collective protection of work organization, which working methods will be used ?

A. Local ventilation

B. Ergonomic methods

C. Personal protective equipment

D. Machine protectors

604. Who is legally responsible in case the workers did not use personal protective equipment delivered in accordance with the risks and standards in the working environment?

A. Worker

B. Employer

C. Occupational safety engineer

D. Occupational health and safety board

605. Which of the following is the institution conducting market supervision and surveillance of personal protective equipment?

A. Chambers of Commerce

B. CSGB

C. Industry Chambers

D. TOBB

606. Which of the following is the abbreviated version of CE, which is enforced by the order of the Council of Ministers and specific in the "Regulation on the Attachment and Use of the Mark of Conformity to the Product"?

A. Common Era

B. Conformité Européene

C. Council Of Europe

D. China Export

607.Machine Safety Regulations do not cover which of the following?

A. Mine-well lifts

B. Partially completed machines

C. Lifting accessories

D. Safety features

608. Market surveillance and inspection of machinery covered by the Machinery Safety Directive

By which of the following institutions?

A. TOBB

B. The Ministry of Labor and Social Security

C. Ministry of Industry and Trade

D. MMO

639. Which one of the following is one of the gases causing explosive air formation? (D)

A. Nitrogen oxides          B. Hydrogen

C. Carbon dioxide          D. Methane

640. Which of the following is carbon monoxide, which is an unscented, odorless and tasteless gas Is it true?  (B)

A. It does not have explosive properties even at high concentrations.

B. It is a poisonous gas and dangerous.

C. It accumulates on the floor of the closed area because its density is heavy on the air.

D. It accumulates on the ceiling of the enclosed area because its density is light from the air.

641. Which one of the following is wrong for methane, which is an ungainly and odorless gas?  (A)

A. It's a poisonous gas.

B. It is collected on the ceiling of closed areas because it is light from air.

C. Mixing with air is called Grizu.

D. The disintegration of plant debris occurs as a result.

642. Which of the following is among the simple choking gases?  (B)

A. Hydrogen cyanide                B. Carbon dioxide

C. Hydrogen sulphate               D. Carbon monoxide

643. Which of the following is among chemical choking gases? (D)

A. Hydrogen                   B. Methane

C. Ethane                      D. Hydrogen sulphide

644. If the concentration of methane, which is a repellent and explosive gas, is less than% And it just burns?  (A)

A. 5                     B. 7

C. 9                     D. 11th

645. What is the concentration of methane gas that can be regarded as non-hazardous?  (B)

A. 10                       B. one

C. 4                        D. 7

646. In order to evaluate the air atmosphere as "low oxygen"

The amount should be at least%%?     (A)

A. 19,5                 B. 18

C. 20                   D. 17

647. Which of the following is wrong for carbon monoxide.       (C)

A. It is a colorless gas.

B. It is light from the air.

C. It is not explosive.

D. It's a poisonous gas.

648. Blood oxygenated hemoglobin cells (red blood cells) are carbon monoxide

How many times more is the propensity to oxygen than oxygen?     (A)

A. 300

B. 200

C. 400

D. one hundred

649. "To be Taken in Workplaces Employing Flammable, Explosive, Harmful and Hazardous Materials According to the Regulation on Measures ", for carbon monoxide to be in the working environment What is the maximum amount allowed (maximum)?        (D)

A. 30 ppm

B. 40 ppm

C. 20 ppm

D. 50 ppm

650.What is the percentage of 650.1000 ppm (%)?   (B)

A. 1

B. 0.001

C. 0.01

D. 0.1

651. Which of the following does not result in carbon monoxide formation?     (A)

A. Breathing

B. Operation of internal combustion engines

C. Burning of plastics

D. Fire

652.What is the% of nitrogen in normal air?     (C)

A. 58

B. 68

C. 78

D. 88

653. Which is correct for ammonia?    (B)

A. There is a sharp smell.

B. Causes of paralysis in the respiratory system.

C. Exposure limit value is lower than carbon monoxide limit value.

D. It is explosive at values ​​exceeding 15%

654. Critical factors which may cause an atmosphere to explode Is it? (D)

A. Oxygen concentration

B. Dustiness

C. No explosive gas

D. All

655. Which of the following can cause poisonous atmospheres in enclosed areas?   (B)

A. Welded work in closed area

B. All

C. Items stored in the indoor area

D. Indoor underwater painting

656. Which of the following is one of the hazards that can be found in enclosed spaces?     (C)

A. Explosive or poisonous atmosphere

B. Oxygen concentration is low or high atmospheres

C. All

D. Noise, temperature or slippery surfaces

657. What is the lowest oxygen rate that filter type respirators can use?      (B)

A. 21.5

B. 19,5

C. 20

D. 17

658. Which of the following is the most traumatic Is it?   (B)

A. Heavy cargo transportation

B. Excessive use of muscle power

C. Lifting material up

D. Respiratory tract irritation

659. One of the following compulsive traumas seen in working with shielded vehicles

Is not it?    (C)

A. Unacceptable posture

B. Eye fatigue

C. All of the drawbacks caused by dust and gases

D. All of the drawbacks caused by heat, humidity and lighting

660.Which of the following is not among the work center equipment?    (A)

A. Table

B. Hand tools

C. Telephone

D. Monitor

661. Which of the following is not about the trainings to be given to employees with screened vehicles?  (A)

A. Every year at least 6 hours should be given.

B. It should be given before starting work.

C. It should be given when there are significant changes in working conditions.

D. It should be repeated periodically.

662. Which of the following training topics will be especially given to employees with screened tools Is it between?  (C)

A. Principles of using personal protective equipment

B. First aid basic principles

C. Habit of short-lived eyes during work

D. Safety and health certificate

663. In which case should the eye examination of workers working with Echinacea vehicles be done?   (D)

A. When the occupational health and safety board decides

B. When a work safety specialist deems necessary

C. When the employer asks

D. Before starting work

664. Which of the following is not mandatory for monitors of European vehicles?    (D)

A. The character must be in shape and form that can be easily selected.

B. There must be enough space between the line and the characters.

C. The screen display should be stable.

D. It should be flat and plasma screen.

665. Which of the following is not true about the keyboards of the European instruments?       (C)

A. There will be enough space in front of the keyboard for the operator's hands and arms.

B. The location of the keyboard characters will be arranged to facilitate their use.

C. Keyboards should be according to the wishes and wishes of employees.

D. The color of the keyboard will be dull and will not reflect the light

666. Employer, in working centers, the compulsory use of screened vehicles

Do not you have to do the following with trauma?  (C)

A. Removing risks from the middle

B. Identifying risks

C. Providing personal protective equipment

D. Take precautions to reduce risks most

667. Which of the following training topics will be especially given to employees with screened tools? Is not between?   (D)

A. Compulsive trauma and ways of protection

B. Break rest

C. Eye protection

D. Protection of hands

668. Considering the risks of hand-borne injury, it is the lightest At what level should the loads be?    (C)

A. Between elbow-shoulder height

B. Between elbow-toe joint height

C. Above shoulder height

D. Between finger joint and lower leg middle height

669. Given the risk of injury from handover, the heaviest

At what level should the loads be?      (B)

A. Between lower leg middle-finger joint height

B. Finger joint - between elbow height

C. Below lower leg center height

D. Between elbow-shoulder height

670. Which of the following does not fall within the scope of individual risk factors for hand-carry-over?   (C)

A. Using unsuitable clothing, shoes or other personal items

B. Not having adequate and appropriate knowledge and training

C. Inadequate duration of call and listening

D. Inability to conduct physical work to do business

671.In the case of hand cargoes having the following characteristics,

Risk of injury may not occur?      (A)

A. If the body does not need to be bent or twisted

B. If it is unbalanced or the contents are being replaced

C. Very heavy and too big

D. If it is rough and difficult to grasp

672. Which of the following work environments have the following characteristics:

Increase?    (B)

A. If the working place is high enough to do the job

B. If temperature, humidity or ventilation is not suitable

C. If the working space is large enough to do the job

D. It is necessary to lower the load on the workplace floor or working floors

If there are no level differences

673. In which law is the work accident defined in our law?   (B)

A. Law No. 5763

B. Law No. 5510

C. Law No. 4857

D. Law No. 818

674. Which of the following is not a work accident according to SSI legislation?     (B)

A. When the worker went to work with the LPG

Bomb injuries

B. The end result of the worker's collision with a packed taxi on the way to work

C. If your worker falls off the bridge while discharging the load from the outside of the factory boundaries

D. Your worker falls in the office during the lunch break and breaks your foot

675.Which of the following is a work accident according to SGK legislation?      (D)

A. When a worker who is treated in a hospital due to a work accident goes home,

Injuries

B. When you pass by the service to go to the house of the descending worker,

Injury to another vehicle

C. The apple tree near the worker working in the road construction falls off the apple while the apple

Crush

D. Worker's workplace has a heart attack

676. Which of the following is a job accident classified according to the type of injury?   (C)

A. Missing foreign object

B. Accidents resulting from touching harmful and dangerous goods

C. Breast cuff and respiratory organs injury

D. Accidents that cause permanent incapacity to work

677. One of the following job accidents, classified according to the weight of the injury

Is not it?       (B)

A. Accidents resulting in death

B. Mental and nerve damage

C. Accidents that cause permanent incapacity to work

D. Accidents that will cause you to get away from work more than one day

678. According to GST statistics, in which sector is the mortality accidents most frequent in our country?       (B)

A. Metal                                   B. Build

C. Chemistry                              D. Textile

679. According to the statistics of the GST, the workplace accidents in our country the most business hours,

Is coming?    (D)

A. Second                            B. Seventh

C. Eighth                            D. First

680. Which of the following is the study that causes occupational diseases and occupational accidents

are not factors in your environment?

a. Mechanical

b. Personal

c. Chemical

D. Physical

681. Which of the following is physical?

a. jamming

b. Posture disorder

c. Noise

D. Static workload

682. Which of the following is chemical?

a. Radiation

b. crushing

c. Particles

D. Moisture

683. Which of the following is one of the direct (direct) causes of work accidents?

a. Chemical factors

b. Dangerous situation and dangerous movements

c. Ergonomic factors

D. Physical factors

684. Which of the following is insecure behavior causing the occurrence of work accidents

Is one?

a. Removing the machine protectors

b. Dangerous working method

c. Stacking at dangerous height

D. Pressure vessels not tested and tested

685. Which of the following is insecure situations that lead to the formation of occupational accidents

Is one?

a. Drowsiness and carelessness

b. Machines and workbenches

c. Not using the right machine for the job

D. Driving at dangerous speed

686. How many business days after the accident should the worker's accidents be reported to SSI Regional Directorate?

a. 3

b. 2nd

c. 5

D. one

687. Work accidents occurred within the number of workdays after the accident at the Regional Labor Office

It should be reported?

a. 2nd

b. 5

c. 3

D. one

688.When conducting a work accident investigation, which of the following purposes of interviewing witnesses

is not between?

a. Verifying what other witnesses are saying

b. Obtain opinions on how to protect from injury

c. Calculating the amount of compensation for those who caused the accident

D. Explain and confirm what is

689. How is the reactive approach used to prevent occupational accidents?

a. Measures are taken after a job accident.

b. Measures are taken without work accidents.

c. Ergonomic measures are taken.

D. Psychological precautions are taken.

690. How to implement a proactive approach to workplace accidents?

a. Measures are taken after a job accident.

b. Measures are taken without work accidents.

c. The work accident report is prepared.

D. Post-work analysis is done.

691. What is the other name of the cause and effect diagram used in the analysis of work accidents?

a. Team work

b. Brainstorming

c. Fishbone technique

D. Cronominal technique

692.Which reasons are not examined in the following main groups?

a. Method

b. Human

c. Material

D. Workplace

Which of the following is not one of the work accident investigation steps?

a. Preparation

b. Conclusion and conclusion

c. Media measurement and reporting

D. Preventive actions

694.If the employee (employee) is in the nature of the work they are performing,

temporary or permanent illness, disability, psychological malfunction

What are they called?

a. Loss of occupational power

b. Occupational disease

c. Occupational disease

D. Work accident

695. Which of the following is the occupational

are not factors?

a. Physiological factors

b. Physical factors

c. Biological factors

D. Chemical factors

696. According to the "SSK Regulation on Health Transactions", which of the following groups of occupational diseases

not involved?

a. Occupational infectious diseases

b. Occupational joint diseases

c. Occupational diseases with chemical agents

D. Occupational skin diseases

697. A study of the health problems of the workers in the kingdom and the book "De Re Metalica"

who is the author of the following?

a. Agricola

b. Nicand is

c. Plini

D. Ramazzini

698. "We have to ask what your illness is doing" and as the father of labor health

Which of the following is the accepted physician?

a. Agricola

b. Plini

c. Ramazzini

D. Nicand is

The first contract of the International Labor Organization, established in 699.1919, signed in 1919

Which of the following is related to?

a. Mine workers

b. Working conditions of child workers

c. Health examinations of workers

D. Female workers not working at night

700. According to the International Labor Organization (ILO) list, the number of occupational diseases

It is being assessed?

a. 10

b. 6

c. 4

D. 2nd

TEST 8

701. According to the International Labor Organization (ILO) list, which of the following occupations

diseases do not take place in the group?

a. Occupational diseases seen in target organs

b. Occupational diseases caused by occupational accidents

c. Occupational diseases caused by agents

D. Occupational cancers

702. In which legislation does our list of occupational diseases appear in our legislation?

a. Regulation on Social Insurance Transactions

b. About Workplace Health and Safety Units and Common Health and Safety Units

Regulations

c. Occupational Health and Safety Regulation

D. Social Insurance Health Transactions Regulation

703. Which of the following is not one of the characteristics of occupational diseases?

a. It shows very different clinical tables.

b. A wide variety of skin causes coloration.

c. There can be many different symptoms.

D. It is seen in many different business lines.

704. Which of the following do not have to be done to diagnose the diseases?

a. Routine laboratory analysis

b. Routine wage analysis

c. Getting a good professional anomnesis

D. Systemic physical examination

705. One of the following illnesses with occupational diseases hospital in our country

It is not?

a. Kocaeli

b. Zonguldak

c. Istanbul

D. Ankara

706. In our case, the authority to appoint a professional diagnosis is legally one of the following

It belongs?

a. Ministry of Health Hospitals

b. Social Security Institution Hospitals

c. Ministry of Labor and Social Security

D. Occupational Diseases Hospital

707. Which of the following is not included in the preventive measures for occupational diseases?

a. Personal protective measures

b. Workplace protection measures

c. Medical safeguards

D. Preventive measures against chemicals

708. The World Health Organization's "only known and recognized occupation

but not the disease, but the nature and mode of

which is an important factor among the causes "is defined by which of the following

It is concerned?

a. Occupational accidents

b. Occupational diseases

c. Work related accidents

D. Work related diseases

709. Which of the following is not one of the work-related diseases?

a. Peptic ulcer

b. Hypertension

c. pneumoconiosis

D. Mental diseases

710. Which of the following roles does not play a role in the emergence of occupational diseases?

a. Family and genetic factors

b. Bad habits

c. Life greyhound

D. Age factor

711. Which of the following is the workplace that causes chronic liver disease?

is not one of the factors?

a. Ammonia

b. Arsenic

c. White phosphorus

D. solvents

712. Which of the following is the workplace that causes chronic kidney disease

is one of the factors?

a. Nitro glucose

b. Carbon sulfur

c. Thallium

D. Mercury

713. Occupational health and safety units and common health and safety units

which is not obliged to do?

a. Surveillance of working environment

b. Periodic machine maintenance and control

c. Registration and statistics

D. First aid and emergency response

714. According to Article 77 of the Labor Law No. 4857,

What is the document requested by the Ministry and also by the Ministry, for example?

a. Business accident notification form

b. Risk assessment report

c. Control forms for pressure vessels

D. Urgent action plan

715. Which of the following documents does not provide an example in occupational health and safety legislation

is it one of the documents?

a. Occupational safety expertise contract

b. Entry health reports

c. Business document application and declaration form

D. Regulation prepared by the OSH Committees at workplaces

716. Regarding the inspection and testing of lifting devices used in the workplace

which one is right below

a. Every month is done periodically.

b. It should be done at least once a year and documented.

c. It should be done and documented every three months as long as it is used.

D. It is done once every three years.

717.What is the validity of the Business Security Professional Certificate?

a. Seven years shall be allowed for participation in renewal courses.

b. It is valid for an indefinite period.

c. It is valid for one year and visaisation by the General Directorate of Business Health every year

must

D. It is valid for seven years and after seven years,

Visa is re-opened to the Directorate.

718. In particular, the examination of workers who work in infra-

document editing required?

a. They are subjected to eye examinations.

b. They are subjected to a heart examination.

c. They are subjected to respiratory and respiratory tract examinations.

D. They are subjected to liver examination.

719. The "Regulations for Severe and Dangerous Work"

contains provisions?

a. alcoholics

b. Disability

c. women

D. Men

720.Which age group workers are prohibited from working in heavy and dangerous jobs?

a. Not 17 years old

b. Not 20 years old

c. Not 18 years old

D. Not 16 years old

721. Which one of the following should be done in order to work in heavy and dangerous work?

is it a legal obligation?

a. First aid training

b. Health report preparation

c. Ear protector installation

D. Participation in fire drill

722. For young workers who have not completed 18 years of age and are allowed to work in dangerous and dangerous work

in what period of time should the health examination be done?

a. In 6 months

b. In 9 months

c. In 12 months

D. In 3 months

723.Health examination for workers who have completed 18 years of age working in poultry and dangerous work

should be done in a while?

a. 3 months

b. 9 months

c. 6 months

D. 12 months

724. For health reports to be arranged for workers who will work in poultry and hazardous work

which one is right below

a. It should be taken from the workplace physician.

b. It should be taken from the internal medicine physician.

c. It should be taken from family physician.

D. It must be taken from the health board.

725. In which regulations are the jobs to which the women can not be employed?

a. Heavy and Dangerous Works Regulation

b. Human Rights Regulation

c. Women's Rights Regulation

D. Regulation on the Works that Women can not Work

726.Yönetmeliğin EK-I'inde verilen çizelgede yer almayan herhangi bir işin, ağır ve tehlikeli işlerden sayılıp sayılmadığına, hangi Bakanlık karar vermeye yetkilidir?

a. Sağlık Bakanlığı

b. Çalışma ve Sosyal Güvenlik Bakanlığı

c. Sanayi ve Ticaret Bakanlığı

d. Çevre ve Orman Bakanlığı

727.İş Yasasının 85. maddesine göre ağır ve tehlikeli işlerde çalışacaklar için aşağıdakilerden hangisi zorunludur?

a. Mesleki eğitim almış olmak

b. İlköğrenimi bitirmiş olmak

c. Yangın eğitimi almış olmak

d. Okuryazar olmak

728.İşveren veya vekili, ağır ve tehlikeli işlerde çalıştırdığı işçilerin hangi belgelerinin onaylı örneklerini saklayarak, bunları İş Müfettişlerinin her isteyişinde göstermekle yükümlüdür?

a. Evlenme cüzdanı

b. Operatör belgesi

c. Ehliyet

d. Nüfus cüzdanı

729.Aşağıdakilerden hangisi kansorejen madde tanımına uygundur?

a. Solunduğunda, ağız yoluyla alındığında, deriye nüfuz ettiğinde kanser oluşumuna neden olan veya kanser oluşumunu yavaşlatan maddeler

b. Solunduğunda, ağız yoluyla alındığında, deriye nüfuz ettiğinde kanser oluşumuna neden olan veya kanser oluşumunu hızlandıran maddeler

c. Solunduğunda, ağız yoluyla alındığında, deriye nüfuz ettiğinde kalıtımsal genetik hasarlara yol açmayan maddeler

d. Solunduğunda, ağız yoluyla alındığında, deriye nüfuz ettiğinde kalıtımsal genetik hasarlara yol açabilen veya bu etkinin oluşumunu hızlandıran maddeler

730.Kanserojen ve mutajen maddelere maruziyet riski bulunan işlerde yapılacak risk değerlendirmesinde aşağıdaki kriterlerden hangisi dikkate alınmaz?

a. Maruziyet miktarı

b. Maruziyet şekli

c. Maruziyet sayısı

d. Maruziyet süresi

731.Kanserojen ve mutajen maddelere maruziyet riski bulunan işlerde yapılacak risk değerlendirmesi her durumda en az kaç yılda bir yenilenmelidir?

a. 5

b. 3

c. 1

d. 10

732.Risk değerlendirmesi ile ilgi esaslar arasında aşağıdakilerden hangisi yoktur?

a. Risk değerlendirmesi yapılırken kanserojen ve mutajen maddelerle çalışmak istemeyenlerin bu isteklerini dikkate almalıdır.

b. Risk değerlendirmesinde kanserojen ve mutajen maddelerin, deri yolu da dahil olmak üzere vücuda giriş yollarının tümü dikkate alınmalıdır.

c. İşveren, risk değerlendirmesinde kullanılan bilgileri istenmesi halinde yetkili makamlara vermek zorundadır.

d. Risk değerlendirmesi beşli matris yöntemi ile yapılmalı ve her yıl tekrarlanmalıdır.

733.Kansorejen ve mutajen maddelerin kullanımın azaltılması ile ilgili olarak işverenler aşağıdaki kurallardan hangisine uymakla yükümlüdürler?

a. İşveren, kanserojen ve mutajen maddeleri işyeri sınırları dışında özel yapılmış depolarda özel kaplar içinde saklayacaktır.

b. İşveren, kanserojen ve mutajen maddeleri aldığı firmaların kalite yönetim sistemine sahip olmalarını isteyecektir.

c. İşveren, kanserojen veya mutajen maddelerin değiştirilmesi konusunda yapılan araştırma sonuçlarını, istenmesi halinde herkese vermek zorundadır.

d.İşveren, teknik olarak mümkün olduğu hallerde, işçilerin sağlığı ve güvenliği açısından kullanım koşullarında tehlikesiz veya daha az tehlikeli madde, karışım veya proses kullanarak işyerindeki kanserojen veya mutajen maddelerin kullanımını azaltacaktır.

734.Kanserojen veya mutajen maddelerle yapılan çalışmalarda maruziyetin önlenmesi ve azaltılması için işveren aşağıdakilerden hangisini yapmak zorunda değildir?

a. Kapalı sistemle çalışmanın teknik olarak mümkün olmadığı hallerde çalışanların maruziyeti mümkün olan en az düzeye indirilecektir.

b. Çalışanların kanserojen veya mutajen maddelere maruziyeti, Yönetmelik ekinde verilen sınır değerleri aşmayacaktır.

c. Yapılan risk değerlendirmesi sonucunda çalışanların sağlığı ve güvenliği yönünden risk bulunduğunun ortaya çıkması halinde çalışanların tehlikeli maddelere maruziyeti önlenecektir.

d. İşveren, kanserojen ve mutajen maddelere maruziyeti azaltmak için bu maddelerin kullanımını işyerinde uygun araçlarla yasaklayacaktır.

735.İşveren kanserojen veya mutajen maddelerin kullanıldığı işlerde aşağıdaki önlemlerden hangisini alacaktır?

a. İşyerinde kullanılacak kanserojen veya mutajen madde miktarı belirlenecek ve yapılan iş için gereken miktardan fazla madde bulundurulacaktır.

b. Kanserojen veya mutajen maddelerin kaynağından lokal veya genel havalandırma sistemi

veya diğer yöntemlerle çalışılan ortamdan dışarı atılması, halk sağlığı ve çevreye zarar verecek şekilde yapılacaktır.

c. Kanserojen veya mutajen maddelere maruz kalan veya kalabilecek işçi sayısı mümkün olan en çok sayıda olacaktır.

d. Kanserojen veya mutajen maddelerin çalışma ortamına yayılmasını önlemek veya en aza indirmek için proses tasarımı uygun şekilde yapılacak ve gerekli mühendislik kontrol önlemleri alınacaktır.

736. Is it not right for the employer to take the following precautions when carcinogenic or mutagenic substances are used?

a. If any accident or unexpected spread of carcinogenic or mutagenic substances occurs, appropriate measurement systems will be available for early detection of this condition.

b. Proper working methods and procedures will be used.

c. Workers working with carcinogenic and mutagenic substances will first provide suitable personal protective equipment with CE marking.

D. Workplace basins, walls and other surfaces will be cleaned regularly and hygiene conditions will be ensured.

737. When it is determined that there is a risk for the health and safety of the employees as a result of the risk assessment performed, the employer should not give sufficient information in the following cases when requested by the Ministry.

a. The use of these substances by processes and / or processes where carcinogens or mutagens are used in the workplace

b. Whether or not substitution can be made

c. The amount of carcinogenic or mutagenic substances and preparations and the substances containing them produced and used in the workplace

D. Material safety data sheets of carcinogenic mutagen substances used

738. In the event of an unexpected event or accident that could cause a high level of employee exposure, which of the following will not be done?

a. Workers who do not have protective gear and regressors will be allowed to work in the affected area.

b. Persons entering the affected area should wear protective clothing and breathing apparatus

It will be provided. Each worker will be operated as short as possible in the affected area, and this situation will not be continuous.

c. The employer will promptly inform the employees about the situation.

D. The affected area will only be allowed to enter by people who will do repairs and other obligatory work.

739.Will the employer take the following precautions for carcinogenic and mutagenic substances likely to be contaminated?

a. Workers will have adequate and sufficient bathing area, toilet and cleaning material.

b. Smoking, food and drink will be provided for employees who are likely to be contaminated with carcinogenic and mutagenic substances.

c. Employees will be given special protective clothing for work and a dressing room will be provided for each employee to store their own clothing and protective clothing items in separate places.

D. The protective equipment and tools shall be stored in special places and on suitable conditions.

740. Among the information and instructions to be given to workers who work with cancer and mutagenic substances, which of the following is not primarily available?

a. Use of protective materials and clothing

b. Risks that may affect health and additional risks to smoking

c. First aid basic principles

D. Precautions to be taken to avoid exposure

741.Will the employer provide information specifically to the following questions?

a. OHSAS 18001 Occupational Health and Safety Management System

b. Prices of carcinogens and mutagens used

c. Without prejudice to the employer's responsibility to determine the effectiveness of protective clothing and protective equipment, the selection, wearing and use of protective clothing materials and equipment shall determine the health and safety implications of employees

D. ISO 9001 quality management system

742. Which of the following is not subject to health surveillance of the workers?

a. According to the risk assessment, employees who work at risk of health and safety risks will make necessary arrangements in the workplace to keep their health conditions under surveillance.

b. If a health problem is suspected of being caused by exposure to carcinogens or mutagenic substances in one of the workers, the same persons who are exposed in the same way will also be taken through health surveillance.

c. Employees will be informed only about the necessary health surveillance during exposure.

D. Health surveillance will be carried out before the exposure starts and then at regular intervals.

743. Which organization will be notified of occupational diseases that have been exposed to carcinogenic or mutagenic substances?

a. The Ministry of Labor and Social Security

b. Ministry of Health

c. Occupational Diseases Hospital

D. Ministry of Environment and Forestry

744.With health check-ups and risk assessment, the updated list of employees in occupational health and safety risky jobs and their records of exposure status will be kept for at least several years after the expiration of the exposure?

a. 50

b. 10

c. 40

D. 20

745. Which ministry is authorized to decide on which business, trade, agriculture and forestry business is to be regarded as a business other than the one listed in the Annex of the "Regulation on the Works of Industry, Commerce, Agriculture and Forestry?"

a. The Ministry of Labor and Social Security

b. Ministry of Environment and Forestry

c. Ministry of Interior

D. industry and Trade Ministry

746. Which of the following is one of the jobs deemed industrial?

a. Purchase and sale of all kinds of raw, semi-finished and fully manufactured vegetable, animal or industrial products and goods

b. Warehousing, warehousing, warehousing and similar works

c. Works done in accommodation facilities such as hotel, hostel, camp, dormitory and guest house

D. Oil and gas exploration

747. Which of the following is not one of the jobs listed?

a. Dissolving and processing of vegetable oils

b. Finishing, painting, writing and printing works

c. Acquisition and sales of information and technology such as engineering, consultancy

D. Works in cold weather installations

748. Which of the following is one of the works deemed commercial?

a. Scientific research centers and lab work

b. Steam production and distribution for heating and energy

c. Waste and sewage services

D. Jewelery and jewelery manufacturing work

749. Which of the following is one of the agricultural and forestry jobs?

a. Operation of collection, liquidation, distribution, treatment and water networks of waters and maintenance and repair work carried out by the same enterprise

b. All kinds of business and income animals breeding, production, breeding and maintenance, guiding, training, cutting, milking and obtaining their products

c. Transportation of people or goods and animals on land, lakes and rivers

D. Agricultural fighting drug dealership jobs

750. Which of the following is not one of the risk groups in working life?

a. Children under 16

b. Workers less than 40 years old, over 18

c. Disability

D. Young girls over 20 and younger than 30

751. When adequate protection measures are taken, can female workers of all ages be employed in the following jobs?

a. Works related to the production of steam, gas and other exciting forces

b. Production and storage of chemical fertilizers

c. In the works done on the fully automated cold forming machines functioning with the magic force

D. Preparation and production of pyrotechnic materials

752. Which of the following statements is the correct one?

a. Women and young workers are allowed to work in health care work that is not suitable to their structure.

b. Apprentices and interns are obliged to be employed under the supervision of a master or formally responsible person.

c. It is not compulsory for young and healthy male workers to pass the health examination at work.

D. Child workers under the age of 14 must be physician-controlled to determine if they are healthy at work.

753. From what age groups of age groups will age over?

a. Older than 30 years

b. Older than 50 years

c. Older than 40 years

D. Older than 60 years

754. What is the rate of disabled persons in private workplaces where more workers are employed

Is it?

a. % 2nd

b. % 4

c. 5%

D. % 3

755. Which of the following studies is considered night work?

a. Starting at 16:00 hours, between 19:00 and 20:00 between meals and rest between 22:30 hours of work

b. Work starts at 11:00 am and ends at 20:00 with lunch between 15:00 and 16:00

c. Starts at 18:00 and works between 20:00 and 21:00, between dinner and rest, until 24:00

D. Starting at 02:30 and working between 06:00 and 07:00 between lunch and dinner and until 11:00

756. Do health examinations of workers who work in the night work have to be done at least every few years?

a. At least twice a year

b. At least once a year

c. At least once in three years

D. At least once in two years

757.What is the maximum number of hours for night work?

a. 6 hours

b. 8 hours

c. 7.5 hours

D. 8.5 hours

According to the 758. Work Law, the definition of "night" in working life starts at what time, what time does it last, and how many hours it lasts?

a. The earliest time is from 9:00 am to 5:00 pm at the latest and at most 9 hours per hour.

b. It is the period that starts at 20.00 hours at the earliest and lasts until 6:00 hours at the latest and lasts at most ten hours in any case.

c. It is the period that starts at 18.00 latest and lasts until 07.00 the earliest and lasts at most twelve hours in any case.

D. It is the period starting from 20:00 at the latest to 6:00 am at the earliest and lasting at most eleven hours in any case.

759. According to the Labor Code, at least how many posts are to be issued during the 24-hour period for jobs carried out by night laborers, one after the other, without interruption?

a. At least two mail are arranged to be run.

b. At least five mail is arranged to be run.

c. At least three mail are arranged to be run.

D. Edit at least four mail to be run

760. Which of the following must be done before starting work on the open ground, in terms of occupational health and safety?

a. Good planning

b. Supply of electricity to the working area first

c. Provide the settlement without taking environmental

D. Pre-installation of machinery and equipment

761. Which of the following is not one of the topics to be considered in the planning of the open work?

a. Distance to public establishments

b. Meteorology

c. High voltage power transmission lines

D. Communication

762. Which of the following should be done in the preparation of the field during the work carried out in open areas far from the settlement areas?

a. All

b. Roads

c. Helicopter track

D. Camping area

763. What points should be taken into consideration in the preparation of the camp sites during the studies conducted in open areas far from the settlement areas?

a. The campground should not be an area with prior permission for activity.

b. The camp site should not be in a position to cut off natural drainage channels.

c. The camp site should be kept as large as possible.

D. The camp site should be planned so that the use of the vehicle is as great as possible.

Which of the following are suitable for the legislative provisions in the studies conducted in open areas far from the settlements where 764,100 people work?

a. A cabin and urinal for every 30 male workers, and at least one toilet cabin for every 25 female workers

b. Construction of toilet cubicles, one for each 50 men and 50 women workers

c. Construction of toilet cabins, one for every 20 men and 20 women workers

D. Construction of toilet cabins, one for each 40 male and 40 female workers

765. Which one of the following is neuroses that are limited in time and lead to repetitive tasks?

a. Sleeping disorders

b. Anemia

c. Temperament disorders

D. Neural fatigue syndrome

766. Which of the following is not included in the concept of work safety?

a. Protection of employees

b. Protection of production

c. Protection of the operator

D. Protection of the economy

767. 10% of work accidents are due to:

a. Dangerous action

b. Dangerous situation

c. Thoughtfulness

D. Carelessness

68. Which of the following is not one of the objectives of the occupational health and safety management system?

a. To ensure production safety

b. Protect employees

c. Providing business security

D. Producing quality products

769.Which valve of oxygen tubes should not come in contact with the following substances?

a. Oil

b. Water

c. Paper

D. Powder

770.When adequate protection measures are taken, can female workers of all ages be employed in the following jobs?

a. Preparation and production of pyrotechnic materials

b. Works related to the production of steam, gas and other exciting forces

c. In the works done on the fully automated cold forming machines functioning with the magic force

D. Production and storage of chemical fertilizers

771. Which of the following is the basis for establishing the number of workers for establishing a health and safety board?

a. Number of workers belonging to the main employer and a sub-employer

b. All workers in the workplace

c. Number of employees in the contracting organization

D. Employer number of workers

772. What does the blue color mean according to the NFPA Indication System?

a. Flammability

b. All

c. reactivity

D. Health

773. Which of the following employment contracts is not in writing?

a. Borrowing contract

b. Work contract on call

c. Collective bargaining agreement

D. Team contract

774. Which of the following is the physician who examines the health problems of the workers in the millennium and is the author of the book "De Re Metalica"?

a. Nicand is

b. Agricola

c. Plini

D. Ramazzini

775. Which of the following is general safety precautions to be taken when working on electricity?

a. Using leakage current relay of 30mA on electric circuit

b. Working with grounded metal bars with high voltage bare conductors

c. Using a suitable diode on the electric circuit

D. Using a 16A fuse on the electric circuit

776.What training should the workers working in construction work have received?

a. literate

b. primary school

c. Vocational training

D. College

777. Which gas comes into contact with sodium in water?

a. Methane

b. Hydrogen

c. Nitrogen

D. Acetylene

778. How many meters of pipe attachments should be made for steel pipe stands?

a. 8

b. 7

c. 6

D. 5

779. "Regulations for Severe and Dangerous Work" includes provisions on which of the following groups of employees?

a. Disability

b. Men

c. alcoholics

D. women

780. Which of the following is known as the founder of workers' health?

a. Ramazzini

b. Agricola

c. Paraselsus

D. Nicand is

781. Which of the following is the type of negative behavior in terms of basic ethics and principles?

a. Doing business outside duty

b. Do not share information

c. Discrimination

D. Do not help your colleagues

782. Does not the hazard exits at workplaces have to be in accordance with:

a. On the roof of Yap

b. The height of the build

c. Made use class

D. User load

783. How many meters will not pass the length of the wooden moving ladders?

a. 6

b. 8

c. 2nd

D. 4

784. The ceiling height of the places where workers are permanently employed in workplaces should be at least how many meters?

a. 3

b. 4

c. 7

D. 5

785. Which of the following is the establishment of the health and safety establishment?

a. Union representative

b. Occupational physician

c. Job security specialist

D. Employer representative

786. Which of the following is a scientist who studies the interaction of people, equipment, environmental conditions and works to reduce and prevent the physical and psychosocial problems arising from this interaction?

a. Worker health

b. Ergonomics

c. Job security

D. psychology

787. What is the definition of "the risk that an establishment can tolerate according to legal obligations and its own occupational health and safety policy"?

a. Risk

b. Danger

c. Acceptable risk

D. Risk assessment

Which of the following is not true about the occupational health and safety services to be provided to workers?

a. Those who attend trainings are paid overtime.

b. Training time considers as working time.

c. Trainings should not put financial burden on workers.

D. Trainings are arranged outside the rest periods.

789. Which of the following is the definition of electricity?

a. The movement of electrons in a matter atom around its atomic nucleus is called electricity.

b. A physical phenomenon caused by static or dynamically charged particles.

c. The spread of sound waves in metals is called electricity.

D. A permanent magnet is called electricity to attract metals.

790. Which of the following is not one of the topics to be considered in the planning of the work to be done openly?

a. Distance to public establishments

b. Meteorology

c. Communication

D. High voltage power transmission lines

791. How many ILO conventions is the "International Labor Contract for Work Inspection in Industry and Trade"?

a. 116

b. 81

c. 102

D. 91

792. What is the other name of the cause-effect diagram used in the analysis of work accidents?

a. Fishbone technique

b. Cronominal technique

c. Brainstorming

D. Team work

79. Which of the following is not one of the dangerous movements?

a. Using tools and machines dangerously

b. Unsafe loading, handling and stacking

c. Maintenance-free machine

D. Take an insecure status

794. Which of the following is not the measures to be taken by the employer when the noise exposure reaches the highest exposure effective values ​​or exceeds these values?

a. Personal protective use

b. Isolation of noise source

c. Worker overtime paid to noise

D. Cutting at the source of noise

795. Which of the work environments with the following characteristics can increase the risk of back injuries?

a. If the working place is high enough to do the job

b. If there is no level difference between the workplace floor and the working floors which necessitates the loading and unloading of the loads

c. If temperature, humidity or ventilation is not available

D. If the working space is large enough to do the job

796. Which sector in our country has the highest number of occupational accidents resulting in death?

a. Textile

b. Build

c. Metal

D. Mine

797. What kind of rules are included in business law?

a. Absolute imperative rules

b. Descriptive rules

c. Relative rules

D. Supplementary rules

798. Which of the following training topics does the emergency situation teams need to take?

a. Earthquake protection and behavior training for employees of buildings and facilities

b. Work accidents and occupational diseases education

c. Office workers fire and earthquake training

D. Hazardous chemicals, storage, handling, protection and transportation training

799. How many hours can be less than the total number of trainings in terms of theoretical and practical training?

a. 220

b. 160

c. 300

D. 180

800. In what age group is it prohibited to employ workers in heavy and dangerous jobs?

a. Not 18 years old

b. Not 16 years old

c. Not 20 years old

D. Not 17 years old

TEST 9

801. Which of the following tasks does the health and safety units of the workplace and the tasks of the common health and safety units have?

a. Health surveillance of workers

b. Surveillance of working environment

c. Fire alarm and exercise

D. Education, counseling and information

802. Which of the following documents is unlisted in occupational health and safety legislation?

a. Occupational safety expertise contract

b. Entry health reports

c. Regulation prepared by the OSH Committees at workplaces

D. Business document application and declaration form

803. Which body is authorized to remove the blood?

a. Turkish Grand National Assembly

b. Council of Ministers

c. ministries

D. Premiership

804. Which of the following is not one of the basic features of enclosures and protection devices?

a. Must be a minimum obstacle to monitoring production process

b. Must be located far enough away from danger zone

c. No additional hazards should arise

D. Easily deactivated or easily bypassed

805.What is the lower limit of the blast explosion?

a. % 2nd

b. % 4

c. % one

D. % 3

806. In what frequency range can people perceive vibrations?

a. 1 kHz to 10 kHz

b. 0.01 Hz to 0.1 Hz

c. 0.1 Hz to 1 Hz

D. 1 Hz to 1 kHz

807. Which of the following is not one of the main objectives of health and safety at work?

a. Protection of employees

b. Protection of the operator

c. Protection of rights

D. Protection of production

808. How many main topics do the harms of the health of the chemicals collect?

a. 11th

b. 7

c. 9

D. 5

809. How many kilograms can the workplace ladders carry at least a square kilometer?

a. one thousand

b. 500

c. 300

D. 700

810. Which of the following workers should be given special attention to education?

a. engineers

b. foremen who

c. masters

D. Disability

811.Doing, transferring, storing, etc. the places where rough works are done and the inner passages, corridors, roads and stairs should be illuminated with at least how many luxes?

a. one hundred

b. 70

c. 50

D. 90

812.What kind of examination should be done and documented, especially for workers who work in infra-red rays?

a. They are subjected to liver examination.

b. They are subjected to respiratory and respiratory tract examinations.

c. They are subjected to eye examinations.

D. They are subjected to a heart examination.

813. Which of the following is true regarding the emergency exit doors?

a. Locks will be found in the authority.

b. It will not be closed or locked.

c. Iple will be connected as easy to open.

D. It will be closed or locked.

814. Which of the following is not one of the characteristics of the rules of law?

a. To be constant

b. Generally

c. Abstract

D. Being private

815. How many workers can be employed in construction work?

a. 17

b. 16

c. 18

D. 19

816. Which of the following is one of the works deemed commercial?

a. Waste and sewage services

b. Steam production and distribution for heating and energy

c. Jewelery and jewelery manufacturing work

D. Scientific research centers and lab work

817. Whichever of the following can be entered into the excipient storage area?

a. Mobile phone

b. Notebook

c. Lighter

D. Radio

818.What kind of group does the building work according to "Communiqué on Hazard Classes Regarding Occupational Health and Safety"?

a. Very dangerous

b. Dangerous

c. Less dangerous

D. Safe

819. Which one of the personal protective equipment to work should be given to the worker who drives the gullwind pinch?

a. Glasses

b. Safety belt

c. Gasmask

D. Face order

820.Which of the following is the issue of health and safety between board members

take place?

a. Safety precautions in welding work

b. Occupational diseases

c. Safety precautions for lifting machines

D. Safety precautions in casting works

821. Which of the following is one of the beams emitted from the source behind?

a. Ultraviolet rays

b. Magnetic rays

c. Cosmic rays

D. X-rays

822. In what way are the biological risk factors most likely to be taken into our bodies?

a. Inhaled by inhalation

b. If not infected, digestive

c. Do not work with proper work clothes in well-avoided environments

D. Through the skin when not in contact

823 Abbreviated European Treaty on the Transport of Dangerous Goods by Road

Which of the following is it?

a. TMT

b. ADR

c. ETR

D. DGR

824. How many% of methane is detected in the measured values, the sewers can not be filled?

a. 5

b. 3

c. one

D. 2nd

How many months should the 825.CO2 fire extinguishers be checked?

a. 9

b. 12

c. 6

D. 3

826. Which of the following is not one of the national organizations on occupational health and safety?

a. Labor Inspection Board

b. TİSK

c. Turkish Red Crescent Society

D. Turkish National Union of Tables

827. How to implement a proactive approach to workplace accidents?

a. Measures are taken without work accidents.

b. Measures are taken after a job accident.

c. The work accident report is prepared.

D. Post-work analysis is done.

828. In our case, the authority to appoint a professional diagnosis is legally one of the following

It belongs?

a. Ministry of Health Hospitals

b. Occupational Diseases Hospital

c. Social Security Institution Hospitals

D. Ministry of Labor and Social Security

829. How many kilograms should not be placed on a square meter in suspension bridges?

a. 300

b. 500

c. 400

D. one thousand

Which of the following is not one of the board members of health and safety?

a. Site chief

b. Worker representative

c. Occupational physician

D. Master representative

831.If loads with the following characteristics are carried by hand,

risk of injury may not occur?

a. If it is rough and difficult to grasp

b. Too heavy and too big

c. If the body does not need to be bent or twisted

D. If it is unbalanced or the contents are being replaced

Damage suffered as a result of labor accidents; for work health and work safety measures

how many times are the payments?

a. 5

b. 4

c. 10

D. 2nd

Which of the following is not a work accident according to SGK legislation?

a. If your worker falls off the bridge while discharging the car from the factory outside the factory boundaries

b. Your worker falls in the office during the lunch break and breaks your foot

c. When the worker went to work with the LPG

bomb injuries

D. The end result of the worker's collision with a packed taxi on the way to work

834. According to the results of the risk assessment made by the employer at the workplace; working methods, work

organization and collective protection measures, the risks in the workplace can not be remedied or

in cases where it can not be sufficiently reduced,

have to use?

a. Signs of safety and health

b. Machine protectors

c. Personal protective equipment

D. Instructions for operating machines

In the year of 1802, the Parliament took the initiative and called "Health and Morality of the Apprentices"

who provides the removal of the law?

a. Thomas Percival

b. Robert Owen

c. Sir Robert Peel

D. Antony Ashly Cooper

How much of the tests of 836.Receiving-Forwarding machines (forklift, crane, etc.) periodically

need to be done in time?

a. Quarterly

b. Bimonthly

c. Once in a month

D. Once in four months

837.While taking precautions related to the protection of health and safety,

are not among the general principles?

a. Prevention of risks

b. Fighting in the source with risks

c. Worker's cautious and careful behavior

D. Assessing risks that can not be avoided

838. Profession emerging as a consequence of prolonged inhalation of dust, gas and fumes containing meat

Which of the following is the disease?

a. Allergy

b. pneumoconiosis

c. Asthma

D. Flu

839. How much is the amount of nitrogen in normal air?

a. 78

b. 68

c. 88

D. 58

840.Barettes should be resistant to how many kg of the ball falls on 1.5 meters?

a. 2nd

b. 3

c. 4

D. one

According to the 841.Makina Safety Regulation; lifting accessories and their accessories

the operating coefficient of the reading rope or slings as a general rule

which is equal?

a. 3

b. 5

c. 7

D. one

842. Which of the following is physical effect on people who are tired (hearing loss)

is not one of the factors that influence the formation of

a. Having the hearing impaired exposed to noise

b. Sex exposed to noise

c. Time to be affected by noise

D. The intensity of the voice that brings the noise to the square

843. Multiple businesses, such as factories, establishments, enterprises or groups of businesses,

which are related to establishment of occupational health and safety

Is this true?

a. Establishment of a health and safety board is not compulsory.

b. Occupational health and safety is established with the participation of one representative from each workplace.

c. Occupational health and safety board is established based on the total number of workers.

D. In each workplace with a large number of workers, a separate occupational health and safety board

Established.

844. Which of the following judicial decisions is binding at the level of the law?

a. Decision of the Council of State

b. Decision of the Supreme Court of Appeal

c. Heavy Penal Court Decision

D. Decision of the Supreme Court of Appeals

845. How many micron-sized solid particles are called dust?

a. one hundred

b. 250

c. 50

D. 150

846. How is the reactive approach used to prevent occupational accidents?

a. Psychological precautions are taken.

b. Ergonomic measures are taken.

c. Measures are taken after a job accident.

D. Measures are taken without work accidents.

847. The institution that conducts market surveillance and oversight of personal protective equipment

is it from the following?

a. MLSS

b. Chambers of Commerce

c. TOBB

D. Industry Chambers

848. What is the concept of altitude defined by the following sentences?

a. Locations that are held by hand and are dangerous to fall are called high.

b. It is called high 5 meters high which can be reached with the help of others.

c. It is at least 7 meters high, which can be left without driving.

D. Places that we can not easily get off in one step can be defined as high.

849. Among the information and instructions to be given to workers who work with cancer and mutagenic substances

which of the following is not primarily?

a. Use of protective materials and clothing

b. First aid basic principles

c. Risks that may affect health and additional risks to smoking

D. Precautions to be taken to avoid exposure

850. The employer shall not disclose the personal health files of the workers for the number of years

do you have to hide it?

Choose an answer.

a. 5

b. 40

c. 20

D. 10

851. Market surveillance and inspection of machinery covered by the Machinery Safety Directive

by which of the following institutions?

a. TOBB

b. MMO

c. The Ministry of Labor and Social Security

D. industry and Trade Ministry

852.Which of the following is the type of arrivals of business accidents on construction work?

Is not one?

a. Material drop

b. Machinery and equipment accident

c. Excavation ditches

D. Personal protective equipment

853. Which of the following is not one of the safety conditions of working at high altitude?

A. Wearing steel toe shoes

b. Seat belt use

c. Scarecrow

D. Establishing a safety net

854. The World Health Organization's "only known and accepted occupation"

But not on diseases, but on the development and development of working environment and working

Diseases are an important factor among the causes "is defined by which of the following

It is concerned?

a. Work related accidents

b. Occupational diseases

C. Work related diseases

D. Occupational Accidents

855.According to the health check-ups and risk assessment, employees' health and safety

A list of employees in risky jobs and their exposure

Records indicating their condition should be kept for at least several years after the expiration date

It will be stored?

a. 20

B. 40

c. 50

D. 10

856. For health reports to be arranged for workers who will work in poultry and hazardous work

Which one is right below

A. It should be taken from the workplace physician.

b. It should be taken from the health institution.

c. It should be taken from family physician.

D. It should be taken from the internal medicine doctor.

857. What is the minimum oxygen concentration allowed in a coal mine?

A. 19

b. 21

c. 18

D. 16

858. Which of the following describes the warning sign?

a. A sign that indicates a compulsory behavior

b. A sign that prohibits any conduct that would cause or threaten to cause harm

C. A warning sign about the situation that could cause or harm a hazard.

D. Emergency exit routes, signs giving information about first aid or rescue

859. Which of the following is not included in the definition of hazardous chemicals?

a. Allergic

b. oxidizing

c. Toxic

D. Flammable

860. Occupational health and safety units should have one of the following

It is not?

a. Inspection, first aid and emergency room

B. Room of business physician

c. Standby

D. Occupational safety specialist room

861.Which is the highest limit value for the small voltage?

A. Voltages with a maximum voltage of 50 volts are called small voltages.

b. Voltages with a maximum voltage of 100 volts are called small voltages.

c. Voltages with a maximum voltage of 75 volts are called small voltages.

D. Voltages of up to 25 volts are called small voltages.

862. According to the International Labor Organization (ILO) list, the number of occupational diseases

It is being assessed?

a. 10

b. 6

C. 4

D. 2nd

863. Which of the following is not one of the employer's obligations?

a. To educate workers about occupational risks

b. audit

C. Participate in occupational health and safety training

D. To inform workers about legal rights and responsibilities

864. Should the work-related accidents be notified within the number of working days after the accident of SSI Regional Directorate?

a. 2nd

B. 3

c. one

D. 5

865.Every step on the escape ladder height .......... cm.

Width cannot be less than .......... cm? Den.

What is the value to be written in the dotted area?

a. 12-14 cm

B. 18-20 cm

c. 14-16 cm

D. 10-12 cm

866. Which of the following classes does carbon dioxide gas enter by classroom effect?

A. Simple mufflers

b. Systemic poisons

c. Chemical mufflers

D. Irritants

867. Which institution is more concerned with occupational health and safety as duties and responsibilities?

a. SS institution

b. Ministry of Health

C. The Ministry of Labor and Social Security

D. Ministry of Industry

868. Which of the following works falls under the labor law?

A. Establishments or businesses where agriculture and forestry work is undertaken with 50 or more workers

b. 3 persons working in accordance with the description of Article 2 of the Artisans and Tradesman Law No. 507

workplaces

c. Home services

D. Sea and air transport work

869. Which of the following jobs would work for only 7.5 hours per day?

A. Oxygen and power supply workers

b. insecticides

c. Carbon sulfur works

D. Works also seen in mercury smelting furnaces

870. When working with European vehicles, the health and safety regulations are

Apply?

a. On screen typewriters

B. In computer systems used in work centers

c. Computer systems open to the use of society

D. On computer systems in transport vehicles

871. Contains unauthorized radioactive material, if within the limits of

Import, export, manufacture, possession, use and storage of materials are prohibited

Which of the following are the materials?

a. Cosmetics

B. All

c. garments

D. stationery;’/

872.Ekranl in working with vehicle regulations about health and safety measures

The maximum number of working centers that are active on the published date shall be

It is imperative that their provisions be adapted?

A. 2nd

b. one

c. 4

D. 3

773. Which of the following employment contracts is not in writing?

a. Borrowing contract

b. Work contract on call

c. Collective bargaining agreement

D. Team contract

774. Which of the following is the physician who examines the health problems of the workers in the millennium and is the author of the book "De Re Metalica"?

a. Nicand is

b. Agricola

c. Plini

D. Ramazzini

775. Which of the following is general safety precautions to be taken when working on electricity?

a. Using leakage current relay of 30mA on electric circuit

b. Working with grounded metal bars with high voltage bare conductors

c. Using a suitable diode on the electric circuit

D. Using a 16A fuse on the electric circuit

776.What training should the workers working in construction work have received?

a. literate

b. primary school

c. Vocational training

D. College

777. Which gas comes into contact with sodium in water?

a. Methane

b. Hydrogen

c. Nitrogen

D. Acetylene

778. How many meters of pipe attachments should be made for steel pipe stands?

a. 8

b. 7

c. 6

D. 5

779. "Regulations for Severe and Dangerous Work" includes provisions on which of the following groups of employees?

a. Disability

b. Men

c. alcoholics

D. women

780. Which of the following is known as the founder of workers' health?

a. Ramazzini

b. Agricola

c. Paraselsus

D. Nicand is

781. Which of the following is the type of negative behavior in terms of basic ethics and principles?

a. Doing business outside duty

b. Do not share information

c. Discrimination

D. Do not help your colleagues

782. Does not the hazard exits at workplaces have to be in accordance with:

a. On the roof of Yap

b. The height of the build

c. Made use class

D. User load

783. How many meters will not pass the length of the wooden moving ladders?

a. 6

b. 8

c. 2nd

D. 4

784. The ceiling height of the places where workers are permanently employed in workplaces should be at least how many meters?

a. 3

b. 4

c. 7

D. 5

785. Which of the following is the establishment of the health and safety establishment?

a. Union representative

b. Occupational physician

c. Job security specialist

D. Employer representative

786. Which of the following is a scientist who studies the interaction of people, equipment, environmental conditions and works to reduce and prevent the physical and psychosocial problems arising from this interaction?

a. Worker health

b. Ergonomics

c. Job security

D. psychology

787. What is the definition of "the risk that an establishment can tolerate according to legal obligations and its own occupational health and safety policy"?

a. Risk

b. Danger

c. Acceptable risk

D. Risk assessment

Which of the following is not true about the occupational health and safety services to be provided to workers?

a. Those who attend trainings are paid overtime.

b. Training time considers as working time.

c. Trainings should not put financial burden on workers.

D. Trainings are arranged outside the rest periods.

789. Which of the following is the definition of electricity?

a. The movement of electrons in a matter atom around its atomic nucleus is called electricity.

b. A physical phenomenon caused by static or dynamically charged particles.

c. The spread of sound waves in metals is called electricity.

D. A permanent magnet is called electricity to attract metals.

790. Which of the following is not one of the topics to be considered in the planning of the work to be done openly?

a. Distance to public establishments

b. Meteorology

c. Communication

D. High voltage power transmission lines

791. How many ILO conventions is the "International Labor Contract for Work Inspection in Industry and Trade"?

a. 116

b. 81

c. 102

D. 91

792. What is the other name of the cause-effect diagram used in the analysis of work accidents?

a. Fishbone technique

b. Cronominal technique

c. Brainstorming

D. Team work

793. Which of the following is not one of the dangerous movements?

a. Using tools and machines dangerously

b. Unsafe loading, handling and stacking

c. Maintenance-free machine

D. Take an insecure status

794. Which of the following is not the measures to be taken by the employer when the noise exposure reaches the highest exposure effective values ​​or exceeds these values?

a. Personal protective use

b. Isolation of noise source

c. Worker overtime paid to noise

D. Cutting at the source of noise

795. Which of the work environments with the following characteristics can increase the risk of back injuries?

a. If the working place is high enough to do the job

b. If there is no level difference between the workplace floor and the working floors which necessitates the loading and unloading of the loads

c. If temperature, humidity or ventilation is not available

D. If the working space is large enough to do the job

796. Which sector in our country has the highest number of occupational accidents resulting in death?

a. Textile

b. Build

c. Metal

D. Mine

797. What kind of rules are included in business law?

a. Absolute imperative rules

b. Descriptive rules

c. Relative rules

D. Supplementary rules

798. Which of the following training topics does the emergency situation teams need to take?

a. Earthquake protection and behavior training for employees of buildings and facilities

b. Work accidents and occupational diseases education

c. Office workers fire and earthquake training

D. Hazardous chemicals, storage, handling, protection and transportation training

799. How many hours can be less than the total number of trainings in terms of theoretical and practical training?

a. 220

b. 160

c. 300

D. 180

800. In what age group is it prohibited to employ workers in heavy and dangerous jobs?

a. Not 18 years old

b. Not 16 years old

c. Not 20 years old

D. Not 17 years old

TEST 9

801. Which of the following tasks does the health and safety units of the workplace and the tasks of the common health and safety units have?

a. Health surveillance of workers

b. Surveillance of working environment

c. Fire alarm and exercise

D. Education, counseling and information

802. Which of the following documents is unlisted in occupational health and safety legislation?

a. Occupational safety expertise contract

b. Entry health reports

c. Regulation prepared by the OSH Committees at workplaces

D. Business document application and declaration form

803. Which body is authorized to remove the blood?

a. Turkish Grand National Assembly

b. Council of Ministers

c. ministries

D. Premiership

804. Which of the following is not one of the basic features of enclosures and protection devices?

a. Must be a minimum obstacle to monitoring production process

b. Must be located far enough away from danger zone

c. No additional hazards should arise

D. Easily deactivated or easily bypassed

805.What is the lower limit of the blast explosion?

a. % 2nd

b. % 4

c. % one

D. % 3

806. In what frequency range can people perceive vibrations?

a. 1 kHz to 10 kHz

b. 0.01 Hz to 0.1 Hz

c. 0.1 Hz to 1 Hz

D. 1 Hz to 1 kHz

807. Which of the following is not one of the main objectives of health and safety at work?

a. Protection of employees

b. Protection of the operator

c. Protection of rights

D. Protection of production

808. How many main topics do the harms of the health of the chemicals collect?

a. 11th

b. 7

c. 9

D. 5

809. How many kilograms can the workplace ladders carry at least a square kilometer?

a. one thousand

b. 500

c. 300

D. 700

810. Which of the following workers should be given special attention to education?

a. engineers

b. foremen who

c. masters

D. Disability

811.Doing, transferring, storing, etc. the places where rough works are done and the inner passages, corridors, roads and stairs should be illuminated with at least how many luxes?

a. one hundred

b. 70

c. 50

D. 90

812.What kind of examination should be done and documented, especially for workers who work in infra-red rays?

a. They are subjected to liver examination.

b. They are subjected to respiratory and respiratory tract examinations.

c. They are subjected to eye examinations.

D. They are subjected to a heart examination.

813. Which of the following is true regarding the emergency exit doors?

a. Locks will be found in the authority.

b. It will not be closed or locked.

c. Iple will be connected as easy to open.

D. It will be closed or locked.

814. Which of the following is not one of the characteristics of the rules of law?

a. To be constant

b. Generally

c. Abstract

D. Being private

815. How many workers can be employed in construction work?

a. 17

b. 16

c. 18

D. 19

816. Which of the following is one of the works deemed commercial?

a. Waste and sewage services

b. Steam production and distribution for heating and energy

c. Jewelery and jewelery manufacturing work

D. Scientific research centers and lab work

817. Whichever of the following can be entered into the excipient storage area?

a. Mobile phone

b. Notebook

c. Lighter

D. Radio

818.What kind of group does the building work according to "Communiqué on Hazard Classes Regarding Occupational Health and Safety"?

a. Very dangerous

b. Dangerous

c. Less dangerous

D. Safe

819. Which one of the personal protective equipment to work should be given to the worker who drives the gullwind pinch?

a. Glasses

b. Safety belt

c. Gasmask

D. Face order

820.Which of the following is the issue of health and safety between board members

take place?

a. Safety precautions in welding work

b. Occupational diseases

c. Safety precautions for lifting machines

D. Safety precautions in casting works

821. Which of the following is one of the beams emitted from the source behind?

a. Ultraviolet rays

b. Magnetic rays

c. Cosmic rays

D. X-rays

822. In what way are the biological risk factors most likely to be taken into our bodies?

a. Inhaled by inhalation

b. If not infected, digestive

c. Do not work with proper work clothes in well-avoided environments

D. Through the skin when not in contact

823 Abbreviated European Treaty on the Transport of Dangerous Goods by Road

Which of the following is it?

a. TMT

b. ADR

c. ETR

D. DGR

824. How many% of methane is detected in the measured values, the sewers can not be filled?

a. 5

b. 3

c. one

D. 2nd

How many months should the 825.CO2 fire extinguishers be checked?

a. 9

b. 12

c. 6

D. 3

826. Which of the following is not one of the national organizations on occupational health and safety?

a. Labor Inspection Board

b. TİSK

c. Turkish Red Crescent Society

D. Turkish National Union of Tables

827. How to implement a proactive approach to workplace accidents?

a. Measures are taken without work accidents.

b. Measures are taken after a job accident.

c. The work accident report is prepared.

D. Post-work analysis is done.

828. In our case, the authority to appoint a professional diagnosis is legally one of the following

It belongs?

a. Ministry of Health Hospitals

b. Occupational Diseases Hospital

c. Social Security Institution Hospitals

D. Ministry of Labor and Social Security

829. How many kilograms should not be placed on a square meter in suspension bridges?

a. 300

b. 500

c. 400

D. one thousand

Which of the following is not one of the board members of health and safety?

a. Site chief

b. Worker representative

c. Occupational physician

D. Master representative

831.If loads with the following characteristics are carried by hand,

risk of injury may not occur?

a. If it is rough and difficult to grasp

b. Too heavy and too big

c. If the body does not need to be bent or twisted

D. If it is unbalanced or the contents are being replaced

832. Damage suffered as a result of labor accidents; for work health and work safety measures

how many times are the payments?

a. 5

b. 4

c. 10

D. 2nd

833. Which of the following is not a work accident according to SGK legislation?

a. If your worker falls off the bridge while discharging the car from the factory outside the factory boundaries

b. Your worker falls in the office during the lunch break and breaks your foot

c. LPG explosion in the wake of the workday after the work is over at the place where the employee went to work

D. The end result of the worker's collision with a packed taxi on the way to work

834. According to the results of the risk assessment made by the employer at the workplace; work methods, work organization and collective protection measures, where the risks in the workplace can not be remedied or reduced sufficiently, which of the following should be kept and used appropriately?

a. Signs of safety and health

b. Machine protectors

c. Personal protective equipment

D. Instructions for operating machines

Who was the initiator of Parliament in 835. In 1802, who gave the law called "Health and Morale of the Apprentices"?

a. Thomas Percival

b. Robert Owen

c. Sir Robert Peel

D. Antony Ashly Cooper

836. How long will it be necessary to periodically carry out the tests of the Rope-Forwarding Machines (forklift, crane, etc.)?

a. Quarterly

b. Bimonthly

c. Once in a month

D. Once in four months

837. Which of the following is not included among the general principles in taking measures for the protection of health and safety?

a. Prevention of risks

b. Fighting in the source with risks

c. Worker's cautious and careful behavior

D. Assessing risks that can not be avoided

838. Which of the following is the occupational disease resulting from prolonged inhalation of metal-containing dust, gas and fumes?

a. Allergy

b. pneumoconiosis

c. Asthma

D. Flu

839. How much is the amount of nitrogen in normal air?

a. 78

b. 68

c. 88

D. 58

840.Barettes should be resistant to how many kg of the ball falls on 1.5 meters?

a. 2nd

b. 3

c. 4

D. one

According to the 841.Makina Safety Regulation; lifting accessories and their components, the working principle of the woven rope or slings is, as a general rule, equal to:

a. 3

b. 5

c. 7

D. one

842. Which of the following factors is not one of the factors that affect the physical impact (hearing loss) on people who are noisy?

a. Having the hearing impaired exposed to noise

b. Sex exposed to noise

c. Time to be affected by noise

D. The intensity of the voice that brings the noise to the square

843. In cases where there are multiple workplaces, such as factories, establishments, enterprises or groups of businesses, which are employees, which of the following is true about establishing occupational health and safety board?

a. Establishment of a health and safety board is not compulsory.

b. Occupational health and safety is established with the participation of one representative from each workplace.

c. Occupational health and safety board is established based on the total number of workers.

D. A separate occupational health and safety board is set up at each workplace where there are too many workers.

844. Which of the following judicial decisions is binding at the level of the law?

a. Decision of the Council of State

b. Decision of the Supreme Court of Appeal

c. Heavy Penal Court Decision

D. Decision of the Supreme Court of Appeals

845. How many micron-sized solid particles are called dust?

a. one hundred

b. 250

c. 50

D. 150

846. How is the reactive approach used to prevent occupational accidents?

a. Psychological precautions are taken.

b. Ergonomic measures are taken.

c. Measures are taken after a job accident.

D. Measures are taken without work accidents.

847. Which organization carries out market surveillance and surveillance of personal protective equipment?

a. MLSS

b. Chambers of Commerce

c. TOBB

D. Industry Chambers

848. What is the concept of altitude defined by the following sentences?

a. Locations that are held by hand and are dangerous to fall are called high.

b. It is called high 5 meters high which can be reached with the help of others.

c. It is at least 7 meters high, which can be left without driving.

D. Places that we can not easily get off in one step can be defined as high.

849. Among the information and instructions to be given to workers who work with cancer and mutagenic substances, which of the following is not the first of them?

a. Use of protective materials and clothing

b. First aid basic principles

c. Risks that may affect health and additional risks to smoking

D. Precautions to be taken to avoid exposure

850. How many years does the employer have to keep the workers' personal health files from the date of departure?

Choose an answer.

a. 5

b. 40

c. 20

D. 10

851.Which of the following institutions does the market surveillance and supervision of the machinery covered by the Safety Directive of the machinery be carried out by the following institutions?

a. TOBB

b. MMO

c. The Ministry of Labor and Social Security

D. industry and Trade Ministry

852. Which of the following is not one of the arrivals of business accidents on construction work?

a. Material drop

b. Machinery and equipment accidents

c. Excavation ditches

D. Personal protective equipment

853. Which of the following is not one of the safety conditions of working at high altitude?

a. Wearing steel toe shoes

b. Seat belt use

c. Scarecrow

D. Establishing a safety net

854. Which of the following is the definition by the World Health Organization that "not only the known and accepted occupational diseases, but the conditions of their occurrence and development, and their way of working are important factors among other causes"

a. Work related accidents

b. Occupational diseases

c. Work related diseases

D. Occupational accidents

855. According to the health check-ups and the risk assessment, the updated list of employees in occupational health and safety risky jobs and their records of exposure status will be kept for at least several years after the expiration of the exposure?

a. 20

b. 40

c. 50

D. 10

856. Which of the following is true for the health reports to be issued for workers who will work in poultry and hazardous work?

a. It should be taken from the workplace physician.

b. It must be taken from the health board.

c. It should be taken from family physician.

D. It should be taken from the internal medicine physician.

857. What is the minimum oxygen concentration allowed in a coal mine?

a. 19

b. 21

c. 18

D. 16

858. Which of the following describes the warning sign?

a. A sign that indicates a compulsory behavior

b. A sign that prohibits any conduct that would cause or threaten to cause danger

c. A warning sign about the situation that could cause or harm a hazard.

D. Emergency exit routes, signs giving information about first aid or rescue

859. Which of the following is not included in the definition of hazardous chemicals?

a. Allergic

b. oxidising

c. Toxic

D. Flammable

860. Which one of the following health and safety departments do not have to be located?

a. Inspection, first aid and emergency room

b. Room of business physician

c. Standby

D. Occupational safety specialist room

861. What is the highest limit value for the small voltage below?

a. Voltages with a maximum voltage of 50 volts are called small voltages.

b. Voltages with a maximum voltage of 100 volts are called small voltages.

c. Voltages of up to 75 volts are called small voltages.

D. Voltages with a maximum voltage of 25 volts are called small voltages.

862. How many occupational diseases are evaluated according to the International Labor Organization (ILO) list?

a. 10

b. 6

c. 4

D. 2nd

863. Which of the following is not one of the employer's obligations?

a. To educate workers about occupational risks

b. audit

c. Participate in occupational health and safety training

D. To inform workers about legal rights and responsibilities

864. Should the work-related accidents be reported to SSI Regional Directorate within the next working day after the accident?

a. 2nd

b. 3

c. one

D. 5

865.Every step on the escape ladder height .......... cm.

width can not be less than .......... cm? den. What is the value to be written in the dotted area?

a. 12-14 cm

b. 18-20 cm

c. 14-16 cm

D. 10-12 cm

866. Which of the following classes does carbon dioxide gas enter according to its effect?

a. Simple mufflers

b. Systemic poisons

c. Chemical mufflers

D. Irritants

867. Which institution is more concerned with occupational health and safety as duty and responsibility?

a. SS institution

b. Ministry of Health

c. The Ministry of Labor and Social Security

D. Ministry of Industry

868. Which of the following jobs fall under the labor law?

a. Establishments or businesses where agriculture and forestry work of 50 or more workers are employed

b. Work places where 3 persons work in accordance with the description of Article 2 of Law No. 507 on Tradesmen and Craftsmen

c. Home services

D. Sea and air transport work

869. Which of the following jobs would work for only 7.5 hours per day?

a. Oxygen and electrical source workers

b. insecticides

c. Carbon sulphide works

D. Works also seen in mercury smelting furnaces

870.Which of the following applies to health and safety regulations when working with European vehicles?

a. On screen typewriters

b. In computer systems used in work centers

c. Computer systems open to the use of society

D. On computer systems in transport vehicles

871. Which materials are prohibited to import, export, manufacture, hold, use and store materials containing radioactive material which are not allowed in the limits of the scope?

a. Cosmetics

b. All

c. garments

D. stationery

872. When working with European instruments, it is imperative that the working centers that are active on the date of publication of the regulation on health and safety precautions have the provisions of the regulation reached in the maximum number of years?

a. 2nd

b. one

c. 4

D. 3

873.What personal protective equipment is used against the following hazards? Choose an answer.

a. All

b. Throwing parts

c. Chemical exposure

D. Harmful dusts

874. Which of the following measures, which will generally be followed when working with pawns, is wrong?

a. Protective goggles should be used in case of small parts splashing

b. Care should be taken that the chisel head and hammer face are not greasy or slippery

c. Chisel and hammer must be kept loose

D. The burrs per chisel must be cleaned with sandpaper or file

875. Graduates of the universities in the fields of chemistry, machinery, mining, geology, metallurgy, industry, electricity, electronics, construction, physics, geophysics, computer, textile, oil, aircraft, shipbuilding, environment, food engineering and architecture and agricultural faculties of agricultural faculties , which includes the following definitions?

a. Technical personnel

b. Engineer

c. Ministry

D. Job security specialist

As defined in the standard 876.1990, for the eight-hour work day, is the time-weighted average of all noise exposure levels, including instantaneous pulsing noise?

a. Daily noise exposure level (LEX, 8 hours) (dB (A) re.20 μPa)

b. Daily noise exposure level (LEX, 8 hours) (dB (A) re.10 μPa)

c. Daily noise exposure level (LEX, 8 hours) (dB (A) re.25 μPa)

D. Daily noise exposure level (LEX, 8 hours) (dB (A) re.15 μPa)

878.What personal protective equipment do we need to use in noisy environments?

a. Work glasses

b. Dust mask

c. Headphone

D. Baret

879. Personal Protective Equipment required for the protection of legs and legs is not:

a. Baret

b. Wooden footwear

c. Protective boots and drawers for chain saw operators

D. Attachable top protectors

880. The duration of the training can not be less than theoretical and practical time so that engineers or technical staff can be appointed as work safety experts at workplaces. What is the value to be written in the dotted area?

a. 60

b. one hundred

c. 80

D. 220

881. Which of the following types of gloves is not oil resistant?

a. PVC

b. Skin

c. Nitrile

D. neoprene

882. How many months should be checked by an authorized technician for the lifting machines and tools?

a. 12 months

b. 2 months

c. 3 months

D. 6 months

883. What are the stationary energies that cause the electrons or protons to flow out of their bodies as a result of their friction or contact with each other?

a. Direct current

b. Alternative current

c. Condenser

D. Static

884. How many corridor widths can not be less?

a. 4 m

b. 1 m

c. 2 m

D. 3 m

885. How many young people can not be employed in raid application?

a. 12

b. 16

c. 18

D. 10

886.What personal protective equipment do we need to use in a dirty environment?

a. Dust mask

b. Headphone

c. Dust mask

D. Work glasses

887. According to the health and safety regulations when working with European vehicles, in which of the following situations is it not appropriate for workers to make eye examinations?

a. If there is a malfunction in the systems of the screened vehicles

b. When there is a visual difficulty to be caused by working with a screened vehicle

c. At regular intervals

D. Before you start working with screened tools

888.What personal protective equipment does the following must be used in jobs where grains or rolled materials are lifted and transported?

a. Baret

b. work glasses

c. Work shoes

D. Headphone

889.Plasma, laser cutting etc. Which Personal Protective Equipment Should Be Used When Working On Machines?

a. Work glasses

b. X-ray glasses, laser beam glasses, ultra-violet, infrared, visible radiation goggles

c. Dust mask

D. Headphone

890. In an environment with microorganisms that pose a danger to human health, which of the following respiratory filters protects against effective protection?

a. P2

b. P3

c. FFP1

D. FFP2

891. Which of the trainings given to employees by means of the euros do not fall within the scope of occupational health and safety?

a. Correct seating styles

b. Eye protection

c. Fast typing techniques

D. Compulsive trauma and protection measures

892. In personal protective workshops, which Personal Protective Equipment must be used by workers who are exposed to heat, cutting, hand and arms?

a. work glasses

b. Headphone

c. Work gloves

D. Baret

893. How many winding ropes should be left on the rope drums of the lifting vehicles?

a. 4

b. 3

c. one

D. 2nd

894. What is the property of NRR and SNR on the packages of the ear protectors that is specified below?

a. Safety Factor

b. Ear Protector Standard

c. Ear Protector Type

D. Noise Reduction Ratio

895. How many young people can not be employed in raid application?

a. 16

b. 10

c. 12

D. 18

896. During the electric shock, the heart continues to work, but the shots are irregular. What is the name of the heartless shed under these conditions?

a. Atrial fibrillation

b. Excitement

c. Palpitation

D. Trauma

897. Which of the following measures is generally wrong with hand tools?

a. Must be kept in a suitable location

b. Must be well-maintained

c. Must be suitable for the job to be done

D. The diameters must be appropriate, easily out, cracked

898. Young workers and children who have not completed their ages and can not be employed in heavy and dangerous work without the professional training of the work they are working with?

a. 16

b. 14

c. 15

D. 12

899. What is the maximum stacking height of materials at work?

a. 5 m

b. 3 m

c. 10 m

D. 8 m

900. Which of the following would be wrong in choosing shoes?

a. Both shoes must be worn and selected by foot

b. The shoe will sit fully on the foot but should not be squeezed

c. Select the front of the shoe so that the fingers can move freely

D. Select by trying in the morning

TEST 10

901. Load with load 25-30 cm. Which one of the following will be checked after uninstalling?

a. ropes

b. Straps

c. hooks

D. Brakes

902. Which of the following measures, which will generally be followed when maneuvering and dismantling tools are used, is wrong?

a. When these tools are used, they should not be damaged by sliding other people

b. Not made of hard steel

c. Selectable for work

D. Wood, plastic or soft material should not be used as support

903. Which of the following can be used as personal protective equipment in the context of Occupational Health and Safety?

a. Stretcher

b. Gas measuring device

c. Boot

D. Gun

904.The lower limit of the dangerous voltage is the value of the voltage below?

a. 50 volts

b. 42 volts

c. 110 volts

D. 65 volts

905. How many years do hydrostatic tests of LPG tanks, spring safety valves?

a. 15

b. 10

c. 20

D. 5

906. Is the Personal Protective Equipment, which protects from construction work, especially works from scaffolds and overhead work platforms, under or near high work platforms, mold making and dismantling, assembly and installation work,

a. Work glasses

b. Baret

c. Work gloves

D. Dust mask

907. Which of the following must be the last of the 5 precautions that must be taken in order to protect the lives of employees during maintenance and repair work to be carried out in high-current facilities?

a. Short-circuit and grounding

b. Cutting tension in every direction

c. Check that there is no tension in the work places

908. Which of the following is wrong with regard to the use of personal protective equipment in the workplace?

a. Subject to exceptional and special circumstances

b. Must be in accordance with ergonomic requirements

c. Must be fully compliant when making necessary adjustments

D. It should be appropriate to avoid the risk without creating risk

909.Which protective devices are used in the following situations?

a. If risks are avoided

b. If collective protection measures are taken

c. If the risks are reduced to a sufficient extent

D. If the technical measures are insufficient

910. Which engineer or technical staff, certified by the Ministry, responsible for occupational safety, includes the following definitions?

a. Ministry

b. Engineer

c. Technical personnel

D. Job security specialist

911.Doing and transporting means must be able to hold at least how many times the maximum load can be held?

a. 2nd

b. 2.5

c. 3

D. 1.5

912. Working condition with more than 6 mSv of active doses or eye lesions, 3/10 of the annual equivalent dose limit for skin, hands and feet,

a. Working Conditional C

b. Working Condition D

c. Working Condition A

D. Working Conditional B

913. What is the limit voltage value between low voltage and high voltage?

a. 1000 volts

b. 660 volts

c. 380 volts

D. 6000 volts

914.When the health and safety worker's representative is employed in the workplace with at least a few employees?

a. 50

b. Number does not matter

c. 30

D. 10

915.I The employer must subject workers to health surveillance in accordance with the health and safety risks to which they will be exposed in the workplace.

II. A health report is issued indicating that health conditions are appropriate for the workplace when workers enter the workplace.

III. According to the nature of work done, health checks of workers are carried out at regular intervals during the course of work.

IV. Workers should be informed about hazards and risks when new equipment is picked up.

a. I, II

b. All

c. II, III, IV

D. I, II, III

916.Türki de enacted in 1936 and located in occupational health and safety as well as the detailed arrangements regarding the Labor Law is which of the following?

a. Law No. 4857

b. Law No. 3008

c. Law No. 931

D. Law No. 1475

917. In the first year of the Republic, which law is included in which the employer's work accident and occupational diseases cause legal liability?

a. Public Hygiene Law

b. Law of Obligations

c. Number 931

D. Municipal Law

I) Appropriate employment placement

II) Control of workplace risks

III) Intermittent control examination

IV) Health care at work

918. Which of the above items is a contemporary occupational health and safety policy?

a. I, IV

b. I, II, III

c. Alone III

D. all

919.M.S. Which person has lived between 23 and 79 years and who recommends to put a bag around his head to replace the mask in order to protect workers against dangerous dust in the working environment?

a. Owen

b. Berdardino Ramazzini

c. Plini

D. Juvenal

920. Which of the following is not covered by job security?

a. Employer's rights

b. Workplace safety

c. Safety of employees

D. Safety of machinery and vehicles

921.17. Which of the following is the person who puts forward the idea that the worker's working style and work-worker harmony in the workplaces where we can say as the principles of ergonomy in the century are influential on health and work efficiency?

a. Berdardino Ramazzini

b. Plini

c. Juvenal

D. Owen

922Which of the following are / are the contemporary OSH practice principles?

I. Proper employment placement

II. Evaluation of workplace environment factors

III. Provision of OHS service in the workplace

IV. Assessment and control of OHS risks

a. All

b. II, IV

c. II, III, IV

D. I, III, IV

923. Which of the following is not the basic concepts of occupational health?

a. Duration of obligation

b. Injury

c. occupational disease

d. Occupational physician

924. Findings such as weakness, constipation, paralysis and visual disturbances were the first to relate to lead

Which of the following is the person who has put out in the ages?

a. Bernardino Ramazzini

b. Heredot

c. Owen

d. Hippocrates

925. Which one of the following is wrong?

a. OHS is an expert of business security secretary at the board.

b. Occupational health and safety (OHS) boards consist of at least 50 employees and six months

Establish at workplaces where permanent jobs are made.

c. It convenes with the majority of the members of the Board.

d. OHS committees are in the nature of advisory bodies.

926. Health and safety workers' representative is at the minimum number of employees?

a. 10                                                 b. 30

c. 50                                                 d. The number is not important.

927. Which of the following jobs can only work for 7.5 hours per day?

a. Carbon sulphide works

b. Insecticides

c. Oxygen and electric welding workers

d. Works in mercury smelting furnaces

928. what does the board secretary of Occupational health and safety do?

a. Occupational physician

b. Job security specialist

c. Worker representative

d. Employer

929. In the workplace, during the work time, health

What is the systematic work done with the aim of protecting from harmful situations?

a. Safety culture

b. Security precautions

c. Occupational health and Safety

d. Risk assessment

930. Which of the following is not one of the main objectives of occupational health and safety?

Choose an answer.

a. Protection rights.

b. Protection of employees.

c. Protection of production.

d. Protection of the operator.

931. What does it mean to be caused by man's own faults and causing the accident?

a. Carelessness

b. Dangerous action

c. Dangerous situation

d. Thoughtfulness

932. What percentage of job accidents are hazardous movements?

a. % 44

b. % 88

c. % 50

d. % 10

933. Which of the following is not one of the dangerous movements?

a. Take an insecure status

b. Maintenance-free machine

c. Dangerous use of tools and machines

d. Unsafe loading, handling and stacking

934. In a facility's premises, work methods and forms, machinery and equipment

If a hazard is found in the devices of the workers' life, this danger According to the nature of workplaces which are dangerous in terms of occupational health and safety until they are eliminated, whether to stop the business in whole or in part or to make a decision to close the business,

Which of the following officers will not enter the commission?

a. Authorized Auditor

b. Employer Representative

c. Workers' Representative

d. Industry sector Director

935. Which of the following situations does the heavy and dangerous works report require?

I- First job entry

II- Occupational occupational disease

III- At least once a year for the duration of the work

a. I-II

b. I-III

c. I-II-III

d. II-III

936. Employers should be aware of the occupational accident that occurred in the workplace and the occupational disease

Must notify the relevant Regional Director in writing within working day?

a. 4                               b. 3

c. 1                               d. 2

937. In case of work accidents, occupational diseases, financial responsibility

Defined?  (C)

a. commercial law

b. Labor Law No. 4857

c. Maritime Labor Law

d. law of Obligations

938. Which of the following labor legislation does not include technical measures?

a. Noise Regulation

b. Hand Carrying Works Regulation

c. Regulation on signs of safety and health

d. Regulation on Severe and Dangerous Works

940. Which of the following is private law?

a. Debt law                        b. Administrative law

c. Business law                  d. Financial law

941. If an accident has occurred due to inevitability, any person involved in the accident

Or Establishment is found to be irregular, the loss of the worker and the employer at certain rates

Needs to be met. What is your employer's obligation?

a. Responsibility for inevitability

b. Compensation

c. Exceptional responsibility

d. Support indemnity

942. The act of knowingly and willingly acting in contradiction to the law,

What does it mean in your law?

a. Intent                                         b. Neglect or careless

c. Negligence or omission             d. Defect

943. In terms of occupational health and safety in the workplace, the health of the worker or the body

The worker who is faced with an imminent, immediate and vital danger that will put his integrity in jeopardy

Can not do the following due to legal rights?

a. Applying to employer or employer's agent

b. Terminate the service immediately

c. Applying to the Occupational Health and Safety Board or committee

d. Applying to the Ministry of Health and Social Assistance

944. From the point of view of criminal law, while one person does not want the outcome contrary to the law, what to do about the offending behavior if it does not show the attention and care that the situation requires to prevent it?  (A)

a. inadvertence                              b. Defect

c. Neglect or careless                    d. Intent

945. Which one of the following is wrong?

a. Acting on behalf of the employer and taking part in the management of the work, the workplace and the employer, the person is called the employer and the deputy (assistant)

b. The principal employer shall be liable to the employer of the sub-employer for the workplace, Of the obligations arising from the collective labor agreement in which the sub-employer is party to the sub-employer Are responsible for

c. The employer is a special status based on special obligations

d. The employer is directly responsible for the actions and obligations of the employer's agent against the workers

946. What legislation does it take to close the workplace?

a. Legal                                        b. Financial

c. Criminal                                  d. Administrative

947. In workplaces where it is not convenient for workers to work in terms of work health and safety,

How to sanction sanctions?

a. Legal                                                  b. Financial

c. Criminal                                            d. Administrative

948. Which blood is defective?

a. SS law (Social Security Law)

b. Law of Obligations

c. Commercial law

d. Business rules or laws

950. If the worker threatens his / her safety due to his / her request or because of the loss of the worker's property or damage to the machines, fixtures or other goods and materials that are not the property of the worker and can not pay for the amount of the daily wage, can the employer immediately fire or dismissal the worker?

a. 30                                   b. 90

c. 10                                   d. 15

951. Yearly worker of 18 years or younger and 50 years of age or older

What is the minimum duration of paid leave?

a. 26 days                          b. 30 days

c. 15 days                          d. 20 days

952. What is the obligation of the employer to provide training on occupational health and safety?

a. Business institution law

b. SSK

c. Business rules

d. Vocational education law

953. What type of application is the business closure process?

a. Financial                                  b. Commercial

c. Administrative                        d. Criminal

954. The employer who occurred before coming to the business accident or occupational disease punishment penalty

Which legislation is included in the following sanctions?

a. Criminal law                               b. Criminal law

c. SS law                                          d. Business rules

955. Which of the following jobs fall under the labor law?

a. Workplaces or businesses where agriculture and forestry works are employed with more than 50 workers

b. Work places where 3 persons work in accordance with the description of Article 2 of Law No. 507 on Tradesmen and Artisans

c. Sea and air transport work

d. Home services

956. Which of the following is not included in the principles of the legislation issued pursuant to the Labor Law and Labor Law No. 4857?

a. Assessing risks that can not be avoided

b. Fighting in the source with risks,

c. Prioritizing personal protective measures,

d. Giving instructions according to workers

957. The service is a pact, with which a worker, a certain or an unspecified servant, and even a worker undertakes to pay him a fee. What is done between the employer and the employer where the judgment is placed? Service flow? What is the definition of what the law is?

a. None

b. Rules of Obligations

c. Public Hygiene Law

d. Social Insurance and Public Insurance Law

958. Labor Law No. 4857? Administrative fines shall be issued by the Regional Director of the Ministry of Labor and Social Security at that place, with the exception of administrative fines in Articles 101 and 106, 101 and 106. Whichever of the following authorities will be given administrative fines in the matter?

a. ÇASGEM - Çalışma ve Araştırma Sosyal Güvenlik Eğitim Merkezi

b. Chairman of Labor Inspection Board

c. SGK Sector Director

d. Sector Director of Turkish Employment Agency

959. A mine engineer who is permanently employed in mining operations can also work as a job security expert if he or she is in possession of a job security expertise. In the event that the number of workers exceeds the number, a mining engineer is appointed as a work safety specialist.

a. 50100                                     b. 500

c. 300                                         d. 100

960. What is the basis for the decision on whether a worker whose worker is injured will be liable to pay his / her own compensation for his / her family OR, if he / she loses his / her life, for his / her family?   (D)

a. Social Insurance and GSS Law

b. Business rules systems

c. Public Hygiene Law

d. Rules of Obligations

961. Which of the following is wrong with regard to workplace inspections of municipalities and other relevant authorities? (B)

a. If it is necessary to obtain an operating document in accordance with the relevant legislation, it shall not be permissible to open municipalities or other relevant authorities to the workplaces for which accreditation and the Ministry of Social Affairs have not issued operational documents.

b. The municipalities authorized to allow the establishment and opening of the workplace and other relevant authorities shall notify the regional directorate where the site is affiliated until the end of the following month, the name and address of the employer and the workplace.

c. Public institutions and organizations will conduct inspections on occupational health and safety

And the results of the inspections and the actions to be taken shall be made to the competent regional directorate

d. Before giving this permission, the municipalities authorized to allow the opening of the workplace and other relevant authorities have to investigate the existence of the necessary administrative disposal according to the labor legislation of the Ministry of Labor and Social Security.

962. Which of the following does not contain substance related to occupational health and safety?

a. Constitution

b. Public hygiene law

c. Law of Obligations

d. Business rules

963. The municipalities authorized to allow establishment and opening workplace and other relevant authorities are obliged to inform the relevant regional directorate of the information on the workplaces they permit.

a. Up to 10 next months?

b. Up to 5 next months?

c. Until the end of next month

d. Up to 15 next months?

964. Which of the following items are not included in Article 77 of the Labor Law?

a. A regulation on which jobs to be deemed to be Heavy and Dangerous will be issued.

b. Employers have to check whether the occupational health and safety precautions are being followed.

c. Employers are obliged to take all necessary precautions to ensure occupational health and safety at workplaces and to keep tools and equipments

D. workers are obliged to comply with any preventive measures taken regarding occupational health and safety.

965. Which of the following is wrong with the Code of Obligations regarding the damage caused by the employers' employees to others while doing the work of the employer?

a. No one is held accountable for the financial situation of the workers because the workers are harmed

b. The employer will not be liable if he proves that he has all the attention and discretion that should not be such a loss.

c. The employer is entitled to a remedy against the employee, the damage caused by the damage.

D. The person who employs others is responsible for the damage they do while they are performing their services.

966. How long should the periodical application of general cleaning be ensektisik and rodenticide with antiseptic solutions be carried out in the worker batches?

a. 1 per month

b. 3 months

c. 6 months

D. 12 months

967. How long do workers with pre-existing workers undergo periodic health examinations?

a. 12 months

b. 1 per month

c. 3 months

D. 6 months

968. Which of the following workplaces is best suited for workers with 2 compartments or 2 separate closets?

a. Flour Mill

b. Factory producing agrochemicals

c. Sugar '66

D. Wafer, biscuit manufacturing

969. Which of the following is true about the appointment of the workplace nurse or health officer?

a. It is imperative to have a workplace nurse in all establishments

b. It is obligatory to have a workplace nurse in 50 worker establishments

c. II. At least one workplace nurse working full-time in a 500-employee workplace must be present in the risk group

D. IV.Risk group should have at least 1 workplace nurse working full-time in a 300-employee workplace

970. Which one of the following is wrong?

a. the job security and safety secretary of the establishment makes the job security specialist

b. The health unit consists of at least one workplace physician and at least one workplace nurse or health officer

c. In the V.risk group, a full-time workplace nurse must work in a 350-employee workplace

D. Health and safety worker must be selected in places with 50 employees

How long should a Class A worker for a workplace in the 971.V. work group go to work at least once a month?

a. At least 3 business days per month

b. At least 2 business days per month

c. At least 1 business day per month

D. At least 5 business days per month

972. Which of the following is wrong with the employees of the workplace health unit?

a. The health unit consists of a workplace physician, workplace nurse and / or health officer and work safety specialist

b. Workplace nurse maintains health-related records

c. an example of the annual activity report of the workplace health unit is sent to the Ministry of Labor and Social Security

D. workplace nurse helps the physician in matters related to worker health

Which of the following is not the duty of the business security expert?

a. To make corrective action plans

b. To manage fire protection and fire fighting activities and ensure that relevant records are kept

c. Prepare emergency plans

D. To increase production

974. Which of the following can not be looked at while investigating the story of the pediatric illness?

a. Number of children

b. Where you sit

c. Class of military service

D. Previous jobs

975. Which of the following is not the purpose of occupational health services?

a. Making worker work

b. To protect the health hazards that may arise from the worker's work

c. To provide consulting services to workers and business

D. To contribute to the maintenance and maintenance of the well-being of the worker

976.Which of the workplaces specified below are required to establish occupational health safety board?

a. None

b. 51 employees who are counted as industrial and 4 months in operation a year

c. A workplace that is regarded as a trade and employs 100 workers,

D. A workplace that employs 51 workers who are counted as industrial and have been operating for 11 months a year

977. In order to prevent the health of the worker in terms of occupational health and safety at work,

the worker who is faced with an imminent, immediate and vital danger that will put his integrity at risk;

When the OHS Board wants to meet and discuss the meeting, how many days must the OHS Board meet?

a. There is no need to convene, it will be discussed at the first ordinary meeting

b. Must be collected in a week

c. None

D. It has to meet the same day,

978. Which of the following about the ISIG Board is intuitive?

a. Occupational physician

b. Foreman, Foreman

c. Employer representative

D. Job security specialist

979. Which of the following is not the duty of the OSIG Board?

a. To plan the necessary safety precautions in the maintenance and repair works to be carried out in the facilities

and to control the implementation of these measures

b. The employer is obliged to implement the decisions of the establishment.

c. 4857 counts urgently and in the case of the demands stated in Article 83 of the Labor Law

D. To monitor the adequacy of workplace fire, natural disasters, sabotage and similar measures and the work of the teams

980. Which of the following members of the Occupational Health Safety Board comes to elect this referral?

a. Workplace Physician

b. Health and Safety Worker Representative

c. None

D. Job security specialist

981. Which of the following is the priority of protection from the following hazards?

a. Priority to privacy protection applications

b. Destruction at the source of danger

c. Increasing the distance of the hazard area or closing it in the working environment

D. Reduction of dangerous source

982. Is not the '68th anniversary from the continuing protection practices?

a. Personal protective equipment use

b. Occupational health and safety trainings

c. Periodic health examination

D. Entry to health examination

983.TS OHSAS 18001? Which is the basic philosophy of?

a. To make a risk assessment

b. Increase productivity

c. Determine needs according to OSH legislation

D. Ensuring work safety and health of workers

984. Which of the purposes of ISO 18001 is absent?

a. Promotion of product and service quality

b. Continuous learning, innovation and learning

c. Protection of trainees and subcontractors

D. Risk assessment

985. Which of the following is the standard that guarantees ethical elements in the production of goods and services?

a. ISO 14001 Standard

b. SA 8000 Standard

c. OHSAS 18001 Standard

D. ISO 9001 Standard

986. Which of the following is beneficial for the employees of the risk assessment process?

a. Access to contacts

b. Selecting secure technology

c. Participation right

D. Predefined rules

987. Which of the following is not the new understanding introduced by the Law No. 4857 in terms of occupational health and safety?

a. Detection based reagent

b. Provision of expert contribution

c. Employee involvement Prevention understanding

988 Which one of the following should be taken into account when evaluating the risk associated with working with hazardous chemicals?

a. Material safety data sheet from manufacturer, importer or vendor

b. The amount of chemical substance, usage conditions and usage frequency,

c. All

D. The type, level and duration of the exposure,

989. Which of the following describes the risk precisely?

a. In the event of a particular danger, some or all of the person's personal assets

possibility of losing

b. Likelihood of losing

c. Damage by the possibility of an event coming from a hazard

lt; / RTI & gt;

D. The amount of possible loss

990. Which of the following is the precautionary action before the events occur?

a. Trial and error method

b. Reagent

c. Proactive activity

D. All

991. Is it not useful for an individual to perform risk assessment exercises based on the nature of the employer? Which of the following is the cause of the crime?

a. The self-confidence of the individual increases

b. One-dimensional

c. Quick result retrieval is provided

D. It prevents the individual from being directed by his colleagues

992. Which of the following is not a chemical hazard in the workplace?

a. Radiant heat

b. Gases (flammable, burning, suffocating, narcotic)

c. Liquids (acids, bases, solvents)

D. Solid powders (organic, inorganic)

993. Is it not useful to make risk assessment work as a team according to the nature of the business? Which of the following is the cause of the crime?

a. Gives seriousness to the participants and spirit of business association

b. All the necessary information is provided by the employees

c. Intra-team interaction can affect the end result.

D. Everyone can get satisfying results

994.Which workplace should the obligation to perform risk assessment be covered by the Labor Code?

a. Dangerous work places

b. All businesses

c. Great work places

D. Workplaces employing 50 or more workers

995. Which of the following is not one of the key steps in risk assessment?

a. Identification of hazards

b. Determination of measures

c. Training of employees

D. Assessing risks

996. Which of the following is not from the "Insecure Situations" seen in the workplace?

a. Electric machines without body grounding

b. Unopened gaps

c. Hazardous machine parts

D. Removing machine protectors

997. Which is not included in the race assessment?

a. Risk score

b. exposed people

c. workplace health premiums

D. Hazard definitions

998.What is the level of risk?

a. 40-50

b. 2-3

c. 60-100

D. 20-25

999.Risk Assessment must be renewed at the latest?

a. Every 2 years

b. Every year

c. A workplace that employs 51 workers who are counted as industrial and have been operating for 11 months a year

D. Every 10 years

1000. Regulations on health and safety measures to be taken in workplace premises and annexes shall be applied in which of the following?

a. In factories and warehouses

b. On the fishing boat

c. Temporary or Moving business areas

D. Mine, oil and gas extraction works