2

Random Access Protocols

The second broad class of multiple access protocols are random access protocols.

When a node experiences a collision, it doesn't necessarily retransmit the frame right away.

Instead it waits a random delay before retransmitting the frame.

Each node involved in a collision chooses independent random delays.

3

Slotted ALOHA

Nodes starts to transmit frames only at the beginnings of slots.

Each node detects collisions and independently decides when to retransmit.

Unlike channel partitioning, slotted ALOHA allows a node to transmit continuously when that node is the only active node.

The protocol is highly decentralized because each node detects collision and independently decide when to send a frame.

The operation of slotted ALOHA in each node:

The node has a fresh frame to send, it waits until the beginning of the next slot and transmits the entire frame in the slot.

The node retransmits its frame in each subsequent slot with probability p until the frame is transmitted.

4

Slotted ALOHA when there are multiple active nodes:

A slot in which exactly one node transmits is said to be a successful slot.

The efficiency of a slotted multiple access protocol is defined to be the long-run fraction of successful slots in the case when there are a large number of active nodes, each always having a large number of frames to send.

Note that if no form of access control were used, and each node were to immediately retransmit after each collision, the efficiency would be zero.

Slotted ALOHA clearly increases the efficiency.

We now proceed to outline the derivation of the maximum efficiency of slotted ALOHA.

5

Efficiency in this context means long-run fraction of successful slots (many nodes all with many frames to send).

Let suppose N nodes with many frames to send, each transmits in slot with probability p

Probability that given node has success in a slot is p(1-p)N-1 .

Probability that any node has a success is N*p(1-p)N-1.

For max efficiency we find p* that maximizes N*p(1-p)N-1.

For many nodes, if we calculate limit as N goes to infinity, it gives: 0.37.

Which means when a large number of nodes have many frames to transmit,  only 37 percent of the slots do useful work.

6

Pure Aloha

The slotted ALOHA protocol required that all nodes synchronize their transmissions to start at the beginning of a slot.

At any given time, the probability that a node is transmitting a frame is p.

If a transmitted frame experiences a collision with one or more other transmissions, the node will then immediately retransmit the frame with probability p.

Otherwise the node waits for a frame time, or waits for another frame time with probability 1 – p.

We'll make the same assumptions as in our slotted ALOHA analysis and take the frame transmission time to be the unit of time.

7

The probability that all other nodes do not begin a transmission in this interval is (1 – p)^(N-1).

Similarly, no other node can begin a transmission while node i is transmitting, as such a transmission would overlap with the latter part of node i’s transmission.

Maximum efficiency of the pure Aloha protocol is exactly half that of slotted Aloha.

8

Carrier Sense Multiple Access (CSMA)

In particular, a node neither pays attention to whether another node happens to be transmitting when it begins to transmit, nor stops transmitting if another node begins to interfere with its transmission.

Specifically, there are two important rules for CSMA:

• 1) Carrier sensing—node listens to the channel before transmitting. If a frame from another node is currently being transmitted into the channel,  waits until it detects no transmissions for a short amount of time and then begins transmission.

• 2) Collision detection—transmitting node listens to the channel while it is transmitting. If it detects that another node is transmitting  an interfering frame, it stops transmitting and waits a random amount of time before repeating the “sense-and-transmit-when-idle” cycle.

9

Figure shows a space-time diagram of four nodes attached to a linear broadcast bus.

At time t 0, node B senses the channel is idle, as no other nodes are currently transmitting. Node B thus begins transmitting, with its bits propagating in both directions along the broadcast medium.

A short time later, B's transmission begins to interfere with D's transmission at D.

From the Figure, it is evident that the end-to-end channel propagation delay of a broadcast channel play a crucial role in determining its performance.

The longer this propagation delay, the larger the chance that a carrier-sensing node is not yet able to sense a transmission that has already begun at another node in the network.

In Figure, nodes do not perform collision detection; both B and D continue to transmit their frames in their entirety even though a collision has occurred.

10

When a node performs collision detection, it ceases transmission as soon as it detects a collision.

This figure shows exactly the same as the last figure, except that the two nodes each abort their transmission a short time after detecting a collision.

Clearly, adding collision detection to a multiple access protocol will help protocol performance by not transmitting a useless, damaged frame in its entirety.

11

Ethernet CSMA/CD algorithm

1. NIC receives datagram from network layer, creates frame

2. If NIC senses channel idle, starts frame transmission. If NIC senses channel busy, waits until channel idle, then transmits.

3. If NIC transmits entire frame without detecting another transmission, NIC is done with the frame !

4. If NIC detects another transmission while transmitting,  aborts and sends jam signal

5. After aborting, NIC enters binary (exponential) backoff:

- After mth collision, NIC chooses K at random from {0 to 2m-1}. NIC waits K*512 bit times, returns to Step 2

12

Tprop is max the prop delay between 2 nodes in LAN

ttrans is the time to transmit max-size frame

As tprop goes to 0, efficiency goes to 1 and as ttrans goes to infinity efficiency also goes to 1.

As a result, carrier sense multiple acces with collision detection has better performance than ALOHA. Also it is simpler, cheaper and more decentralized!