Internet of Things: CoAP and CoCoA (Summary)

IoT:

IoT (Internet of Things) is the communication network between physical objects or connections to bigger system from physical objects. The main idea of IoT is increasing performance by gathering information from collective network for devices while devices are applying their default tasks. The term was invented by Kevin Ashton in 1999. IoT’s usage areas are: Wearables, healthcare, agriculture, traffic monitoring, smart home, smart city, energy, Industrial, transportation, retail. IoT devices are low capacity devices and they need a special network protocol. CoAP is a compatible internet protocol for requirements of IoT. For CoAP’s congestion control, CoCoA is developed.

CoAP:

CoAP (Constraint Application Protocol) is an application layer protocol which is specialized for constrained devices like IoT devices. It is developed by the Internet Engineering Task Force (IETF) and defined in RFC 7252. It works on UDP for limited bandwidth and limited capacity. CoAP’s interaction model is similar to HTTP’s client/server model. Every IoT nodes acts as both client and server roles with CoAP because CoAP is designed for M2M concepts. CoAP supports multicast. An IoT device sends a message, then it can receive responses from multiple sensors. HTTP doesn’t support this. HTTP needs synchronous communication. It cannot send the next query without receive a response. In CoAP, sending multiple request without receive a response is possible. HTTP uses TCP and that’s because it has more cost and it is more complex and slower than CoAP which uses UDP. CoAP’s coast is less and it’s simpler and faster than HTTP.

CoCoA:

CoCoA (CoAP Simple Congestion Control/Advanced) is a congestion control mechanism that is standardized by the IETF CoRE working group. CoAP needs a control mechanism for congestion. It is called Simple Congestion Control/Advanced or CoCoA. It calculates RTO (Retransmission timeout) and is based on RTT estimations with or without loss.