# What is Spanning Tree Protocol ?

Spanning Tree Protocol is a set of rules designed for ports to communicate with each other or broadcast messages without forming loops. It builds a loop free logical topology for ethernet networks

# What is the goal of STP ?

The goal of STP is to provide path redundancy while preventing the undesired loops

# How does STP work ?

STP first decides a root depending on the set of rules. Every switch wants to become root. Each switch list themselves in the BPDU ( Bridge Protocol Data Units ) as root. Then the switches in a LAN exchange “Hello” and send BPDU to each other. The BPDU contains the route cost, the route and the local BID. They then compare and select the root bridge with the lowest value. There are redundant tie breakers to make sure there are no more than 1 root is selected in the whole system. After selecting root, they select root ports and designated ports and the most important thing that prevents loops at the end which is blocking all non-designated ports.

# What are the Alternatives to STP ?

The alternatives to STP are Shortest Path Bridging (SPB) and Transparent Interconnect of Lots of Links (TRILL). Multiple Spanning Tree Protocol (MSTP) Rapid Spanning Tree Protocol (RSTP) and PortFast with BPDU guard.