# ÇUKUROVA UNIVERSITY

# ENGINEERING FACULTY

DEPARTMENT OF COMPUTER ENGINEERING

Internet Communication

Voice Over LTE

Assistant Professor Fatih Abut

RÜVEYDA KILIÇ

2017556036

Berk Eroğlu

2018556027

CAHİT BARKIN ÖZER

2018556059

6.12.2020

ADANA

Voice Over LTE

Abstract                                                                                                   3

1. Introduction                                                                                         3

a. Explanation of the VoLTE                                                                  3

b. History of VoLTE                                                                               5

2. Analysis                                                                                               6

a. Advantages of VoLTE                                                                          6

b. Disadvantages of VoLTE                                                                     7

3.  Comparison with VoIP                                                                      8

a. Similarities between VoLTE and VoIP                                                 8

b. Differences between VoLTE and VoIP                                                8

Summary and Conclusion                                                                     9

References                                                                                             10

- Introduction

Abstract

We are going to start with what is VoLTE and what is its past. We are going to get a basic understanding of how it operates and what its sub-functions are. Later, we will analyze VoLTE’s advantages and disadvantages. We are trying to examine what kind of benefits this system provides. At the end, we will compare VoLTE with VoIP and conclude this document.

- Explanation of the VoLTE

Long-term evolution (VoLTE) is an internet-based communication data transfer technology for mobile phones, internet of things and wearables. VoLTE is based on IP Multimedia Subsystem(IMS) network which is specified by GSMA on pdr.922. This method results in a voice service being provided as traffic flows inside the LTE data carrier, without dependency) the circuit-switched voice network to be in the call route. VoLTE has a speech and network range up to three times that of older 3G UMTS and up to six times that of 2G GSM. VoLTE packet headers are smaller than VoIP / LTE headers so they use less bandwidth. VoLTE calls are usually cost at similar rate as different calls.[1]

To communicate over VoLTE, the device and the telephone provider must be within the network boundaries of the region. VoLTE is a technique based on IMS (information management system). This methodology allows the system to be compatible with the suite of other LTE applications. A reduced version has been defined so that IMS can be applied properly to operators. This also simplifies interconnectivity focusing on the elements required for VoLTE and decreases the prices for network providers beacuse it was a big problem that hindered the implementation of IMS. [2]

Figure1- Reduced IMS Network for VoLTE [2]

As can be seen in Figure 1, there are many entities inside the reduced IMS network used by VoLTE:

IP-CAN IP( Connectivity Access Network): IPCAN consists of EUTRAN (air interface of 4G technology) and MME (multicast mobility anchor). [2]

P-CSCF (Proxy Call State Control Function):  The P-CSCF is a network porxy user therefore, all incoming and outgoing SIP signaling to the user is performed with P-CSCF. [2]

I-CSCF (Interrogating Call State Control Function: If the initiator does not know which S-CSCF to receive the appeal, the I-CSCF forwards the original SIP request to the S-CSCF. [2]

S-CSCF (Serving Call State Control Function: S-CSCF performs a series of actions inside the entire system and includes a number of interfaces that allow it to communicate with other subsystems. [2]

AS (Application Server): AS is an application server that manages the voice as an application. [2]

HSS (Home Subscriber Server): Hss is the primary IMS subscriber database. It enables HSS subscribers to access their status by presenting their information to other devices in the IMS network.[2]

IMS VoLTE calls are processed by the users S-CSCF on the home network. The S-CSCF relation is through the P-CSCF. Depending on the network in operation and the general location within the network, the P-CSCF can differ, and the identification of the P-CSCF is a crucial factor in allowing voice call capabilities.[2]

Another requirement for Volte networks is to provide a way to revert back to circuit-switched old networks while providing only one radio transmission on the handset to increase battery life, so a system called SRVCC (Single Radio Voice Call Continuity) is required.[2]

- History of volte

In 2009, 12 major telecommunication companies partnered and suggested a solution for the VoLTE network. The solution was IMS based and  project was called One Voice. One Voice was the idea of using existing standards to produce a generic solution with the VoIP network. GSMA (Global System for Mobile Communication) started to work on the idea to make the solution official for general use in February 2010. When GSMA's VoLTE solution was released it was funded by 40 organizations worldwide. [3]

In 2012, worlds first commercial VoLTE service and worlds first VoLTE phone LG Connect 4G launched by MetroPCS in Texas USA.[3]

- Analysis

### Advantages of VoLTE

Delivers an unusually clear and high definition voice:
The major benefit of VoLTE is that the efficiency of the call is better than 3G or 2G when much more data can be transmitted over 4G than 2G or 3G. The compression method used in general calls uses an 8 kbps codec, but with VoLTE the codec can use 13kpbs codec. The frequency range also increased from 300-3400 Hz range to 50-7000 Hz range. This means higher quality of a call than a regular mobile call. The calls are delivered over the LTE networks so the latency (Delay) is much lower when compared to current networks.[6]

Faster connections:
VoLTE provides connection twice time quicker than the previous methods. Some  frequencies of 4G networks have much wider reach than 2G or 3G networks so it provides better indoor coverage. [6]

Provides a more efficient use of your network sources:
It would enable a simpler network that is more cost-effective to maintain by allowing service providers to eventually handle only one network instead of providing voice services through an older 2G, 3G voice network.[6]

Rich Conversations: Besides calling, operators can offer a range of services known as RCS (Rich Communication Services). These services include features  such as video calls, data transfer and simultaneous translation during the call.[4]

b. Disadvantages of VoLTE:

We emphasize that VoLTE is a new technology and as such it still needs to be improved for quality service. So far, we can list the disadvantages as follows.

Limited to certain devices:
As we just mentioned, only 4G terminals will support VoLTE. For example, Xiaomi Mi4 named cellphone does not support 4G VoLTE option in India.[5]

Disconnection of the call:
This is a new technology and 4G network uses antennas different from other technologies hence VoLTE still cannot guarantee a call without disconnections.[4]

Without communication between operators:
In principle it is only possible to call via VoLTE between the customers of the same operator, for example calls between one operator and another cannot be made via VoLTE.[4]

Price change:
Now that we are used to the flat rate for calls, we change technology. VoLTE calls use the data network. That is why the expense of VoLTE calls is reflected in our bill.[4]

Battery life:
There is no general opinion although some research has show that the VoLTE feature reduces the battery life of the device by half. One research says that VoLTE uses continuous stream of small packets with low data rate, so it maximizes the battery life.[5]

3. Comparison with VoIP

What is VoIP (Voice Over IP)?

Voice over Internet Protocol (VoIP) is a technology used to transmit communication data over Internet Protocol (IP) networks. [7]

- Similarities Between VoLTE and VoIP

- They both are using Internet to transmit voice data. [1][7]

- They both are using “Packet Switching” technology means that they are transferring your voice data as binary 1’s and 0’s, rather than analogue method. [8]

- They both are using same technology, only difference about technology is VoLTE transfers voice data to clients over 4G cell towers. [1][7]

- Differences Between VoLTE and VoIP:

VoIP is an older technology than VoLTE. But that does not mean VoLTE is better than VoIP. [9]

VoLTE uses 4G technology. VoIP uses 2G, 3G, Ethernet and Wi-Fi. [10]

VoLTE provides more scalability: VoIP is supported by the Web and by VoIP phones, but VoLTE is supported by all modern smartphones and devices. [10]

VoLTE is more costly. VoIP can use old copper cables known as ADSL to transmit voice data, but VoLTE needs 4G supported cell towers. That comes with costs for service provider and client.

VoIP may provide less latency. Usually, VoIP transmit voice data over network     cables, but VoLTE always must transmit voice data from cell towers. 4G is a wireless radio technology [11] and radio waves can be affected by distance, weather, other radio waves and buildings [12]. Those issues may increase latency of VoLTE, but VoIP usually uses cables that cannot affect by weather and radio waves. Also, industrial data cables use shielding against external factors.

Summary

VoLTE is a communication technology for cell phones which works on the 4G. Almost all phones support this technology as 2020. To use VoLTE, the user requires only two things; VoLTE supported smartphones and network. VoLTE has much lower latency(delay) when compared to current mobile networks. VoLTE has much clearer call quality than a traditional mobile call and can connect calls up to twice as quick as the previous methods. Using VoLTE will create a simpler network. On the other hand, Network also does not have enough extension to ensure a call without disconnections since it uses antennas other than previous technologies. It is only possible to call the same operator's customers through VoLTE. If we consider the previous technologies, the price of the service may be higher. The impact of VoLTE to battery life is still uncertain. VoIP is a technology for transmitting communication data over IP networks such as internet. Both VoIP and VoLTE use internet to transmit data. Both VoIP and VoLTE use similar technology except VoIP based on 2G, 3G, Ethernet or Wifi while VoLTE is based on 4G.

Conclusion

Battery life at VoLTE research is not yet concluded, so there is a need for further research about this subject. If we consider the improvements on batteries it looks like it will be a less problem over time. VoIP and VoLTE are similar technologies that give certain advantages and disadvantages to each other, but they cannot overpower one another. As technology has progressed over time, the data we use is  expanding; our need for better technology on communication is not over and it does not appear to be over yet.

REFERNCES

1.Introduction

[1] Wikipedia contributors.(2020,November,25).Voice over LTE.

[https://en.wikipedia.org/wiki/Voice_over_LTE]

# [2] No name.(no date) What is Voice over LTE, VoLTE.

[https://www.electronics-notes.com/articles/connectivity/4g-lte-long-term-evolution/volte-voice-over-lte.php]

[3] Prashant Panigrahi. (?2016).VoLTE History and Timeline

[http://www.3glteinfo.com/volte-history-timeline/]

2. Analysis

[4] Ankit Bhardwaj(2018, July,1) What is VoLTE technology Advantages and Disadvantages:

[https://teknologya.com/what-is-volte-technology-advantages-and-disadvantages/]

[5] Gizbot Bureu(2017, March,23) VoLTE benefits and disadvantages:

[https://www.gizbot.com/mobile/features/volte-benefits-and disadvantages/articlecontent-pf71893-039343.html]

[6] No name (2019, April,11) What is VoLTE:

[https://www.summanetworks.com/blog/what-is-volte]

3.  Comparison with VoIP

[7] Wikipedia contributors. (2020,December,1). Voice over IP:

[https://en.wikipedia.org/wiki/Voice_over_IP]

[8] Wikipedia contributors. (2020, November,29). Packet switching:

[https://en.wikipedia.org/wiki/Packet_switching]

[9] Katherine Stone. (2020, August,29). The history of VoIP and internet telephony:

[https://getvoip.com/blog/2014/01/27/history-of-voip-and-internet-telephones/]

[10] Pulsar360. (2018. July, 10). What is the difference between VoIP and VoLTE:

[https://www.pulsar360.com/blog.aspx?p=what-is-the-difference-between-voip-and-volte-180710]

[11] Wikipedia contributors. (2020, November , 20). 4G:

[https://en.wikipedia.org/wiki/4G]

[12] No name. (No date). Causes of Wifi interference:

[https://activereach.net/support/knowledge-base/connectivity-networking/wifi-interference/]