What is BOOTP (Bootstrap protocol)?

The Bootstrap Protocol (BOOTP) is a computer networking protocol used in Internet Protocol networks to automatically assign an IP address to network devices from a configuration server. The BOOTP was originally defined in RFC 951.  When a computer that is connected to a network is powered up and boots its operating system, the system software broadcasts BOOTP messages onto the network to request an IP address assignment. A BOOTP configuration server assigns an IP address based on the request from a pool of addresses configured by an administrator.  BOOTP is implemented using the User Datagram Protocol (UDP) as transport protocol, port number 67 is used by the (DHCP) server to receive client requests and port number 68 is used by the client to receive (DHCP) server responses. BOOTP operates only on IPv4 networks.

Advantages and disadvantages of the BOOTP

BOOTP is a protocol which enables static configuration. The reason behind the BOOTP static nature is that the need of dynamically discover routers or change router is eliminated when only one router is connected to the rest of the internet. However, if there are multiple routers are connected to the internet. A host attempts to obtain the default route at the startup can result in connection loss if a single router crashes. And also the crash can be not detected. BOOTP server uses a table that has a mapping of the physical address to the IP address when a client enquires its IP address. BOOTP does not support mobile machines; it works well only when the binding between Physical and IP addresses is static and fixed in the table. It uses limited broadcast address.

Advatages of diskless workstations

Distinction between diskless nodes and centralized computing Diskless nodes process data, thus using their own CPU and RAM to run software, but do not store data persistently—that task is handed off to a server. This is distinct from thin clients, in which all significant processing happens remotely, on the server—the only software that runs on a thin client is the "thin" (i.e. relatively small and simple) client software, which handles simple input/output tasks to communicate with the user, such as drawing a dialog box on the display or waiting for user input.

For network auto-booting, the Preboot Execution Environment (PXE) or Bootstrap Protocol (BOOTP) network protocols are commonly used to find a server with files for booting the device.

What is Trivial FTP (TFTP)?

Trivial File Transfer Protocol (TFTP) is a simple lockstep File Transfer Protocol which allows a client to get a file from or put a file onto a remote host. One of its primary uses is in the early stages of nodes booting from a local area network. TFTP has been used for this application because it is very simple to implement.  TFTP was first standardized in 1981[1] and the current specification for the protocol can be found in RFC 1350

Advantages and Disadvantages of  The TFTP

Trivial File Transfer Protocol (TFTP) is a network protocol used to transfer files between hosts in a TCP/IP network. It is a simpler version of FTP and it doesn’t have all of its functions; for example, you can not list, delete, or rename files or directories on a remote server. In fact, TFTP can only be used to send and receive files between the two computers. TFTP doesn’t support user authentication and all data is sent in clear text.

The only real advantage that TFTP has over FTP is that it uses less resources. It is not widely used today, but Cisco does still use it on its devices, for example to backup a router’s IOS image.