ÇUKUROVA UNIVERSITY

FACULTY OF ENGINEERING

DEPARTMENT OF COMPUTER ENGINEERING

CEN344 Computer Networks

Final Homework

05.06.2021

Cahit Barkın Özer

2018556059

İ.Ö.

Random Access Protocols

What are pure ALOHA and slotted ALOHA protocols? What are the differences between them?

Aloha Protocols are Random Access Protocols. There are two types of Aloha Protocols, pure Aloha, and Slotted Aloha. In pure Aloha, Stations transmit when data is accessible at any time, and colliding frames are deleted. In slotted Aloha, Station A must wait until the next slot begins before transmitting while in scheduled status. In comparison to pure Aloha, the vulnerable period is decreased. Any station can send data at any time in Pure Aloha, but in Slotted Aloha, any station can send data only at the start of any time slot. Time is continuous in Pure Aloha and is not globally synced, however time is discrete in Slotted Aloha and is globally synced. In pure aloha, vulnerable time is 2 x Tt, whereas in slotted aloha vulnerable time is 2 x Tt. It is vulnerability time. In pure aloha, the chance of successful data packet transmission is G x e-2G, however in slotted aloha, the chance of successful data packet transmission is G x e-G. Maximum efficiency in pure aloha is 18.4%, but maximum efficiency in slotted aloha is 36.8%. Collisions are reduced when there is pure aloha. By halving the number of collisions, Slotted Aloha doubles the efficiency by pure aloha. [1][2]

References:

[1] Kurose and Ross,2012,Computer Networking: A Top Down Approach, 6th edition book, Pearson

[2] Mahesh Parhar,(26-November-2019),Difference Between Pure Aloha and Slotted Aloha:

[https://www.tutorialspoint.com/differences-between-pure-aloha-and-slotted-aloha]